# Operating Instructions – 7-11 Filter, Pump & Motor

Please read the following instructions carefully before starting assembly of the product. The manufacture, 7-11 Pool Products Ltd. does not cover warranty on improper installation and any deviation from the instructions.

- Always turn pump “OFF” before changing the valve positions.

- Never run pump dry as it depends on water for lubrication.

- Motor bearings are sealed. DO NOT lubricate.

- Always maintain minimum water lever to ½ way up the skimmer mouth. This prevents air being drawn though the skimmer or potential pump damage resulting from a dry run condition.

- If pump priming difficulties are experienced, check all suction lines, valves, gaskets and strainer lid for tightness.

- Clean pump strainer regularly. Maintain a proper chemical balance.

Included in small Parts Bag

18” Sand Filter					24” Sand Filter

60 PSI Pressure Gauge				60 PSI Pressure Gauge

Clear Plastic Sand Fill Disc				Clear Plastic Sand Fill Disc

Clamps (6) 					Clamps (2)

11/4” Adapter (5)					11/2” Adaptor (2)

## Installation

- When unpacking the filter, be sure the unit is complete and no visible shipping damage has occurred.

- Place the filter on a stable foundation, either a concrete pad or sidewalk slab. Filter systems once loaded with media can exceed a weight of 400-700 lbs.

- Allow sufficient clearance around filter system for access and maintenance. Control valve, connections, drains, etc. Should be arranged to provide easy access for operation and serving.

- Provide adequate ventilation for pumping equipment installed in conjunction with filter. The filter and pump should be protected from weather and temperature rise on the pump motor.

- Installation of filters and pumps should be located as close to the pool as possible to avoid excessive piping friction loss.

- If system is installed below pool level it is important to install isolating valves to prevent risk of flooding from back-up water from the pool or spa.

- The proper media selection is critical to good filter performance. Insure the media is an approver Quartz Silica with a sizing range of 40-55mm. (No 20 Silica). 1cu. Ft. silica sand weighs approx. 100 lbs.

- The installation of filters requires no special tools.

- Confirm that plumbing lines from pool are correctly identified. This will insure proper connection to makings on Filter Valve (i.e. “Pump”, “Waste” and “Return” ).

- The tank should be situated in such a way to allow easy access to winter drain on underside of filter.

## Loading Filter Media

- Once the filter system has been positioned the installation of filter media may be undertaken as follows:

- Check Filter size in order to determine necessary amount of media required.

- Double-check that the internal system consisting of an eight lateral hub and standpipe is intact inside the tank.

- Fill tank half full of water. Place the disposable clear plastic loading disc in upper tank opening to stabilize the filter standpipe. It is important that the standpipe remain centered through the media loading procedure to insure alignment with valve assembly opening.

- Proceed to load correct volume of dry media slowly (if media is saturated loading can be difficult). Generally, a coffee can or funnel can aid in the loading process.

- Once media is loaded, discard sand loading disc and check to determine if standpipe is properly centered in tank opening

## Pump Priming

- With the pump “OFF”, remove the lid from the pump strainer basket and pour in approximately 1 gallon of water.

- Replace lid.

- If valve has been installed on the pump suction line, close before filling strainer. Open again after lid has been securely fastened in place. Any valves on the return lines should be open. The pump should not need to be required unless the pump has been drained for servicing or winterizing (refer to complete pump operations manual provided with pumps).

## Backwash Information

- Shut off pump. Steel backwash position for dial valve

- Turn motor on. Let water flow until water in sight glass is clear (approx. 3 min).

- Turn motor off. Select rinse position on dial valve.

- Turn motor on. Let water run approx. 30 seconds

- Turn motor off. Select filter position on dial valve

- Turn motor on.

## Winterize

- Depress valve handle and lock in the position mid-way between any two-valve positions. This preserves the valve seat gasket during the winter by relieving the spring pressure.

- Remove the cap on bottom of filter tank, and completely drain water from tank. Store plug for replacement in the spring.

- Above Ground pump and motor must be completely removed, drained and taken inside. Indoor storage of the Above Ground Pump is a must.

- Inground pumps have two drain plugs that must be removed which will allow you to leave the unit on the filter site. Make sure No water is left in pump and pot. Follow pump instructions carefully.

## Initial Start-up

- Make sure the correct amount of media is in the filter tank

- Prime pump

- Set valve in “back-wash” position

- Start pump and operate in “back-wash” cycle for two minutes. This will purge the filter of excess fines in the media.

- Turn pump “OFF” and set valve handle to “RINSE”. Start pump and run for one (1) minute.

- Stop pump and set valve handle to “FILTER”. Re-start pump. System is now operating on a normal “FILTER” cycle.

- Adjust the valves in the simmer and main drain liners (if provide) to achieve balanced flow rates.

- When a pool is filled initially, the filter may have to be Back-Washed one a day until the water becomes sparkling clear. From that point on, Back-wash when pressure gauge indicates 10lbs/ p.s.i. higher than at stating pressure.

## Limited Warranty

To original purchaser of its Filter Equipment, 7-11 Pool Products Ltd., warrant its product to be free of defect in material and workmanship for period of one year from date of purchase.

Equipment which fails or is defective during warranty period, except as a result of freezing, accident, negligence, improper installation, or dry run, shall be repaired or replaced at our option without charge within 90 days of defective product.

In order to obtain warranty replacements or repair, defective products should be returned, transportation paid, to the place of purchase, or the nearest authorized “7-11” service center.

No return may be made directly to the factory without express written authorization of 7-11 Pool Products Ltd.

Some provinces or states do not allow a limitation on the duration of implied warranty, or the exclusion or limitation of incidental consequential damages, so the above limitation or exclusion may not apply.

This warranty gives you specific legal right, and you may also have other rights which could vary from province to province or state to state.