## How to Apply OMNISHIELD

1. Cool and clean the nozzle

Make sure the nozzle is cool, then remove any leftover filament or residue.

2. Shake and uncap

Gently shake the OMNISHIELD applicator, then remove the cap.

03. Brush it on

Use the microfiber brush tip to lightly coat the outside of the nozzle in thin, even strokes. Avoid the tip opening.

4. Let it dry

The coating dries white. Give it a few minutes to set fully.

5. Print normally

OMNISHIELD is rated for 0-320°C and works with all thermoplastics.

6. Reapply as needed

Thin coats work best. Reapply when you notice filament starting to cling again.