MEMORANDUM

TO	: Steve Lipman, Yvonne Unger

CC	: Dave Ellis, Chief Solid Waste Management Section

: David Johnston, Deputy Regional Director, BWP

FROM	: Mark Dakers, BWP-SWM

RE	: Army Corps Dredging Marshfield

DATE	: March 25, 2004

The Department was asked by the Army Corps of Engineers to attend a meeting on March 16, 2004 to discuss Army Corp's dredging of inner Green Harbor in Marshfield Massachusetts.  The main purpose of the meeting was to discuss options for dredge reuse and disposal.  This memorandum briefly summarizes the meeting and required follow-up actions by the Department.

Follow-up Actions Required by Department:

1.  Provide Vine Associates Inc. (the consultant for the town) with recent Department Beneficial Use Determination Decisions.--Completed by Mark Dakers on March 18, 2004

2.  SERO is to review sediment-sampling results in order to determine possible reuse/disposal options and provide this information to the Town and Army Corps.  ---Subject of this Memorandum

3.  Schedule meeting to discuss Town's preferred reuse/disposal options, if necessary.

--To be scheduled

Project Background:

- The Corps expects to generate 50,000 cubic yards of silty sediments from the dredging of Green Harbor

- There are several other piggyback dredge projects (town and local entities) that are expected to generate another 20,000 cubic yards of sediments (Total dredge volume = 70,000 yards).

- The last time the inner Harbor was dredged was 1983 and the town of Marshfield/Army Corps and the DEM were permitted to dispose of the sediment at large basin located very near the Harbor.  The dredge remained at this location until late 1990s when the material was placed at the Town of Marshfield's landfill prior to closure.

- Until very recently the Town and Army Corps were assuming they would dispose of the sediment in the basin just as they did before.  The Department spoke with the Army Corps and indicated that the 401 program could not approve disposal of sediment at any location other than a landfill.  Alternative disposal and reuse options were discussed.  The Corps then requested a meeting with the Department and the Town to discuss why they could not dispose of the sediment at the basin and what other alternatives there may be for the dredge.

Review of Sediment Data for inner Harbor dredging by Army Corps:

I finished reviewing the 1999 laboratory results for sediment samples collected as part of inner Green Harbor Dredging Project. Based on this review, I have concluded the following:

- Seven cores were collected in April 1999 and analyzed individually, no compositing occurred.

- All seven samples were analyzed for the following parameters:

- grain size analysis,

- Arsenic, cadmium, chromium, copper, lead, mercury, nickel and zinc,

- PCBs and pesticides,

- PAHs,

- EPH/VPH.

- Grain size analysis indicates six of the seven cores contains sediment with greater than 15 percent silt and clay (passing through 200 sieve)

- Pesticides were not detected in any of the seven sediment samples.

- PCBs (one compound Aroclor 1260) were detected in four of the seven samples collected.  The report incorrectly states that PCBs were detected in one sample.  PCB concentrations ranged from 0.023 ppm to 0.052 ppm.  Reporting Limits for the test method were 0.005 ppm.

- PAHs were detected in all seven sediment samples.  Total PAH concentrations ranged from 0.439 ppm to 1.563 ppm.

- No VPH components were detected about detection limits.  However VPH holding times (two weeks) were exceeded (3-4 weeks).

- Total aliphatic and aromatic EPH components were detected.

C9-C18 range in seven samples—3 - 8 ppm

C19-C36 range in seven samples—3 - 91 ppm

C11-C 22 range from --10 - 21 ppm.

Comparison to Standards/Background/and Other Criteria:

- For those compounds for which the Department has promulgated cleanup standards all seven samples contained concentrations of contaminants less than S-1/GW-1 (310 CMR 40.000).

- For those PAHs and metals for which the Department has "Identified Background Levels in Soil" all seven samples contained concentrations of contaminants less than "Concentration in "Natural" Soil" (refer to Technical Update Background levels of Polycyclic Aromatic Hydrocarbons and metals in soil (5/23/2002), Updates: Section 2.3 Guidance for Disposal Site Risk Characterization-in Support of Massachusetts Contingency Plan (1992).

- The concentrations of metals and PAHs do not exceed acceptance criteria for Route 44 Gravel and Sand Carver, Massachusetts (site-specific background was established for this gravel pit).

- The concentrations of PCBs do exceed the acceptance criteria for Route 44 Gravel and Sand.

- Upland Reuse/Reuse within 100 Year Floodplain or Wetland Buffer Zone/Landfill Reuse or Disposal: The concentrations of PCBs detected in four of the seven sediment samples limits possible reuse options:

- Option 1 Upland Reuse: The dredge can only be used at an upland location that has similar concentrations of PCBs in order to meet antidegradation provisions of the MCP.  There are probably not many upland locations that will meet this criteria in the vicinity of the project.

- Option 2: Reuse within 100 Year Floodplain or Wetland Buffer Zone As Defined at 310 CMR 10.000: The dredge can only be used as fill at locations within hundred year floodplain or wetland buffer zone that has similar concentrations of PCBs in order to meet antidegradation provisions of the MCP.  The historic dewatering location, that is adjacent to the Harbor, has accepted dredge sediments from inner Green Harbor in the past.  PCBs are likely to be present at this location and in the immediate vicinity of this location (need for testing?).  Sediment samples from other dredge projects planned for Green Harbor may provide additional information regarding distribution of PCBs in the sediments.

- Option 3: Reuse or Disposal at a Permitted Landfill (lined/unlined): The dredge does not exceed maximum allowable contaminant levels for sediment reuse at lined landfills as specified in the Department' s Policy, COMM-94-007, the Interim Policy for Sampling, Analysis, Handling and Tracking Requirements for Dredged Sediment Reused or Disposed at Massachusetts Permitted Landfills.  In order to reuse the dredge at an unlined landfill a landfill-minor modification permit or other DEP approval would be required.

In addition to the aforementioned reuse and disposal options, confined aquatic disposal and unconfined open ocean disposal are available.  However, based on discussions at the March 16, 2004 meeting they are not being considered.

NEXT STEPS:

- Please review attach memorandum and provide comments.

- Any plans regarding the historic dewatering site location in reference to wetlands, floodplain, etc. would be useful in order to discuss with regional wetland staff- Dave Hill.  The Department should contact the town's consultant and request any plans they have available.  If they don't have any plans the Department should generate a GIS map for discussions with regional wetland staff.

- Need to schedule follow meeting and or communicate reuse options with town.