SECTION 1 - IDENTIFICATION____________________________________________________________

Product name: ISOPROPANOL 99%

Other Names/Synonyms: IPA

Distributed by:

Green Products Company

810 Market Avenue

Richmond, CA  9480

www.greenproductsco.net

Emergency Telephone Number: 1-800-535-5053

Product Use: Cleaning agent, sterilizing/medical

SECTION 2 – HAZARD(S) IDENTIFICATION__________________________________________________

UN1219, Isopropanol, 3, PGII, ERG129

Warning

Emergency Overview

Appearance: liquid, colorless

WARNING! FLAMMABLE LIQUID AND VAPOR. MAY AFFECT THE CENTRAL NERVOUS

SYSTEM CAUSING DIZZINESS, HEADACHE OR NAUSEA. MAY CAUSE EYE IRRITATION.

PROLONGED OR REPEATED CONTACT MAY DRY THE SKIN AND CAUSE IRRITATION

AND BURNS.

Potential Health Effects

Exposure routes

Inhalation, skin absorption, skin contact, eye contact, ingestion

Eye contact

Can cause eye irritation. Symptoms include stinging, tearing, redness, and swelling of eyes.

Skin contact

May cause mild skin irritation. Symptoms may include redness and burning of skin. Prolonged

or repeated contact may dry the skin. Symptoms may include redness, burning, and drying and cracking

of skin, skin burns, and other skin damage.

Ingestion

Swallowing small amounts of this material during normal handling is not likely to cause harmful effects. Swallowing large amounts may be harmful. This material can get into the lungs during swallowing or vomiting. This results in lung inflammation and other lung injury.

Inhalation

Breathing of vapor or mist is possible. Breathing small amounts of this material during normal

handling is not likely to cause harmful effects. Breathing large amounts may be harmful. Symptoms are

not expected at air concentrations below the recommended exposure limits, if applicable (see Section

8.).

Aggravated Medical Condition

Preexisting disorders of the following organs (or organ systems) may be aggravated by exposure

to this material: Skin, lung (for example, asthma-like conditions), kidney.

Symptoms

Signs and symptoms of exposure to this material through breathing, swallowing, and/or passage of the material through the skin may include: stomach or intestinal upset (nausea, vomiting, diarrhea), irritation( nose, throat, airways), central nervous system depression (dizziness, drowsiness, weakness, fatigue, nausea, headache, unconsciousness), lowered blood pressure, mild, temporary changes in the liver, effects on heart rate, respiratory depression (slowing of the breathing rate), lack of coordination, confusion, lung edema (fluid buildup in the lung tissue), kidney damage, coma

Target Organs

Exposure to this material (or a component) has been found to cause kidney damage in male rats. The mechanism by which this toxicity occurs is specific to the male rat and the kidney effects are not expected to occur in humans. Breathing isopropanol vapors has caused damage to the lining of the middle ear in experimental animals. The relevance of this finding to humans is uncertain. Overexposure to this material (or its components) has been suggested as a cause of the following effects in laboratory animals: mild, reversible liver effects

Carcinogenicity

This material is not listed as a carcinogen by the lnternational Agency for Research on Cancer (IARC), the National Toxicology Program (NTP), or the Occupational Safety and Health Administration (OSHA).

Reproductive hazard

This material (or a component) has been shown to cause harm to the fetus in laboratory animal studies. Harm to the fetus occurs only at exposure levels that harm the pregnant animal. The relevance of these findings to humans is uncertain.

SECTION 3_- COMPOSITION/INFORMATION ON INGREDIENTS________________________________

HAZARDOUS COMPONENTS		CAS No.		CONCENTRATION

Isopropanol		   67-63-0		        <=100%

SECTION 4 – FIRST-AID MEASURES_______________________________________________________

Eyes

If symptoms develop, immediately move individual away from exposure and into fresh air.

Flush eyes gently with  water for at least 15 minutes while holding eyelids-apart; seek immediate medical

attention.

Skin

Remove contaminated clothing. Wash exposed area with soap and water. If symptoms persist,

seek medical attention. Launder clothing before reuse.

Ingestion

Seek medical attention. If individual is drowsy or unconscious, do not give anything by mouth;

Place individual on the left side with the head down. Contact a physician, medical facility, poison control center for advice about whether to induce vomiting. If possible, do not leave individual unattended.

Inhalation

If symptoms develop, move individual away from exposure and into fresh air. If symptoms

Persist, seek medical attention. If breathing is difficult, administer oxygen. Keep person warm and quiet; seek immediate medical attention.

Notes to physician

Hazards: Administration of high doses of isopropanol in combination with known hepatotoxic

Chemicals resulted in enhanced liver toxicity in experimental animals. This material is an aspiration hazard. Potential danger from aspiration must be weighed against possible oral toxicity (See Section 2 -

Swallowing) when deciding whether to induce vomiting.

Treatment: No information available.

SECTION 5 – FIRE-FIGHTING MEASURES_____________________________________________

Suitable extinguishing media

Dry chemical, Carbon dioxide (CO2), Water spray

Hazardous combustion products

carbon dioxide and carbon monoxide

Precautions for fire-fighting

Material is volatile and readily gives off vapors which may travel along the ground or be moved by ventilation and ignited by pilot lights, flames, sparks, heaters, smoking, electric motors, static discharge or other ignition sources at locations near the material handling point. Never use welding or cutting torch on or near drum (even empty) because product (even just residue) can ignite explosively. Wear full firefighting turn-out gear (full Bunker gear) and respiratory protection (SCBA). Water may be ineffective for extinguishment unless used under favorable conditions by experienced fire fighters. Use water spray to cool fire exposed containers and structures until fire is out if it can be done with minimal risk. Avoid spreading burning material with water used for cooling purposes.

NFPA Flammable and Combustible Liquids Classification

Flammable Liquid Class IB

SECTION 6 – ACCIDENTAL RELEASE MEASURES______________________________________

Personal precautions

For personal protection see section 8. Persons not wearing protective equipment should be excluded from area of spill until clean-up has been completed. Ensure adequate ventilation. Eliminate all ignition sources (flares, flames including pilot lights, electrical sparks). Pay attention to the spreading of gases especially at ground level (heavier than air) and to the direction of the wind.

Environmental precautions

Prevent spreading over a wide area (e.g.by containment or oil barriers). Do not let product enter drains. Do not flush into surface water or sanitary sewer system. Local authorities should be advised significant spillages cannot be contained.

Methods for cleaning up

Contain spillage, and then collect with non-combustible absorbent material, ( e.g. sand, earth, diatomaceous earth, vermiculite) and place in container for disposal according to local / national

regulations (see section l3).

Other information

Comply with all applicable federal, state, and local regulations. Suppress (knock down) gases/vapors/mists with water spray jet.

SECTION 7 – HANDLING AND STORAGE___________________________________________________

Handling

Containers of this material may be hazardous when emptied. Since emptied containers retain

product residues (vapor, liquid, and/or solid), all hazard precautions given in the data sheet must be observed. Static ignition hazard can result from handling and use. Electrically bond and ground all

containers, personnel and equipment before transfer or use of material. Special precautions may be

necessary to dissipate static electricity for non-conductive containers. Use proper bonding and

grounding during product transfer as described in National Fire Protection Association document NFPA

77. Warning. Sudden release of hot organic chemical vapors or mists from process equipment

operating at elevated temperature and pressure, or sudden ingress of air into vacuum equipment, may

result in ignitions without the presence of obvious ignition sources. Published "autoignition” or

"ignition" temperature values cannot be treated as safe operating temperatures in chemical processes

without analysis of the actual process conditions. Any use of this product in elevated temperature

processes should be thoroughly evaluated to establish and maintain safe operating conditions.

Storage

Store in a cool, dry, ventilated area, away from incompatible substances. Small quantities of

peroxides may form on prolonged storage. Exposure to light and/or air significantly increases the rate of peroxide formation. If evaporated to a residue, the mixture of peroxides and isopropanol may explode

when exposed to heat or shock.

SECTION 8 - EXPOSURE CONTROLS/PERSONAL PROTECTION_____________________________

EXPOSURE GUIDELINES

Isopropanol				67-63-0

ACGIH                                    time weighed average				200 ppm

ACGIH                                    short term exposure limit				400 ppm

NIOSH                                    recommended exposure limit (REL)		400 ppm

NIOSH                                    recommended exposure limit (REL)		980 mg/m3

NIOSH                                    short term exposure limit				500 ppm

NIOSH                                    short term exposure limit				1225 mg/m3

OSHA Z1                                permissible exposure limit			400 ppm

OSHA Z1                                permissible exposure limit			980 mg/m3

GENERAL ADVICE

These recommendations provide general guidance for handling this product. Personal protective equipment should be selected for individual applications and should consider factors which affect exposure potential, such as handling practices, chemical concentrations and ventilation. It is ultimately the responsibility of the employer to follow regulatory guidelines established by local authorities.

Exposure controls

Provide sufficient mechanical(general and/or local exhaust) ventilation to maintain exposure below exposure guidelines (if applicable) or below levels that cause known, suspected or apparent adverse effects.

Eye protection

Wear chemical splash goggles when there is the potential for exposure of the eyes to liquid, vapor or mist.

Skin and body protection

Wear normal work clothing including long pants, long-sleeved shirts and foot covering to prevent direct contact of the product with the skin. Launder clothing before reuse. If skin irritation develops, contact your facility health and safety professional or your local safety equipment supplier to determine the proper personal protective equipment for your use.

Wear resistant gloves such as:

Nitrile rubber

Respiratory protection

A NIOSH-approved air purifying respirator with an appropriate cartridge and/or filter may be permissible under certain circumstances where airborne concentrations are expected to exceed exposure limits (if applicable) or if overexposure has otherwise been determined. Protection provided by air-purifying respirators is limited. Use a positive pressure, air-supplied respirator if there is any potential for uncontrolled release, exposure levels are not known or any other circumstances where an air-purifying respirator may not provide adequate protection.

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES________________________________________

Physical state	liquid

Form	liquid

Color	colorless

Odor	pleasant

Boiling point/boiling range	180.5 F°/82.5 C° @ 101.32 kPa

Melting point/range	-127.3 F°/-88.5 C°

Sublimation point	no data available

pH	not applicable

Flash point	54 F°/12 C° Closed cup

Ignition temperature	no data available

Evaporation rate	7.70 Ethyl Ether

Lower explosion limit/Upper explosion limit	2.5% (V)/12% (V)

Particle size	no data available

Vapor pressure	6.052 kPa @ 77 F°/25C°

Relative vapor density	2.1 AIR=1

Density	0.789 g/cm3

6.65 lb/gal @ 60.1 F°/15.6C°

Bulk density	No data

Water solubility	soluble

Solubility(ies)	no data available

Partition coefficient: n-octanol/water	no data available

Log pow	0.05

Autoignition temperature	750 F°/399 C°

Viscosity, dynamic	2.4 mPa.s

Viscosity, kinematic	no data available

Solids in solution	no data available

Decomposition temperature	no data available

Burning number	no data available

Dust explosion constant	no data available

Minimum ignition energy	no data available

SECTION 10 – STABILITY AND REACTIVITY_________________________________________________

Stability

Stable.

Conditions to avoid

Heat, flames and sparks.

Incompatible products.

Acids, Aldehydes, alkalis, amines, ethylene oxide, halogenated hydrocarbons, halogens, isocyanates, strong oxidizing agents, Do not use with aluminum equipment at temperatures above 120 degrees F.

Hazardous decomposition products

Carbon dioxide and carbon monoxide

Hazardous Reactions

Product will not undergo hazardous polymerization

Thermal decomposition

No data

SECTION 11 – TOXICOLOGICAL INFORMATION______________________________________________

Acute oral toxicity

ISOPROPONAL					No data available

Acute inhalation toxicity				LC50 rat: 16000 ppm; 4h

Acute dermal toxicity				No data available

SECTION 12 – ECOLOGICAL INFORMATION_________________________________________________

Biodegradability

ISOPROPONAL					No data available

Bioaccumulation

ISOPROPONAL					No data available

ECOTOXICITY EFFECTS

Toxicity to fish					96 h LC Fathead minnow (Pimephales promelas): 5770.00 						- 7450.00 mg/l Method: Flow through; Mortality

Toxicity to daphnia and other aquatic 		24 h static test LC50 Water flea (Daphnia magne):>

Invertebrates					10,000 mg/l Method: Static Mortality

Toxicity to algae

ISOPROPONAL					No data available

Toxicity to bacteria

ISOPROPONAL					No data available

Biochemical Oxygen Demand (BOD)

ISOPROPONAL					No data available

Chemical Oxygen Demand (COD)

ISOPROPONAL					No data available

Additional ecological information

ISOPROPONAL					No data available

SECTION 13 – DISPOSAL CONSIDERATIONS________________________________________________

WASTE DISPOSAL METHODS

Dispose of in accordance with all applicable local, state and federal regulations. For assistance with your waste management needs – including disposal, recycling and waste stream reduction, contact INFOTRAC at 1-800-535-5053

SECTION 14 – TRANSPORT INFORMATION_________________________________________________

REGULATION

ID		PROPER SHIPPING	HAZARD	SUSIDIARY	PACKING 	MARINE

NUMBER	        NAME		CLASS		HAZARDS	GROUP	POLLUTANT

/LTD QTY

____________________________________________________________________________________

MEXICAN REGULATION FOR THE LAND TRANSPORT OF HAZARDOUS MATERIALS AND WASTES

UN 1219	Isopropanol		3				II

INTERNATIONAL AIR TRANSPORT ASSOCIATION – PASSENGER

UN 1219	Isopropanol		3				II

INTERNATIONAL AIR TRANSPORT ASSOCIATION – CARGO

UN 1219	Isopropanol		3				II

INTERNATIONAL MARITIME DANGEROUS GOODS

UN 1219	Isopropanol		3				II

TRANSPORT  CANADA – INLAND WATERWAYS

UN 1219	Isopropanol		3				II

TRANSPORT CANADA – RAIL

UN 1219	Isopropanol		3				II

TRANSPORT CANADA – ROAD

UN 1219	Isopropanol		3				II

U.S. DOT – INLAND WATERWAYS

UN 1219	Isopropanol		3				II

U.S. DOT – RAIL

UN 1219	Isopropanol		3				II

U.S. DOT – ROAD

UN 1219	Isopropanol		3				II

Dangerous goods descriptions (if indicated above) may not reflect quantity, end-use or region-specific exceptions that can be applied. Consult shipping documents for descriptions that are specific to the shipment.

SECTION 15 – REGULATORY INFORMATION_______________________________________________

CALIFORNIA PROP. 65

This product does not contain any chemicals known to the State of California to cause cancer, birth defects or any other reproductive harm.

SARA Hazard Classification

Fire Hazard

Acute Health Hazard

SARA 313:This material doesn’t contain any chemical components with known CAS numbers that

exceed the threshold ( De Minimis) reporting levels  established by SARA Title III, Section 313.

New Jersey RTK Label Information

ISOPROPANOL							67-63-0

Pennsylvania RTK Label Information

ISOPROPANOL							67-63-0

Notification status

Australia. Industrial Chemical( Notification and Assessment)		y (positive listing)

Act

Canada. Canadian Environmental Protection Act (CEPA).		y (positive listing)

Domestic Substances List (DSL). (Can.Gaz.Part II, Vol. 133)

China. Inventory of Existing Chemical Substances			y (positive listing)

Japan. Kashin-Hou Law List						y (positive listing)

Japan. Industrial Safety& Health Law (ISHL) List				y (positive listing)

US. Toxic Substances Control Act					y (positive listing)

Korea. Toxic Chemical Law Control (TCCL) List				y (positive listing)

Philippines. The toxic Substances and Hazardous and Nuclear		y (positive listing)

Waste Control Act

New Zealand. Inventory of Chemicals (NZIoC), as published		y (positive listing)

By ERMA New Zealand

|  | HMIS | NFPA |
| --- | --- | --- |
| Health | 2 | 1 |
| Flammability | 3 | 3 |
| Physical hazards | 0 | XXXXXXXXXXXXXXXXXXXXXX |
| Instability | XXXXXXXXXXXXXXXXXXXXXX | 0 |
| Specific hazard | -- | -- |

SECTION 16 – OTHER INFORMATION______________________________________________________

The information accumulated herein is believed to be accurate but is not warranted to be whether originating with the company or not. Recipients are advised to confirm in advance of need that the information is current, applicable, and suitable to their circumstances.

Revision Date: February 15, 2015