SECTION 1 - IDENTIFICATION____________________________________________________________

PRODUCT NAME: Green’s Liquid Paint, Varnish & Lacquer Remover

GENERAL ID: Compound Lacquer, Paint, and Varnish Removing Liquid

MANUFACTURER: Green Products Company

810 Market Avenue

Richmond, CA  94801

United States

www.greenproductsco.net

510-235-9667

Emergency Contact : INFOTRAC  800-535-5053

Product Restrictions: Do not use on linoleum, asphalt, tile, plastic surfaces or fiberglass. Do not use in areas where vapors can accumulate and concentrate such as basements, bathrooms, bathtubs, and small enclosed areas.

SECTION 2 – HAZARD(S) IDENTIFICATION__________________________________________________

HAZARD CLASS: UN1263, Flammable Liquid, 3, PGII, NAERG#127

DANGER

Flammable liquid and vapor. Vapor harmful. May be fatal or cause blindness if swallowed. Harmful if inhaled or absorbed through skin. Causes eye burns. Causes skin and respiratory tract irritation.

SECTION 3_- COMPOSITION/INFORMATION ON INGREDIENTS________________________________

INGREDIENT	% (BY VOL)			ACGIH(TLV)

Chlorinated Hydrocarbons (Methylene Chloride)	10-30				50 ppm

(CAS# 000075.09-2)

Aromatic Hydrocarbons	10-30				200 ppm

Ketone	10-30				1000 ppm

Alcohol	10-30				200 ppm

The specific chemical identity and/or exact percentage (concentration) of composition has been withheld as a trade secret.

SECTION 4 – FIRST-AID MEASURES_______________________________________________________

IF ON SKIN: Thoroughly wash exposed area with soap and water. Remove contaminated clothing. Launder clothing before re-use.

IF IN EYES: Flush with large amounts of water, lifting upper and lower lids occasionally; SEEK MEDICAL ATTENTION IMMEDIATELY.

IF SWALLOWED: Call a physician, emergency room or poison control center IMMEDIATELY for instructions to induce vomiting.

IF INHALED: If affected, remove to fresh air. If breathing is difficult, administer oxygen. If breathing has stopped, give artificial respiration. Keep person warm and quiet, and SEEK MEDICAL ATTENTION. Do not give stimulants. Epinephrine or ephedrine may adversely affect the heart with fatal results.

SECTION 5 – FIRE-FIGHTING MEASURES_____________________________________________

Flashpoint: 25F

Extinguishing Media: Regulate foam or water fog, or carbon dioxide, carbon monoxide, hydrogen chloride, phosgene, nitrogen compounds, various hydrocarbons.

Firefighting Procedures: Self-contained breathing apparatus with a full facepiece operated in pressure-demand or other positive pressure mode.

Special Fire & Explosion Hazard: DANGEROUS FIRE HAZARD WHEN EXPOSED TO HEAT, SPARK OR FLAME! Vapors are heavier than air and may travel along the ground or may be moved by ventilation and ignited by pilot lights, other flames, sparks, heaters, smoking, electric motors, electrical outlets or other ignition sources at locations distant from material handling point. Never use a welding or cutting torch on or near the drum (even if empty) because product or residue can ignite explosively.

SECTION 6 – ACCIDENTAL RELEASE MEASURES______________________________________

For a small release: Absorb liquid on paper, vermiculite, floor absorbent material and transfer to hood.

For a large release: Eliminate all ignition sources (flares, flames, pilot lights, electrical sparks, etc.). Persons not wearing protective equipment should be excluded from area of spill until cleanup has been completed. Stop spill source, dike area of spill to prevent spreading. Pump liquid to salvage tank. Remaining liquid may be taken up on sand, clay, earth, floor absorbent or other absorbent material and shoveled into containers.

Water disposal method:

For small release: Allow volatile portion to evaporate in hood. Allow sufficient time for vapors to completely clear the hood ductwork. Destroy remaining material by burning in an iron pan.

For large release: Destroy by liquid incineration with off-gas scrubber. Material collected on absorbent material may be deposited in a posted toxic substance landfill in accordance with local, state, and federal regulations.

SECTION 7 – HANDLING AND STORAGE___________________________________________________

Store product in cool, dry area away from electrical outlets & pilot lights. Make sure cap is tightly closed.

This product contains chemicals known to the State of California to cause cancer. Contains Methylene Chloride, Methanol & Mineral Spirits. Methylene Chloride has been shown to cause cancer in certain laboratory animals. DANGER POISON! MAY BE FATAL OR CAUSE BLINDNESS IF SWALLOWED. SKIN AND EYE IRRITANT. USE ONLY WITH ADEQUATE VENTILATION TO PREVENT BUILDUP OF VAPORS. Do not use in areas where vapors can accumulate and concentrate such as basements, bathrooms, bathtubs, and small enclosed areas. Whenever possible use outdoors in an open air area. If using indoors, open all windows and doors and maintain cross ventilation of moving fresh air across the work area. If strong odor is noticed or you experience slight dizziness – STOP – ventilation is inadequate. Leave the area immediately. Contact with flame or hot surfaces may produce toxic gases. Clean up rags, papers and all waste promptly. If work area is not well ventilated, DO NOT USE THIS PRODUCT. If properly used, an “organic vapor” respirator may offer additional protection.

SECTION 8 - EXPOSURE CONTROLS/PERSONAL PROTECTION_____________________________

RESPIRATORY PROTECTION: If TLV of any component of the product is exceeded, a NIOSH\MESA jointly approved self-contained breathing apparatus with a full face piece operated in pressure demand or other positive pressure mode is advised; however, OSHA regulations also permit other NIOSH\MESA respirators under specific conditions. (See your safety equipment supplier.)

VENTILATION: Provide sufficient mechanical (general) and\or local exhaust ventilation to maintain exposure below the TLVs of components.

PROTECTIVE GLOVES: Wear resistant gloves such as: polyvinyl alcohol, natural rubber, etc.

EYE PROTECTION: Chemical splash goggles in compliance with OSHA regulations are advised; however, OSHA regulations also permit other type of safety glasses. (See your safety equipment supplier)

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES________________________________________

Appearance                     For Product		Clear liquid

Boiling Point                    For Product		232.00 Deg F

Vapor Pressure               For Product		335.00 mm Hg@ 65.00 Deg F

Specific Vapor Density		Greater than air

Specific Gravity	      	.96

Percent Volatile		73%

VOC*		2.64 lbs\per gallon of paint remover		                                                                                                or 316  grams per liter

Evaporation Rate		Slower than ether

All other information not available

SECTION 10 – STABILITY AND REACTIVITY_________________________________________________

Hazardous Polymerization:  Cannot occur

Stability: Stable

Incompatibility: Avoid contact with strong oxidizing agents, strong alkalis, and strong mineral acids.

SECTION 11 – TOXICOLOGICAL INFORMATION______________________________________________

Effects of overexposure:

EYES: Can cause severe irritation, redness, tearing, blurred vision.

SKIN: Prolonged or repeated contact can cause skin irritation, even a burn.

BREATHING: Excessive inhalation of vapors can cause nasal and respiratory irritation, dizziness, weakness, fatigue, nausea, headache, possible loss of consciousness, central nervous system depression, and even asphyxiation.

SWALLOWING: Can cause gastrointestinal irritation, nausea, vomiting, diarrhea, blindness, and death. Aspiration of material into lungs can cause chemical pneumonitis, which can be fatal.

Warning: This product contains a chemical, which is known to the State of California to cause cancer in some laboratory animals. Risk to your health depends on level and duration of exposure. Use with adequate ventilation. Avoid contact with skin.

SECTION 12 – ECOLOGICAL INFORMATION_________________________________________________

Environmental Toxicty:

No environmental studies have been conducted on the product. The following are known component data:

Toluene (108-88-3): Freshwater Algae Data = 96 Hr EC50 Selenastrum capricornutum:>433 mg/L; Freshwater Fish

Species Data = 96 Hr LC50 Pimephales promelas: 25 mg/L  [flow-through] (1 day old); 96 Hr LC50 Oncorhynchus mykiss: 24.0 mg/L [flow-through]; 96 Hr LC50 Lepomis macrochirus: 24.0 mg/L [static]; 96 Hr LC50 Lepomis macrochirus: 13 mg/L [static]; Microtox Data = 30 min EC50 photobacterium phosphoreum: 19.7 mg/L; 48 Hr EC50

Water flea Data: 11.3 mg/L; 48 Hr EC50 water flea: 310 mg/L; 48 Hr RC50 Daphnia magna: 11.3 mg/L

Methanol (67-56-1): Freshwater Fish Species Data = 96 Hr LC50 Pimephales promelas: 28100 mg/L [flow-through]; 96 Hr LC50 Oncorhynchus mykiss: 13200 mg/L; Microtox Data = 5 min EC50 photobacterium phosphoreum: 43000 mg/L; 15 min EC50 photobacterium phosphoreum: 40000 mg/L; 25 min EC50 photobacterium phosphoreum: 39000mg/L

Methylene Chloride (75-09-2): Freshwater Algae Data = 96 Hr EC50 Selenastrum capricornytum: >660 mg/L; Freshwater Fish Species Data = 96 Hr LC50 Pimephels promelas: 193 mg/L [flow-through]; 96 Hr LC50 Pimephels promelas: 310 mg/L [static]; 96 Hr LC50 Oncorhynchus mykiss: 10.95 mg/L [flow-through]; 96 Hr LC50 Lepomis macrochirus: 193 mg/L [static]: 96 Hr LC50 Lepomis macrochirus: 193 mg/L [flow-through]; Microtox Data = 24 Hr EC50 Nitrosomonas: 1 mg/L; 15 min EC50 Photobacterium phosphoreum: 2.88 mg/L; Water flea data = 48 Hr EC50 water flea: 140 mg/L [static]

Acetone (67-64-1): Freshwater Fish Species Data = 96 Hr LC50 Oncorhynchus mykiss: 5540 mg/L [static]; 96 Hr LC50 Pimephales promelas: 6210 mg/L [flow-through]; 96 Hr LC50 Lepomis macrochirus: 8300 mg/L [static]; Microtox Data = 15 min EC50 photobacterium phosphoreum: 14500 mg/L; Water flea data = 48 Hr EC50 water flea: 0.0039 mg/L; 48 Hr EC50 Water flea: 12700 mg/L [static]; 48 Hr EC50 Daphnia magna: 12600 mg/L

Environmental Fate: No data available

SECTION 13 – DISPOSAL CONSIDERATIONS________________________________________________

Containers of this material may be hazardous when emptied. Since emptied containers retain product residues (vapor, liquid, and\or solid), all hazard precautions given in this data sheet must be observed.

Overexposure to components has apparently been found to cause the following in laboratory animals: liver abnormalities, lung damage, kidney damage, spleen damage, eye damage, brain damage, and central nervous system damage.

Do not pour waste down drains, sewers, or onto the ground where they may pollute the ground water. Contact your local health department for instructions on dealing with waste.

SECTION 14 – TRANSPORT INFORMATION_________________________________________________

DOMESTIC (Land, D.O.T.)

Proper shipping name: 	Paint Related Materials

Hazard Class: 		3

UN/NA:			UN1263

Packing Group: 		II

SECTION 15 – REGULATORY INFORMATION_______________________________________________

US EPA SARA Title III

Hazardous Components		     CAS #	Sec.302	     Sec. 304      Sec.313	      Sec. 110

(chemical name)				(EHS)	         RQ	(TRI)

Toluene {Benzene, Methyl-; Toluol}   108-88-3	No	    Yes 1000 lbs	  Yes	       Yes

Methanol {Methyl  alcohol; Carbinol;  67-56-1         No	    Yes 5000 lbs    Yes	        No

Wood alcohol}

Dichloromethane {Methylene Chloride) 75-09-2      No	    Yes 1000 lbs	   Yes            Yes

Acetone				          67-64-1	 No	     Yes 5000 lbs	   Yes	        Yes

SARA (Superfund Amendments and Reauthorization Act of 1986) Lists:

Sec.302		 EPA SARA Title III Section 302 Extremely hazardous Chemical with TPQ.  *indicates 10000  LB TPQ  if not volatile.

Sec.304: 	 EPA SARA Title III Section 304: CERCLA Reportable + Sec.302 with Reportable Quantity.   ** indicates statutory RQ

Sec.313:		EPA SARA Title III Section 313 Toxic Release Inventory. Note: -Cat indicates a member of a chemical category.

Sec.110:		EPA SARA 110 Superfund Site Priority Contaminant List

EPA Hazard Categories:

This material meets the EPA ‘Hazard Categories’ defined for SARA Title III Sections 311/312 as indicated:

[x] YES [  ] NO  Acute (immediate) Health Hazard

[x] YES [  ] NO  Chronic (delayed) Health Hazard

[x] YES [  ] NO  Fire Hazard

[  ] YES [x] NO  Sudden Release of Pressure Hazard

[  ] YES [x] NO  Reactive Hazard

SECTION 16 – OTHER INFORMATION______________________________________________________

The information accumulated herein is believed to be accurate but is not warranted to be whether originating with Green Products Company or not.

Responsibility for any damage to person or property in the use of this material rests with the purchaser.

Created on: January 31, 2011

Last revised:  April 27, 2015