Battery Maintenance

Safety:

- Keep flames, sparks or metal objects away from batteries

- Always wear eye protection, gloves, protective clothing when working with batteries

- Check that terminals are properly tightened, terminals that are too tight or too loose could result in post breakage, meltdown or fire (Do not torque over 105 INCH LBS)

Watering:

- Add water, never acid, to cells (distilled water recommended)

- DO NOT OVERWATER!

- Check water level once a month

- ALWAYS MAKE SURE THE PLATES ARE COVERED WITH WATER WHEN BATTERIES ARE NOT CHARGED FULLY

- IF the batteries are fully charged, add water to a level 1/8” below the bottom of the fill tube.

- After watering, replace and tighten all vent caps

# Inspection & Cleaning

- Keep batteries clean and dry from residue

- Check that all vent caps are tight

- Use a solution of baking soda and water to clean acid residue on the batteries or corrosion on the terminals. (Never clean batteries with vent caps off or loose)

- Terminal protectors, protective spray or petroleum jelly should be applied to terminals to reduce corrosion

# Storage / Maintenance

- Batteries should be fully charged prior to and during storage.

- Every time you finish using the car be sure to plug in to get batteries back to a full charge (This will help with the life of the batteries)

- Storing over the winter be sure to keep charger plugged in and once a month unplug from the car for about 20 seconds and plug back in to bring batteries back to a full charge.

- Never discharge batteries completely (Below 31.5 Volts)

- Store is a cool, dry place

- Make sure the batteries are fully charged before putting back in service.

- Cars stored more than 3 months should have a battery minder / tender maintenance charger connected your car. This increases the battery life up to 35%

- Put car in tow position