99 GT

1999 Mustang GT, 4.6 liter, 280cu, SOHC, 2V, 9.0:1 Compression

The 99 Modular Motor features Revised Camshafts, Revised Intake Manifold, PI Heads, Larger Valves, Coil-On-Plug Ignition 3.27:1 Posi-Traction 8.8" Rear, 5-speed Tremec T-45 Manual Transmission, 4 Wheel Disc Brakes, 17" Rims, and Traction Control. The 4.6-Liter SOHC Modular V8 engine produces 260 horsepower at 5000 rpm  with 302 foot pounds of torque at 4000 rpm. Those numbers are rated at the flywheel, the Rear Wheel numbers are @ 235 HP/270 TQ  
(another words they're slow as shit). 

1999 is the last of the Windsor Motors, (different valve covers, 8 bolt crank, and thicker block webbing) 
2000-2004 are Romeo Motors, (different valve covers, 6 bolt crank, thinner block webbing).
(This is why all SC Mfg's dont recommend more than 8lbs of Boost on 2000-2004 Motors)

Options: 
A/C, Cruise Control, Power Windows, Power Locks, Power Steering, Remote Keyless Entry, Black Leather Interior, Power Seats, Driver and Passenger Airbag's, Alarm, Anti-Lock Brakes, Fog Lights, AM FM Cassette CD 80 Watt premium sound system, and Rear Window Defroster.

So basically a stock Mustang GT looks great, handles ok, BUT IS SLOW AS SHIT!!! 
I immediately took care of that by increasing the power output by a full 60%  in one day.....
It's also been lowered for better handling, shocks and struts changed for better comfort, rims and tires for looks and traction, 4.10 gears and a short throw shifter for THE FUN FACTOR, and more......

Mods:
Procharger P-1SC @ 10lbs (1FE211-10I H.O. Intercooled System)
2 Core Air to Air Intercooler
42lb FRPP Injectors
90mm FRPP Maf
255lph SVT Focus Fuel Pump
4.10 FRPP Gears
MGW Shifter
Tokico Shocks & Struts 
Eibach Lowering Springs 
Urethane Bushings
Caster Camber Plates
17" Polished Cobra Rims
BF P275/40R17 Drag Radials
SCT Tuning by Sal of PSP

JKU3 EECV

AFTER
CAN YOU SAY @60% POWER INCREASE : o)
371HP / 371TQ SAE, 384 / 384 uncorrected
That "well over" 400/400 to the Flywheel
and an awesome HP to Weight Ratio ; o )
The 99 Convertible has a Curb Weight of 3,424 lbs