Equipment Block ID:

11-A-34-208

Equipment Hierarchy:

208-32

Project Description:

Preventive Maintenance

Job Description:

Inspect Production Line 101

| Frequency: | Monthly |
| --- | --- |

| Estimated Craft Hours: | 1 x 2.0 |  | Estimated Elapsed Time: | 2.0 |
| --- | --- | --- | --- | --- |
| Estimated Production Downtime: |  | 2 |  |  |

| Originator: | Tony Millsapp | Origination Date: |  | 06/26/2018 |
| --- | --- | --- | --- | --- |
| Owner: | Maint Value Steam Ldr | Version #: |  | 1 |
| Previous Version(s) Modifications: |  |  |  |  |
| Approval: | RS | Version #: | 1.0 |  |

| Warnings: | Lock Out Tag Out – Could result in serious injury |
| --- | --- |
| Cautions: | Failure to follow procedures could result in product contamination |

| Personal Protective Equipment Required: | Safety Glasses, Clean Gloves, Hearing Protection |
| --- | --- |

| Part # (Stores ID) | Part Description | Quantity | Quantity Description |
| --- | --- | --- | --- |
| Maintenance Shop | FMO-500-AW Oil | 34 | ounces |
| Maintenance Shop | Shop Towels |  |  |
| 532140156 | Oil Filter | 1 | each |
| 5003859 | Oil Filter Element | 1 | each |
| 500038591 | Oil Filter Canister | 1 | each |

Consumables Needed:

Degreaser, paper towels

Special Tools Required:

10mm Wrench, 7mm wrench, Oil Filter Wrench, Catch Pan

Mobile/Special Equipment:

None

Required Departmental Coordination:

Downtime

Other Procedures Referenced:

Lockout Tagout, SQF

| ID | Description | Craft | # of Crafts | Clock Hours | Craft Hours |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Lockout Tagout | M | 1 | 2 | 2 |  |
| 2 | Insert 2’ pry bar between plates to check for movement. Is any movement present? Yes ______ No ______ |  |  |  |  |  |
| 3 | Inspect sprocket | Mech | 1 | 0.3 | 0.3 |  |
| 3-1 | Visually inspect for: Cracks Yes ______ No _______ Broken Teeth Yes ______ No _______ Visible Signs of Wear? If indicated, report findings below and to immediate supervisor for appropriate actions |  |  |  |  |  |
| 4 | Inspect retainer cap | Mech | 1 | 0.2 | 0.2 |  |
| 4-1 | Visually inspect for broken bolts Are there any broken bolts? Yes ______ No _______ |  |  |  |  |  |
| 4-2 | If broken bolts are found, replace as required Torque bolts to 80 ft. lbs |  |  |  |  |  |
| Condition (As Found): |  |  |  |  |  |  |
| Condition (As Left): |  |  |  |  |  |  |
| Comment(s): |  |  |  |  |  |  |

Craft’s Feedback on Procedures:

Craft’s Signature(s):

Date: