Equipment Block ID:

SeaRock Mine, 2-ROTOR MARIETTA MINERS

Equipment Hierarchy:

ES60XXX Miner

Project Description:

2-Rotor Miner Front End Inspection

Job Description:

Inspect shear pin plates

| Frequency: | Monthly |
| --- | --- |

| Estimated Craft Hours: | 1 x 1.0 |  | Estimated Elapsed Time: | 1.0 |
| --- | --- | --- | --- | --- |
| Estimated Production Downtime: |  |  |  |  |

| Originator: | Dave Stone | Origination Date: |  | 03/12/2012 |
| --- | --- | --- | --- | --- |
| Owner: | Mine Maintenance | Version #: |  | 1 |
| Previous Version(s) Modifications: |  |  |  |  |
| Approval: | DS | Version #: | 1.0 |  |

| Warnings: | Always use face shield over safety glasses when blowing off equipment Lockout procedure must be followed |
| --- | --- |
| Cautions: | Failure to follow torque specs can result in equipment failure |

| Personal Protective Equipment Required: | Gloves, face shield, hearing protection |
| --- | --- |

| Part # (Stores ID) | Part Description | Quantity | Quantity Description |
| --- | --- | --- | --- |
| Bolt bin | 1/2” x 2” Gr. 5 socket head bolts | 6 | each |

Consumables Needed:

Degreaser, paper towels

Special Tools Required:

2’ pry bar ½” torque wrench

Mobile/Special Equipment:

Required Departmental Coordination:

Production shutdown / position / blow off equipment

Other Procedures Referenced:

Job Preparation / Lockout Procedure #XXX

| ID | Description | Craft | # of Crafts | Clock Hours | Craft Hours |
| --- | --- | --- | --- | --- | --- |
| 1 | Clean area to be inspected using compressed air or degreaser as required | Mech | 1 | 0.2 | 0.2 |
|  | Warning: use face shield when blowing with compressed air |  |  |  |  |
|  | Warning: Ensure hydraulic pump drive motor is racked out; Jog test before proceeding |  |  |  |  |
| 2 | Inspect shear pin plates | Mech | 1 | 0.3 | 0.3 |
| 2-1 | Visually check for cracks on shear pin plates Are any cracks evident Yes ______ No _______ |  |  |  |  |
| 2-2 | Insert 2’ pry bar between plates to check for movement. Is any movement present? Yes ______ No ______ |  |  |  |  |
| 3 | Inspect sprocket | Mech | 1 | 0.3 | 0.3 |
| 3-1 | Visually inspect for: Cracks Yes ______ No _______ Broken Teeth Yes ______ No _______ Visible Signs of Wear? If indicated, report findings below and to immediate supervisor for appropriate actions |  |  |  |  |
| 4 | Inspect retainer cap | Mech | 1 | 0.2 | 0.2 |
| 4-1 | Visually inspect for broken bolts Are there any broken bolts? Yes ______ No _______ |  |  |  |  |
| 4-2 | If broken bolts are found, replace as required Torque bolts to 80 ft. lbs |  |  |  |  |

Condition (As Found):

Condition (As Left):

Comment(s):

Craft’s Feedback on Procedures:

Craft’s Signature(s):

Date: