Limited LATEMODEL RULES

RULE BOOK DISCLAIMER

The rules and/or regulations set forth herein are designed to provide for the orderly conduct of racing events and to establish minimum acceptable requirements for such events. These rules shall govern the condition of all events, and by participating in these events, all participants are deemed to have complied with these rules. NO EXPRESSED OR IMPLIED WARRANTY OF SAFETY SHALL RESULT FROM PUBLICATIONS OF OR COMPLIANCE WITH THESE RULES AND/OR REGULATIONS. They are intended as a guide for the conduct of the sport and are in no way a guarantee against injury or death to a participant, spectator or official.

The race director shall be empowered to permit reasonable and appropriate deviation from any of the specifications herein or impose any further restrictions that in his/her opinion does not alter the minimum acceptable requirements. NO EXPRESSED OR IMPLIED WARRANTY OF SAFETY SHALL RESULT FROM SUCH ALTERATION OF SPECIFICATIONS.  Any interpretation or deviation of these rules is left to the discretion of the officials. Their decision is final.

The Management of Golden Mountain Speedway.

NO TRACTION CONTROL DEVICES OF ANY KIND!

Anyone caught will be DQ for that night and forfeit of winnings. A second offense will result in a three-week ban.

BODY.

All 2021 Lucas body rules apply

Standard Dirt Style body required with a stock looking nose

Rear Deck Height maximum 39 inches, (NO Tolerance)

Spoilers: Maximum 8 inches of height and 72 inches of width

No more than 3 spoiler supports may be used

Wheelbase 103 inches Minimum

WEIGHT

2300 lbs. – No Burn-off 604/602

2350 lbs. -Flat Tappet Sportsman Engine

2400 lbs. -Standard runner Aluminum Heads**

2450 lbs. -Raised runner Steelheads

2400 lbs. with 50 lbs. in front Midplate-CT 525 Crate

  - 2400lbs-Standard runner steelhead and stock SAS Spec heads

  - 2400lbs-Standard runner Aluminum head cannot exceed a 60/40 valve spacing

ALL Weights are after the race

  - 1 pound per lap burn off in feature only

ENGINE – STEEL/ALUMINUM HEAD

All Chevy heads 23 degrees-based design and Ford Based 20 degrees

ALL ENGINES MUST HAVE STEEL BLOCK

365 Maximum cubic inches

No aluminum raised runner head’s

No Mullin’s aluminum outlaw heads

Steel/ Aluminum head Standard Runner

Steel Raised Runner head

Any Cam

Any Lifters

Any Piston

NO Titanium or Aluminum Rods

Any bore and stroke combination 365 cubic inches

All Heads can be ported

All engines must be naturally aspirated

Any Oil system

Any Rocker Arms

All Chevy heads must be 23 degree-based (Angle Milled Allowed)

Ford heads must be 20 degree-based steel head (Angle Milled Allowed)

Ford Motor Sports N Head Allowed

Ford Crate 2350lbs, must be as come from Ford

All Crate engines can be bored no more than 030

Intake- Any single four-barrel intake permitted (May port and polish)

Maximum engine set back no more than six (6) inches from center of top ball joint to number 1 spark plug

ENGINE- GM CRATE

602/604

Sealed or Unsealed MUST meet GM specifications

ENGINE- SPORTSMAN TYPE MOTOR

MUST have flat tappet Cam

MUST be wet sump oil system

NO shaft mount rocker arms

ENGINE- CT 525

CT525 Fuel Sealed Engine ALLOWED

CARBURETOR

Any 4150 series-based carburetor

No injection

Carburetor spacer maximum of 2 inches (any material)

FUEL

Racing gasoline and E-85 ONLY

NO Alcohol, NO Nitrous Oxide, or other fuel additives

Fuel Cell: All fuel cells must be mounted inside a 20-gauge metal box, supported by two (2) 1/8 x 2” steel straps, mounted inside the protective structure of the vehicle

All fuel cells must have a flap on top with a ball valve to keep fuel from running out if the car is upside down

Fuel cell must have the pick-up line coming from the top half of the cell.

TRANSMISSION

Drive train must have transmission, mounted on the rear of the engine

NO direct drive or in-boxes

Any Transmission, with working forward and reverse gears permitted, and must lead to one driveshaft

All drive shafts must be painted white or silver and must be protected by a minimum of one safety hoop or a sling. (Carbon fiber shafts are recommended)

DRIVE SHAFT

Driveshaft must be a minimum of 2 inches diameter

REAR END

Any type of quick-change rear end, No exotic materials allowed

Any steel or aluminum tube permitted. NO lead may be bolted to the rear end

No Spring Rods

SHOCKS

No Bluetooth or computerized components

Standard Late Model Shock, limit one per wheel except Left Rear may have one in front and one in back

NO Shocks may be adjustable from the cockpit area

NO Shock arms on the rear

WHEELS

Any Aluminum wheel that is mounted by lug nuts, NO carbon fiber

Maximum wheel width 14 inches

TIRES

{Open LF Front}

Hoosier 3 & 4 RF/LR/RR

Hoosier D21&D55 RF/LR/RR

American Racer 48 and 56 or Pro 2(NO Pro 3 AR56 Allowed) RF/LR/RR

All numbers and names must remain on the tire (NO grinding off numbers and names)

No Chemical Alteration of tires allowed!!!

• Tire samples may be taken at any time from any car. (If taken prize money will be held pending results)- If caught with chemicals in tires loss of pay for race and eligibility for points money.

MUFFLERS

No mufflers required

PERSONAL SAFETY EQUIPMENT

SEAT BELTS

Car should be equipped with SFI 16.5 or SFI 16.1 approved seat belt restraint system until the date of belt expiration.

Seat belts restraint system shall be installed in accordance with the directions of system supplier or manufacturer

It is the responsibility of the driver, to ensure his/her seat belt restraint system and All components are SFI approved and correctly installed, maintained.

SEATS

High-back full containment seat should be securely mounted to frame as recommended by the manufacturer

Suitable FIA manufacturer-approved Full containment seats are permitted

Cars must have a full-containment racing seat utilizing the left and right halo protecting drivers side-to-side head and shoulder movement (25-pound weight penalty for cars competing without full containment seats)

SAFETY APPAREL

Driver must wear Fire resistant uniform meeting SFI 3.2A/5 specs and display valid SFI 3.2/5 label.

Each Driver should wear Fire resistant accessories that effectively cover the remaining parts of the body

Shoes should meet the FSI 3.3 specifications and display a valid SFI 3.2/5 label.

Gloves and window net ONLY if tracks require.

HELMETS

Driver must wear full-face helmet, with at least a valid SA 2010 or SA 20154 Standard Snell and/or valid SFI 31.1,31.2, or 31.1 2010 label always on the track when competing.

Driver should wear a helmet in accordance with the directions provided by the helmet manufacturer and/or supplier.

Head and Neck restraints are strongly recommended and should conform to manufacturer’s mounting instructions.

STRONGLY RECOMMENDED ITEMS

A Marked electrical Kill Switch and a marked fuel shut off valve in reach of the driver

An easily accessible fire extinguisher of at least 2.5 gals FFF or Equivalent.

ANY DRIVER WHO GETS OUT OF THEIR CAR ON THE RACING SURFACE WILL BE DISQUALIFIED FROM THE EVENT AND WILL NOT BE AWARDED PRIZE MONEY.

PROTEST RULES

500.00 dollars to pull ONE head, check the bore and stroke/ cubic inch (Tech Man receives 100.00 dollars, Winner of protest receives 400.00 dollars)

150.00 dollars to check Rods (Tech Man receives 50.00 dollars Winner of protest receives 100.00 dollars)

100.00 dollars to check Cam (Tech Man receives 25.00 dollars, winner of protest receives 75.00 dollars)

75.00 dollars to check weights (Post-race) (Tech Man receives 25.00 dollars, winner of protest receives 50