2025 Annual Drinking Water Quality Report

January 1 to December 31, 2025

City of Albany PWS ID. No. 2090001

City Hall (325) 762-3133      Water Treatment Plant (325) 762-2163

This report is intended to provide you with important information about your drinking water and the efforts made by the water system to provide safe drinking water.

For more information regarding this report contact: WTP Team at (325) 762-2163.

Special Notice for the Elderly, Infants, Cancer Patients, people with HIV/AIDS or other immune problems

You may be more vulnerable than the general population to certain microbial contaminants, such as Cryptosporidium, in drinking water. Infants, some elderly or immune-compromised such as those undergoing chemotherapy for cancer; those who have undergone organ transplants; those who are undergoing treatment for steroids; and people with HIV/AIDS or other immune system disorders can be particularly at risk from infections. You should seek advice about drinking water from your physician or health care provider. Additional guidelines on appropriate means to lessen the risk of infection by Cryptosporidium are available from the Safe Drinking Water Hotline (800) 426-4791.

Our Drinking Water Meets or Exceeds All Federal (EPA) Drinking Water Requirements

This report is a summary of the quality of water we provide our customers. In order to ensure that tap water is safe to drink, EPA prescribes regulations which limit the amount of certain contaminants in water provided by public water systems. FDA regulations establish limits for contaminates in bottled water which must provide the same protection for public health.  Analysis was made by using the data from the most recent U.S. Environmental Protection Agency (EPA) required tests and is presented in the attached pages. A list of all the federally regulated or monitored contaminants which have been found in your drinking water. The EPA requires water systems to test for up to 97 contaminants.  We hope this information helps you become more knowledgeable about what is in your drinking water.

All Drinking Water May Contain Contaminants

When drinking water meets federal standards there may not be any health benefits to purchasing bottled or point of use devices. Drinking water, including bottled water may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily indicate water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the EPA’s Safe Drinking Water Hotline (1-800-426-4791).

En Espanola:

Este reporte incluye informacion importante sobre el agua para tomar. Para asistencia en espanol, favor de llamar al telefono (325) 762-3133.

Public Participation Opportunities: The City of Albany Water Department is governed by the Albany City Council, which meets every second Monday of every month at 7:00 p.m. at the City Hall. For more information about how to express your opinion, you may contact City Hall at 325-762-3133.

Water Loss for 2025

In the water loss audit submitted to the Texas Water Development Board for the period of Jan-Dec 2025, our system loss an estimated 6,377,223 gallons of water. If you have any questions about the water loss, please call 325-762-2163.

Information about your Drinking Water

The sources of drinking water (both tap and bottled water), includes rivers, lakes, ponds, streams, reservoirs, springs or wells. As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals, and in some cases radioactive material and can pick up substances resulting from the presence of animals or from human activity. Contaminants that may be present in source water include: Microbial contaminants, such as viruses and bacteria, which may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife. Inorganic contaminants, such as salts and metals, which can be naturally-occurring or results from urban storm water runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming. Pesticides and Herbicides, which may come from a variety of sources such as agriculture, urban storm runoff, and residential uses. Organic chemical contaminants, including synthetic and volatile organic chemicals, which are by products of industrial processes and petroleum production, and can also, come from gas stations, urban storm water runoff, and septic systems. Radioactive contaminants, which can be naturally occurring or be the result of oil and gas production and mining activities. More information about contaminants and potential health effects can be obtained by calling the EPA’s Safe Drinking Water Hotline at (800) 426-4791.

Where do we get our drinking water?

Our drinking water is obtained from Surface water sources. City of Albany purchases raw water (untreated) from West Central Texas MWD, Hubbard Creek Reservoir in Stevens County and McCarty Lake in Shackelford County which the City of Albany owns the water rights.

TCEQ completed an assessment of your source water and results indicate that our sources are susceptible to certain contaminants. The sampling requirements for your water system are based on this susceptibility and previous sample data. Any detection of these contaminants may be found in this Consumer Confident Report. For more information on source water assessments and protection efforts at our system, contact Plant Staff at (325) 762-2163. This information describes the susceptibility and types of constituents that may contact with your drinking water source based on human activities and natural conditions. The information contained in the assessment allows us to focus source water protection strategies. Some of this source water assessment information is available on Texas Drinking Water Watch at http://ww.tceq.state.texas.gov/gis/swaview. For more information about your sources water, please refer to the Source Water Assessment Viewer available at the following URL: http://gis3.tceq.state.tx.us/swav/Controller/index.jsp?wtrsrc=

Secondary Constituents

Many constituents (such as calcium, sodium, or iron) which are often found in drinking water, can cause taste and odor problems. These taste and odor constituents are called secondary constituents and are regulated by the State of Texas, not the EPA. These types of problems are not necessarily a health concern. Therefore, secondary is not required to be reported in this document but they may greatly affect the appearance and taste of your water. For more information on taste, odor, or color of drinking water, please contact the system’s business office.

Abbreviations

NTU-Nephelometric Turbidity Units (a measure of turbidity)	                      MFL-million fibers per liter (measure of asbestos)

ppq-parts per quadrillion, pictograms per liter (pg/L)			 pCi/L-picocuries per liter (a measure of radioactivity)

ppt-parts per trillion, or nanograms per liter (ng/L)			 NA: not applicable.

ppb-parts per billion, or micrograms per liter (ug/L) or one ounce in 7,350,000 gallons of water.

Mrem: millirems per year (a measure of radiation absorbed by the body)

ppm-parts per million, or milligrams per liter (mg/L) or one ounce in 7,350 gallons of water.

Definitions: The following tables contain scientific terms and measures, some of which may require information.

Maximum Contaminant Level (MCL):  The highest level of a contaminant that is allowed in drinking water. MCLs are set as close to the MCLGs as feasible using the best available treatment technology.

Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs allow for a margin of safety.

Maximum Residual Disinfectant Level (MRDL): The highest level of disinfectant allowed in drinking water. There is convincing evidence that addition of disinfectant is necessary for control of microbial contaminants.

Maximum Residual Disinfectant Level Goal (MRDLG): The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants.

Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow.

Action Level Goal (ALG): The level of a contaminant in drinking water below which there is no known or expected risk to health. ALGs allow for a margin of safety.

Avg: Regulatory compliance with some MCLs, based on running annual average of monthly samples.

Level 1 Assessment: A level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system.

Level 2 Assessment: A Level 2 Assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions.

Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water.

Cryptosporidium Monitoring Information

Not yet tested for. Cryptosporidium, a microbial parasite that might be commonly found in surface water. Cryptosporidium may come from animal and human feces in the watershed.

| Year | Inorganic Contaminants | Highest Level Detected | Range of Levels Detected | MCLG | MCL | Units | Violation | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2025 | Arsenic | 1 | 1 | 0 | 10 | ppb | No | Erosion of natural deposits; runoff from orchards, glass production and electronic production waste |
| 2025 | Barium | .2 | .2 | 2 | 2 | ppm | No | Discharge from drilling waste; discharge from metal refineries; erosion of natural deposits. |
| 2025 | Fluoride | .34 | .34 | 4 | 4 | ppm | No | Erosion of natural deposits; additive which promotes teeth; discharge from fertilizer and aluminum factories. |
| 2025` | Nitrate [measured as Nitrogen] | .32 | .32/.13 | 10 | 10 | ppm | No | Runoff from fertilizer used; leaching from septic tanks, sewage, and erosion of natural deposits. |
| 2025 | Selenium | 5 | 5 | 50 | 50 | ppb | No | Discharge from petroleum and metal refineries; erosion of natural deposits; discharge from mines. |
| 2025 | Chromium | 1 | 1 | 100 | 100 | ppb | No | Discharge from steel and pulp mills; Erosion of natural deposits. |
| 2025 | Cyanide | 52.8 | 52.8 | 200 | 200 | ppb | No | Discharge from plastic and fertilizer factories; Discharge from steel/metal factories. |

Nitrate Advisory: Nitrate in drinking water at levels above 10 ppm is a health risk for infants of less than six months of age. High nitrate levels in drinking water can cause Blue Baby Syndrome. Nitrate levels may rise quickly for short periods of time because of rainfall or agricultural activity. If you are caring for an infant, you should ask advice from your health care provider.

| Year | Disinfection Byproducts | Highest Level | Range of Levels | MCLG | MCL | Unit | Violations | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2025 | Haloacetic Acids | 21 | 15.5/21 | No goal for the total | 60 | ppb | No | By-product of drinking water Disinfection. |
| 2025 | Total Trihalomethanes | 46.1 | 27.2/46.1 | No goal for the total | 80 | ppb | No | By-product of drinking water Disinfection. |
| 2025 | Chlorite | .97 | .21/.97 | .8 | 1 | ppm | No | By-product of drinking water Disinfection |

| Year | Radioactive Contaminants | Highest Level | Range of Levels | MCLG | MCL | Units | Violation | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/23/24 | Gross Beta Emitters | 9.3 | 9.3 | 0 | 50 | pCi/L* | No | Decay of natural and man- made Deposits. |

*EPA considers 50 pCi/L to be the level of concern for beta particles

Unregulated Contaminants

Bromoform, Chloroform, Bromodichloromethane, and Dibromochloromethane are disinfectant byproducts. There is no maximum contaminant level for these chemicals at the entry point to the distribution system.

| Year | Contaminant | Average Level | Minimum Level | Maximum Level | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| 2025 | Chloroform | 1.11 | 1 | 1.55 | ppb | Byproduct of drinking water disinfection |
| 2025 | Bromoform | 19.46 | 13 | 24.3 | ppb | Byproduct of drinking water disinfection |
| 2025 | Bromodichloromethane | 4.79 | 3.44 | 6.49 | ppb | Byproduct of drinking water disinfection |
| 2025 | Dibromochloromethane | 12.2 | 9.58 | 15.8 | ppb | Byproduct of drinking water disinfection |

Lead and Copper

| Lead and Copper | Collection Date | MCLG | Action Level | 90th Percentile | Number of sites over AL | Units | Violation | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead | Sept. 2024 | 0 | 15 | 0 | 0 | ppb | No | Corrosion of household plumbing system; erosion of natural deposits; leaching from wood preservatives. |
| Copper | Sept. 2024 | <1 | <1 | 0 | 0 | ppm | No | Corrosion of household plumbing system; erosion of natural deposits; leaching from wood preservatives |

Additional Health Information for Lead

“If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. This water supply is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have it tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead.

| Year | Turbidity | Highest Single Measurement | Percent of samples meeting the limit | Limit | Unit of Measure | Violations | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2025 | Turbidity | .05 | 100 | .3 | NTU | No | Soil runoff |

Information Statement: Turbidity is a measure of the cloudiness of the water caused by suspended particles. We monitor it because it is a good indicator of water quality and the effectiveness of our filtration process.

Coliform Bacteria

| MCLG | Total Coliform and E. coli | Number of Samples collected | Fecal Coliform Maximum Contaminant Level | Number of Positive Samples | Violations | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Total Coliform | 24 | NA | 0 | No | Naturally present in the environment |
| 0 | E. coli | 24 | NA | 0 | No | Naturally present in the environment |

Total Organic Carbon

Total Organic Carbon (TOC) no health effects. The disinfectant can combine with TOC to form disinfection byproducts. Disinfection is necessary to ensure that water does not have unacceptable levels of pathogens. Byproducts of disinfection include trihalomethanes (THMs) and haloacetic acids (HAA) which are reported elsewhere in this report.

| Year | Contaminant | Average | Minimum | Maximum | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| 2025 | Source Water | 5.38 | 3.08 | 6.06 | ppm | Naturally occurring |

Disinfectant Level

| Year | Disinfectant Used | Average | Range of Detection (low/High) | MRDL | MCDL | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2025 | Chloramines | 3.29 | 1.13/3.84 | 4 | <4 | ppm | Disinfectant used to control microbes |
| 2025 | Chlorine Dioxide | 0.01 | 0.0/.13 | .8 | .8 | ppm | Disinfectant used to control microbes |

Secondary and Other Non-Regulated Constituents (No associated adverse health effects)

| Year | Constituent | Average | Minimum | Maximum | Limit | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2025 | Bicarbonate | 109 | 109 | 109 | NA | ppm | Corrosion of carbonate rocks such as limestone |
| 2025 | Calcium | 56.8 | 56.8 | 56.8 | NA | ppm | Abundant naturally occurring element |
| 2025 | Chloride | 178 | 178 | 178 | 300 | ppm | Abundant naturally occurring element; used in water purification and byproducts of oilfield activity |
| 2025 | Iron | 0.05 | 0.05 | 0.05 | .3 | ppm | Erosion of natural deposits; iron or steel water delivery equipment or facilities |
| 2025 | Magnesium | 20.2 | 20.2 | 20.2 | NA | ppm | Abundant naturally occurring element |
| 2025 | Nickel | .001 | .001 | .001 | NA | ppm | Erosion of natural deposits |
| 2025 | pH | 7.5 | 7.2 | 7.9 | 7 | Units | Measure of corrosivity of water |
| 2025 | Sodium | 87.9 | 87.9 | 87.9 | NA | ppm | Erosion of natural deposits; byproduct of oilfield activity |
| 2025 | Sulfate | 70.1 | 70.1 | 70.1 | 300 | ppm | Naturally occurring; common industrial byproduct; byproduct of oilfield activity |
| 2025 | Total Alkalinity as CaC03 | 109 | 109 | 109 | NA | ppm | Naturally occurring soluble mineral salts |
| 2025 | Total Dissolved Solids | 493 | 493 | 493 | 1000 | ppm | Total dissolved mineral constituents in water |
| 2025 | Total hardness as CaC03 | 225 | 225 | 225 | NA | ppm | Naturally occurring calcium |

Interim Enhanced SWTR The Interim Enhanced Rule improves control of microbial contaminants, particularly Cryptosporidium, in systems using surface water, or ground water under the direct influence of surface water. The rule builds upon the treatment technique requirements of the Surface Water Treatment Rule.

Lead Service Line Inventory

The City of Albany has developed	 an inventory of both city owned and customer owned service lines. This inventory serves as a crucial foundation for water systems to address a significant source of lead in drinking water. To access the inventory, please contact Brian Holson at City Hall (325) 762-3133.