RV COVERS: USAGE and CARE

To be used on CLEAN Vehicles ONLY. PLACE COVER OVER A COOLED DOWN VEHICLE ONLY.

Freshly painted vehicles MUST be fully cured before using a Cover.

TO INSTALL COVER:  Please see Instruction Sheet provided with your Cover.

USE AND CARE AND SUGGESTIONS

1. FOLD IN THE MIRRORS and REMOVE ANTENNA Mast

2. DO NOT USE ANY FORM OF HEAT ON the PREMIUM OUTDOOR MATERIAL—THIS MATERIAL WILL MELT.

3. Use care if you must remove a cover that is frozen to your unit. (you can use warm water to help loosen any frozen material)

4. Washing your cover is easy:

- Your PREMIUM OUTDOOR MATERIAL COVER- CAN BE HOSED DOWN FROM A CLOTHES LINE  OR CAN BE WASHED IN A COMMERICAL FRONT LOADER WASHER. DO NOT PUT IN A DRYER-HEAT WILL MELT THIS MATERIAL. ALLOW TO AIR DRY BEFORE PUTTING BACK ONTO UNIT…DO NOT allow brushes, sponges, or any harsh chemicals  to contact the  MATERIAL and use cold Water and a very small amount of Fantastic Cleaner ONLY.

5. If any leakage develops, remove cover from vehicle and spray the EXTERIOR OF THE COVER “ONLY” with a very very light dusting only! Using a scotch guard or similar water repellent treatment (You do not want to overspray as this will cause the “Important Breathability” of the cover to FAIL)

5. Moisture may develop under a PREMIUM OUTDOOR MATERIAL cover as a result of changes in humidity or during heavier rainfalls. This will evaporate through the cover when weather improves.

6. DO NOT USE THIS COVER if your vehicle has recently been PAINTED; consult the paint manufacturer and/or body shop with regards to curing and drying times. Today’s paints will require different times to fully dry and cure due to the wide range of additives and catalysts that are used in the painting and finishing process.

7. Safety first – ensure engine and exhaust are completely COOLED before installing the cover.

8. IMPORTANT—All Sharp objects, antennas, stress points or jagged items MUST BE covered or re-enforced before installing cover to prevent tearing or ripping the material which is not covered by warranty.

The Warranty of all Hugger Covers is determined by the material type.  All “Hugger Covers” are guaranteed to be free from workmanship for a period of 1 year and material defects for a period of three years (pro-rated).  Abnormal Wear and Tear and improper use are not covered under Warranty.  Hugger Covers Inc. extends the manufactures warranty to the original purchaser for original material only.  Manufactures policies and procedures may vary.  Because each Hugger Cover is Custom Made for You, it cannot be returned or exchanged.  Since customer satisfaction is our top priority, we will be happy to make any necessary alterations. Shipping costs in both directions is not covered.

Hugger Covers Inc. shall be in no way responsible for the improper use of any merchandise purchased.

YOUR NEW RV COVER HAS BEEN MADE WITH QUALITY MATERIALS.

PROPER CARE WILL GIVE YOU MANY YEARS OF SERVICE.

Hugger Covers Inc.

365 Healey Road, Unit # 28, Bolton, Ontario L7E 5C1

Telephone: 905.857.1800 Fax: 905.857.1805 Email:  info@huggercovers.com

www.huggercovers.com