CAR COVERS:   USAGE and CARE

To be used on CLEAN Vehicles ONLY. PLACE COVER OVER A COOLED DOWN VEHICLE ONLY.

Freshly painted vehicles MUST be fully cured before using a Cover.

ANTENNA GROMMET/PATCH APPLICATION: FOR NON RETRACTING ANTENNA’S

1. Open the cover on the hood of the vehicle. The “Hugger Cover" crest should be located at the front of the vehicle

2. Unroll the cover over your vehicle, pulling it down over the sides and then over the bumper at the rear of the vehicle (if there

is a rear mount antenna, pull the cover over the front bumper instead).

3. Pull the cover as far over the end of the vehicle as possible with the antenna on the left front fender, for example, the cover

may go completely over the right fender and bumper. You will now find the antenna holding the cover back so that you cannot

complete the installation.

4. By judging the seams on both sides of the vehicle, you can now easily see where you must using a pen or pencil, mark where

your X / hole will be. Remove the cover to a work surface. Just cut an "X" using scissors or a small knife in the fabric.  With patience

insert self piercing grommet from the top of cover. Ensure all 4 prongs are now pierced through to the underneath of the cover.  You

may want to use a large sewing needle to help widen the material for the prongs to easily slide thru.  Turn the cover over and insert

the flat backing of the grommet to the 4 prongs to complete the application. You will still have the 4 prongs sticking towards your paint. Insert an X on the enclosed round patch and use it as protection from the prongs.  You can either attach the round patch with

a fabric glue or simply tack it down with a needle and thread.  Your cover can now be placed over the car allowing the antenna to protrude through the hole/grommet.

CARE AND SUGGESTIONS

1. Stress points such as sharp hood ornaments and protruding license plates must be reinforced using rags, sponges or towels.

DO NOT iron onto and DO NOT USE ANY FORM OF HEAT ON the PREMIUM OUTDOOR MATERIAL—THIS MATERIAL WILL MELT.

2. Use care if you must remove a cover that is frozen to your car (also. you can use warm water to help loosen any frozen material)

3. Washing your cover is easy:

a) Cloth cotton covers may be washed in cold water in a front loader washer. Air dry ONLY. (Dryer’s may shrink material)

(Covers with colored cotton material and/or combinations may bleed coloring, wash at own risk or dry clean only)

b) PREMIUM OUTDOOR MATERIAL CAN BE WASHED IN A COMMERICAL FRONT LOADER WASHER. DO NOT PUT IN A DRYER—ALLOW TO AIR DRY BEFORE PUTTING BACK ONTO VEHICLE…DO NOT allow brushes, sponges, or any harsh chemicals to contact the MATERIAL use cold Water and a very small amount of Fantastic Cleaner ONLY.   If any leakage develops, remove cover from vehicle and spray the EXTERIOR ONLY very very lightly with scotch guard or similar water repellent treatment (You do not want to overspray as this will cause the “Important Breathability” of the cover to FAIL)

4. Moisture may develop under a PREMIUM OUTDOOR MATERIAL cover as a result of changes in humidity or during heavier rainfalls. This will evaporate through the cover when weather improves.

5. DO NOT USE THIS COVER if your vehicle has recently been PAINTED; consult the paint manufacturer and/or body shop with regards to curing and drying times. Today’s paints will require different times to fully dry and cure due to the wide range of additives and catalysts that are used in the painting and finishing process.

6. Safety first – ensure engine and exhaust are completely COOLED before installing the cover.

7. DO NOTTRAILER your vehicle with the cover on the vehicle.

The Warranty of all Hugger Covers is determined by the material type.  All “Hugger Covers” are guaranteed to be free from workmanship for one year and material defects for a period of three years (Pro-Rated).  Abnormal Wear and Tear and improper use are not covered under Warranty.  Hugger Covers Inc. extends the manufactures warranty to the original purchaser for original material only for a period of 3 years (Pro-Rated).  Manufactures policies and procedures may vary.  Because each Hugger Cover is Custom Made for You, they cannot be returned or exchanged.  Since customer satisfaction is our top priority, we will be happy to make any necessary alterations. Shipping costs in both directions is not covered.

Hugger Covers Inc. shall be in no way responsible for the improper use of any merchandise purchased.

YOUR NEW CAR COVER HAS BEEN MADE WITH QUALITY MATERIALS.

PROPER CARE THAT WILL GIVE YOU MANY YEARS OF SERVICE.

Hugger Covers Inc.

365 Healey Road, Unit # 28, Bolton, Ontario L7E 5C1

Telephone: 905.857.1800 Fax: 905.857.1805 Email:  info@huggercovers.com

www.huggercovers.com