SECTION 09 65 19

RESILIENT FLOORING

PART 1 – GENERAL

1.1 RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

Section Includes:

Cabrillo Core Series resilient flooring systems, including glue-down and click-lock products.

1.3 SUBMITTALS

Product Data, SDS, Warranty, Installation Instructions.

Samples for Initial Selection.

Samples for Verification: Three (3) samples.

1.4 QUALITY ASSURANCE

Installers shall be trained or approved by Catalina Products International (CPI).

1.5 DELIVERY, STORAGE, AND HANDLING

Store materials indoors at 65°F–85°F for 48 hours before, during, and after installation.

1.6 PROJECT CONDITIONS

Maintain ambient temperature 65°F–85°F.

Relative humidity 35%–60%.

Subfloor must be clean, flat, dry, and rigid.

PART 2 – PRODUCTS

2.1 MANUFACTURER

Catalina Products International

1215 N. Redgum Street

Anaheim, CA 92806

www.catalinaproducts.international

2.2 CABRILLO CORE – GLUE-DOWN SERIES

ASTM F1700 – Solid Vinyl Floor Tile

2.2.A Cabrillo Core

Thickness: 2.0 mm

Wear Layer: 12 mil

Warranty:

- Lifetime Residential

- 10-Year Light Commercial

2.2.B Cabrillo Core Select

Thickness: 3.0 mm

Wear Layer: 20 mil

Warranty:

- Lifetime Residential

- 15-Year Light Commercial

2.3 CABRILLO CORE – CLICK-LOCK SERIES

ASTM F1700 – Solid Vinyl Floor Tile

2.3.A Cabrillo Core Select +

Overall Thickness: 5.5 mm (4.0 mm plank + 1.5 mm attached pad)

Wear Layer: 12 mil

Warranty:

- Lifetime Residential

- 10-Year Light Commercial

2.3.B Cabrillo Core Optimum

Overall Thickness: 6.0 mm (4.5 mm plank + 1.5 mm attached pad)

Wear Layer: 20 mil

Warranty:

- 20-Year Residential

- 5-Year Light Commercial

2.4 INSTALLATION MATERIALS

Approved patching, leveling compounds, and CPI adhesives (glue-down products only).

PART 3 – EXECUTION

3.1 INSTALLATION

Install flooring in accordance with manufacturer instructions.

Glue-down products require approved CPI adhesives.

Click-lock products shall be installed as floating floors.

3.2 CLEANING AND PROTECTION

No traffic for 24 hours.

Begin maintenance after initial cleaning.