SECTION 09 65 19

RESILIENT TILE FLOORING

PART 1 – GENERAL

1.1 RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

Section Includes:

Heavy Duty Tile (HDT®)

1.3 SUBMITTALS

Product Data, SDS, Warranty, Installation Instructions.

Samples for Initial Selection.

Samples for Verification: Three (3) samples.

1.4 QUALITY ASSURANCE

Installers shall be trained or approved by Catalina Products International (CPI).

1.5 DELIVERY, STORAGE, AND HANDLING

Store materials in dry enclosed spaces at 65°F–85°F for 48 hours before, during, and after installation.

1.6 PROJECT CONDITIONS

Maintain ambient temperature 65°F–85°F.

Minimum substrate temperature: 50°F.

Relative humidity: 35%–60%.

Subfloor must be clean, flat, dry, and rigid.

PART 2 – PRODUCTS

2.1 MANUFACTURER

Catalina Products International

1215 N. Redgum Street

Anaheim, CA 92806

www.catalinaproducts.international

2.2 HEAVY DUTY TILE (HDT®)

Multilayer heterogeneous resilient tile with hardened ceramic-reinforced wear surface.

ASTM F1700

Thickness: 3.0 mm

Wear Layer: 28 mil

Warranty: 15-Year Heavy Commercial

No substitutions.

2.3 INSTALLATION MATERIALS

Approved patching, leveling compounds, and CPI adhesives.

PART 3 – EXECUTION

Install in accordance with manufacturer instructions.

No traffic for 24 hours.