MODEL QUESTION PAPER

II SEMESTER CIVIL ENGINEERING

# SURVEYING I

## TIME: 3 hours								MAX MARKS: 100

ALL QUESTIONS ARE COMPULSARY

- a) Explain geodetic surveying

b) Write a note on classification of survey based on instruments used

- Explain indirect ranging and illustrate with a neat diagram.

- Explain the working of the Prism Square with a neat sketch

### OR

A 20m chain was found to be10 cm too long after chaining a distance of 1500m. It was found to be 18cm too long at the end of days work after chaining a total distance of 2900m. Find the true distance if the chain was correct before the commencement of the work

- Explain the following terms

    - Base line

    - Check line

    - Tie line

    - Swing offset

    - Oblique offset

#### OR

Plot the following cross staff survey of a field ABCDEFG and calculate its area

750D

650	 210E

C180		490

- 250F

B160		180

- 50G

0A

- a) Define 1) fore bearing 2) Magnetic meridian 3) Arbitrary meridian 4) True bearing.

- Explain the prismatic compass with a neat sketch.

- a) Convert the following QB to WCB and WCB to QB

    - N30E

    - N15W

    - S30W

    - 225

    - 75

b) Explain local attraction and causes for local attraction.

##### OR

The following bearings were observed while traversing with a compass

| LINE | FB | BB |
| --- | --- | --- |
| AB | 45 45 | 226 10 |
| BC | 96 55 | 277 5 |
| CD | 29 45 | 209 10 |
| DE | 324 48 | 144 48 |

Mention which stations were affected by local attraction and determine the corrected bearings.

8.	 Write note on errors in compass surveying

# OR

The following are bearings taken on a closed compass traverse

| LINE | FB | BB |
| --- | --- | --- |
| AB | 18 10 | 259 0 |
| BC | 120 20 | 310 50 |
| CD | 170 50 | 350 50 |
| DE | 230 10 | 49 30 |
| EA | 310 20 | 130 15 |

Compute the interior angles and correct them for observed errors. Assuming the observed bearing of the line CD to be correct adjust the bearing of the remaining sides.

9. Define 1) bench mark 2) fore sight 3) height of instrument 4) intermediate station

10. 	a) Explain temporary adjustment of level

b) Describe height of instrument method of booking and reducing levels.

##### OR

The following staff readings were observed successively with a level, the instrument having been moved after third, sixth and eighth readings

2.225; 1.605; 0.985; 2.090; 2.865; 1.265; 0.600; 1.985; 1.045; 2.685 meters

enter the above readings in a page of a level book and calculate the RL of points if the first reading was taken with a staff held on a bench mark of 432.385m.

MODEL QUESTION PAPER

I SEMESTER CIVIL ENGINEERING

# SURVEYING I

## TIME: 180 MIN								MAX MARKS: 100

ALL QUESTIONS ARE COMPULSARY

| SECTION I |  |  |
| --- | --- | --- |
| 1 | a) Explain geodetic surveying b) Write a note on classification of survey based on instruments used | 5+5 |
| 2 | Explain indirect ranging and illustrate with a neat diagram | 10 |
| 3 | Explain the working of the Prism Square with a neat sketch ### OR A 20m chain was found to be10 cm too long after chaining a distance of 1500m. It was found to be 18cm too long at the end of days work after chaining a total distance of 2900m. find the true distance if the chain was correct before the commencement of the work | 10 |
| 4 | Explain the following terms - Base line - Check line - Tie line - Swing offset - Oblique offset #### OR Plot the following cross staff survey of a field ABCDEFG and calculate its area 750D 650 210E C180 490 - 250F B160 180 - 50G 0A | 10 |
| SECTION II |  |  |
| 5 | a) Define 1) fore bearing 2) Magnetic meridian 3) Arbitrary meridian 4) True bearing | 10 |
| 6 | Explain the prismatic compass with a neat sketch. | 10 |
| 7 | a) Convert the following QB to WCB and WCB to QB - N30E - N15W - S30W - 225 - 75 b) Explain local attraction and causes for local attraction. ##### OR The following bearings were observed while traversing with a compass LINE; FB; BB. AB; 45 45; 226 10. BC; 96 55; 277 5. CD; 29 45; 209 10. DE; 324 48; 144 48. Mention which stations were affected by local attraction and determine the corrected bearings. | 10 |
| 8 | Write note on errors in compass surveying # OR The following are bearings taken on a closed compass traverse LINE; FB; BB. AB; 18 10; 259 0. BC; 120 20; 310 50. CD; 170 50; 350 50. DE; 230 10; 49 30. EA; 310 20; 130 15. Compute the interior angles and correct them for observed errors. Assuming the observed bearing of the line CD to be correct adjust the bearing of the remaining sides. | 10 |
| SECTION III |  |  |
| 9 | Define 1) bench mark 2) fore sight 3) height of instrument 4) intermediate station | 10 |
| 10 | a) Explain temporary adjustment of level b) Describe height of instrument method of booking and reducing levels. ##### OR The following staff readings were observed successively with a level, the instrument having been moved after third, sixth and eighth readings 2.225; 1.605; 0.985; 2.090; 2.865; 1.265; 0.600; 1.985; 1.045; 2.685 meters Enter the above readings in a page of a level book and calculate the RL of points if the first reading was taken with a staff held on a benchmark of 432.385m. | 10 |