MODEL QUESTION PAPER

ENGINEERING DRAWING(CIVIL)

# II SEMESTER CIVIL

## DURATION: 4 HRS						MAX MARKS :100

INSTRUCTIONS:

- ANSWER ALL THE SECTIONS

- Q.NO 1, 4 , 7 , 10 ARE COMPULSORY

- ANSWER MINIMUM OF TWO QUESTIONS IN EACH SECTION

| Q.NO | # SECTION-I |  |
| --- | --- | --- |
| 1 | Construct a diagonal scale to RF 1/40000 long enough measure 6 km and mark on it a distance of 4.37 km and 5.43 km | 10 |
| 2 | Print the following statement to an height of 18mm in single stroke vertical capital letters # “ SURVIVAL OF THE FITTEST” | 10 |
| 3 | Draw the given figure.1 to a scale of 2:1 and dimension by aligned system of dimensioning | 10 |
|  | # SECTION-II |  |
| 4 | Inscribe five equal circles in a regular pentagon of 50mm side , each circle touching one of its side of a pentagon and other two circles | 10 |
| 5 | Draw an ellipse having a major axis of 110 mm and minor axis of 70 mm using concentric circles method | 10 |
| 6 | A highway bridge of parabolic shape is to construct with a span of 10m and rise of 5m | 10 |
|  | # SECTION-III |  |
| 7 | (a) A line PQ 70mm long is parallel to HP and inclined at 30  to VP . The end P is 25mm above HP and 40mm in front of VP. Draw the projections | 05 |
|  | (b) A pentagonal lamina of side 25mm is resting on one of its corner on HP such that the corner containing the edges makes equal inclination. The lamina is inclined at an angle of 30  to HP and the edges opposite to the corner makes an angle 45 to VP . Draw its projections | 10 |
| 8 | (a) One end of line PQ 55mm long is 35mm in front of the VP and 25mm above HP . The line is inclined at 40 to HP and 30 to VP. Draw its projections | 07 |
|  | (b) A square lamina PQRS of side 40mm rests on the ground in its corner P in such a way that the diagonal PR is inclined at 45 to HP and apparently inclined at 30 to VP. Draw its projections | 08 |
| 9 | A square prism of base 40mm and axis length 60mm rests on the HP on one of its base edges with its axis inclined at 45 to HP and parallel to the VP . Draw its projections | 15 |
|  | # SECTION-IV |  |
| 10 | Draw isometric projection of an object as shown in figure 2 | 15 |
| 11 | Draw the isometric view of an object shown in figure 3 | 15 |
| 12 | A rectangular block 30mm X 20mm X 15mm is lying on the ground on one of its largest faces. A vertical edge is in PP and longer edge containing that face makes an angle of 30 to the picture plane. The station point is 50mm in front of PP , 30mm above ground and lies in central plane which passes through the center of the block. Draw the perspective view by two point angular method | 15 |