Silver Staining Protocol

Alternatively, one can use the SilverQuest kit by Invitrogen or another MS compatible staining kit

Note: Do Not fix the gel with gluteraldehyde or formaldehyde; both irreversibly  modify the protein, yielding it unrecoverable

- Fix gel in 50% methanol, 5% acetic acid in ddH2O for 20 minutes

- Wash for 10 minutes with 50% methanol followed by a 10 minutes wash with ddH2O to remove remaining acid

- Sensitize gel for 1 minute by incubation in 0.02% sodium thiosulfate

- Rinse with two changes of HPLC grade ddH2O for 1 minute each

- Submerge gel in chilled 0.1% silver nitrate solution and incubate for 30 minutes at 4C

- After incubation the silver nitrate is discarded and the gel slab rinsed twice with fresh ddH2O for 1 minute (critical: no longer than 1 minute maximum each rinse).

- Develop in fresh 0.04% formalin in 2% sodium carbonate. Add an initial amount sufficient to cover and then discard within the first 30-60 sec. Replace with enough solution to continue development easily with intensive shaking

- After developer turns yellow it is discarded and replaced with fresh developer.  The solution must remain absolutely transparent at all times.

- After staining intensity is satisfactory (typically 1-5min) terminate the development by discarding the developer and washing the gel slab with 5% acetic acid

- Wrap the gel with clean Sara wrap. Acquiring gel image can be done with a scanner. Do Not touch the gel without gloves

- Store in 1% acetic acid until bands are excised