Kobelco with Isuzu Installation

1- Remove panels behind seat in cab.

2- Locate the obd2 diagnostic connector, we will be splicing into two wires behind it We will be splicing into 6 and 14 pin as shown in picture.

3- Splice the green from the Def Link box to the white wire pin 14. 4- Splice the white from the Def Link box to red wire pin 6.

5- Locate 24v fused power in cab black is ground red is power connect to the Def Link box Red  and Black wires.

6- Locate dcu controller in cab and unplug the Bosch Isuzu controller plugs located on the door  side of the cab. Tie off and leave disconnected.

7- Turn key on and verify def gauge goes to about 3/4 full.