TECHNICAL SPECIFICATION SUMMARY

HEIDELBERG CYLINDER SBG

Paper Handling

Maximum speed		            	4000 Sheets Per Hour (SPH)

Inverter Drive					No

Maximum sheet size				560 mm x 770 mm

Minimum sheet size				160 mm x 230 mm

Maximum foiling area                                     550mm x  710mm

Maximum sheet thickness			1.00 mm

Minimum sheet thickness			airmail

Minimum grip distance			10 mm

Height of feeder pile				400 mm

Height of delivery pile			490 mm

Foil Handling

Maximum foil diameter			90 mm = 305Mtrs metallic foil

Foil Core internal diameter			25.4 mm

Minimum distance between foil rolls		6.00 mm

Number of foil rewind units (standard)	2

Maximum foil roll width                                560mm

Step and repeat up to 			            99steps

Number of pulls per length			1

Pull resolution					0.25 mm

Hologram Registration System (optional)	NO

Foil run out warning				Yes

Impression counter				Yes

Speed control for foil draw			Yes

Heated Bed

Recast						Yes

Honeycomb					1 Honeycomb

In-fill heat plate				No

Heat zones					4

Sensors per bed				4

Maximum temperature			200°C

Die Height					7 mm

General Data

Height						1.65 M

Width						2,1  M

Length						3.7 M

Weight						5500 Kgs approximately

Power Supply					3 phase 50 Hz 380 - 440 V 22 kW