Daily Excavation Checklist

NOTE:  The following checklist must be completed daily, during adverse weather conditions, such as rain, where the excavation is subject to water accumulations or run off, and any time conditions within the excavation change.  This checklist is to be completed by a “competent person”.  Please refer to the company excavation program for more details on competent person training.  This checklist must be completed prior to excavation entry!  The competent person completing this checklist has full authority to take prompt corrective measures to eliminate any working conditions identified by this checklist as unsanitary, hazardous, or dangerous to employees.

Project location: ___________________________________________

Weather: __________________________	Date: _________________

Measurements of Trench (in feet):  Depth: _____	Length: _____	Width: _____

Soil Type: ______________ (See Appendix A of Excavation Program.)

Type of Protective System Used: _________________________________________

# General Inspection of the Job site

Yes	No	N/A

___	___	___	Excavations, adjacent areas, and Protective Systems inspected by the Competent Person daily, prior to the start of work.

___	___	___	Surface encumbrances are supported or removed.

___	___	___	Employees are protected from loose rock or soil that could possibly pose a hazard by falling or rolling into the excavation.

___	___	___	All employees are wearing hard hats.

___	___	___	Spoils, materials, and equipment are set back a minimum of 2' from the	edge of the excavation.

___	___	___	Barriers are provided at all remote excavations, wells, pits, shafts, etc.  Walkways over excavations 4' or more in depth, are equipped with guardrails

___	___	___	Warning vests, or other highly visible garments, are provided and worn by all employees exposed to public vehicular traffic.

___	___	___	Employees are required to stand away from vehicles being loaded or unloaded.

___	___	___	Employees are prohibited from working or walking under suspended loads.

___	___	___	Employees are prohibited from working on the faces of sloped or benched excavations above other employees.

___	___	___	A warning system has been established and is utilized when mobile equipment is operating near the edge of an excavation.

## Utilities

Yes	No	N/A

___	___	___	Utility companies have been contacted and/or utilities located.

___	___	___	The exact location of the utilities is marked when approaching the utilities.

___	___	___	Underground installations are protected, supported, or removed when the excavation is open.

## Means of Access and Egress

Yes	No	N/A

___	___	___	Lateral travel distance to a means of egress does not exceed 25', for excavations 4' or more in depth.

___	___	___	Ladders, when used, extend 3' above the edge of the trench and are secured.

___	___	___	A Competent Person has designed structural ramps used by employees.

___	___	___	Structural ramps used for equipment have been designed by a Registered Professional Engineer (RPE).

___	___	___	Ramps are constructed of materials of uniform thickness, secured together on the bottom, and have a non-slip surface.

Daily Excavation Checklist, Cont.

## Wet Conditions

Yes	No	N/A

___	___	___	Precautions have been taken to protect employees from accumulation of water.

___	___	___	A competent person monitors water removal equipment.

___	___	___	Surface water is controlled or diverted.

___	___	___	Inspections are made after each rainstorm.

## Hazardous Atmospheres

Yes	No	N/A

___	___	___	Atmospheres are tested when there is a reasonable possibility of oxygen deficiency, or build up of other hazardous gases, that may expose employees to a hazard.

___	___	___	Oxygen content is between 19.5% and 21%.

___	___	___	Ventilation is provided to prevent flammable gas from building up to 20% of the lower explosive limit of the gas.

___	___	___	Testing is conducted to ensure that atmospheres remain safe.

___	___	___	Emergency Response Equipment is readily available where a hazardous atmosphere could or does exist.

___	___	___	Employees are trained on the use of Personal Protective and Emergency Response Equipment.

___	___	___	Safety harnesses and lifelines are individually attended when an employee is entering a deep confined excavation or bell-bottom pier.

## Protective Support Systems

Yes	No	N/A

___	___	___	Materials and/or equipment is selected based on soil analysis, expected loads, and trench parameters.

___	___	___	Materials and equipment are inspected and in good condition.

___	___	___	Materials and equipment not in good condition are removed from service and not returned until repaired, inspected, and approved by a

Registered Professional engineer.

___	___	___	Protective systems are installed without exposing employees to hazards of cave-ins, collapses, or from being struck by materials of equipment.  They are installed from the top, down, and from the bottom up.

___	___	___	Members of the protective support system are securely fastened.

___	___	___	Adjacent structures are securely supported.

___	___	___	Excavations below the footing of base are approved by a Registered Professional Engineer.

___	___	___	The backfill process progresses with the removal of the support system.

___	___	___	Material is excavated to a level no greater than 2' from the bottom of the Protective Support System, and only if the system is designed to support the calculated loads.

___	___	___	Shield system is placed to prevent lateral movement.

___	___	___	Employees are prohibited from remaining in a Trench Box when it is being moved vertically.

___________________________________		____________

Signature of Competent Person			Date