BMW CAR CLUB OF AMERICA

RUN GROUP

__________

BAYOU CHAPTER, INC.

## CAR #_____

### PRE-EVENT SAFETY INSPECTION INSTRUCTIONS

The Driver’s School Participant is solely responsible for the safe condition of his/her vehicle to be driven to, at, and from this even. Participant must have the vehicle inspected no earlier than two (2) weeks prior to this event.  A qualified individual who is familiar with the make and model for this vehicle (i.e. dealerships, specialty service shops) must perform the safety inspection.  At the time of the safety inspection, the Participant must sign the completed safety inspection report (2nd sheet) that must be presented to the authorized Bayou Chapter Tech Personnel at track registration.  The participant WILL NOT be allowed to drive the vehicle on the track without presenting the following pre-event inspection indicating that all items have been approved and signed by a qualified shop and passing the final tech inspection on the day of the event.  Seat belts MUST be of the same type for both driver and instructor.  Snell SA or MA 2000 or later helmets are required.  Loaner helmets will not be available.

Please provide the following information:

Entrant Name _______________________________

Make of Car_____________________	Model_________________	Year___________

VIN____________________________________

License Plate #___________________________				State___________

Location of Inspection______________________

Name of Inspector_________________________

#### PRE-TECH INSPECTION “DONE PRIOR TO EVENT”

Vehicle Interior

APP REJ

| Interior Mirror |  |
| --- | --- |

Seat Belts (same for driver & passenger)

No loose interior parts

Clutch pedal operation

Brake pedal firm

Emergency brake operation

Steering wheel play

Windows open fully (both sides)

##### Vehicle Exterior

| Brake lights |  |
| --- | --- |

No fluid leakage

No cracked glass(in drivers line of sight)

Exterior mirrors(must have on both sides)

Windshield wipers

Exhaust system

Gas cap secure

Tire wear condition(2/32 tread dept min)

Rims (good condition, no missing lugs)

No loose body parts

Hood and deck latches secure

Suspension & Brakes				                                         APP REJ

| Steering linkage, tie rod |  |
| --- | --- |

Suspension & shocks

Wheel bearings

Brake lines

Brakes bled within 6 mos.

Sufficient brake pads (50% min)

Brake rotors/drums

Proper alignment

##### Engine, Fluids & Electrical

| Battery secure |  |
| --- | --- |

Electrical harness/relays

Engine belts & hoses

Throttle linkage

Motor/tranny mounts

Driveshaft/guibo

Coolant system

Fuel system

Oil system

Brake fluid (must be clear)

All fluids correct level

Inspector’s Signature__________________________	Date inspected_____________________

#### FINAL TECH INSPECTION “DONE AT TRACK PRIOR TO EVENT”

APP  REJ

| Tires in good condition (2/32 min) |  |
| --- | --- |

Wheel covers removed

Lug nuts torqued properly

No loose objects anywhere in car

Brake lights operable

Car#’s placed on both sides

Proper/equal seat restraints

Car mechanically sound

No leaking fluids

Car suitable for track use

APP  REJ

Helmet

| (Snell MA or SA 2000 or later) |  |
| --- | --- |

Tech Inspector’s Signature_____________________________	Date Inspected___________________

###### DRIVER/PARTICIPANT CAR TECHNICAL INSPECTION CERTIFICATION AND INDEMNITY

DATE

EVENT    	BMW CCA Bayou Chapter Driver’s Education

Circuit Grand Bayou at No Problem Raceway Park

6740 Hwy 996, Belle Rose, LA 70341 (near Donaldsonville, LA)

The undersigned hereby certifies that:

- The OFFICIAL TECHNICAL INSPECTION FORM (the “Form”) submitted to BMW Car Club of America, Bayou Chapter Incorporated, in connection with the above Driver’s Education Event was performed on the date shown by a qualified BMW or other mechanic;

- The Form correctly and accurately shows the condition of the Car identified above (the “Car”) on the date of the inspection; and

- There has been no change(s) in the condition of the Car since the date of the inspection that would affect the trackworthiness of the Car.  The undersigned hereby agrees to indemnify and hold harmless Bayou Chapter Inc. as to any and all occurrences arising from any incorrect statements contained in the Form.

I understand the technical inspection performed on my automobile is solely for the purpose of meeting minimum standards of car preparation for the BMW Bayou Chapter Driving Event.  No warranties are implied or expressed in the passing or failure the inspection performed.  I release the inspectors from any and all liability arising from their inspection of my car.  I acknowledge that the safe condition and operation of this vehicle is entirely my responsibility as well as any vehicle problems, malfunctions or damage that may occur in connection with the operation of this vehicle prior, during or subsequent to the event.

ENTRANT’S SIGNATURE_________________________________  DATE_________________

Note:  (Car Entrant, not Driver, if different from Driver

Approved and recommended local New Orleans/Metairie area inspection stations:  These shops are familiar with high performance driving will do the inspection for a reasonable charge.  You must schedule an appointment with these shops.  You may take your car to other reputable shops but be sure to express the significance of this inspection.

Peake BMW, 2630 Veterans Blvd., Kenner, LA 70062	504-469-6165

Greff Motors, 4809 Airline Drive, Metairie, LA 70001	504-888-4809

NOLA Sport, 9700 Palm Street. N.O. LA 70118	504-301-2797

Wedge Kyes Import Specialties, 10485 Airline Hwy, B.R. LA 70816	225-295-0744