Activity Instruction Sheet

Pioneering Activity

Supervising Adults/Explorers should have read the risk assessment for this activity before running.

Equipment required:  4 x Pioneering Spars, 20x lashing ropes, Instruction sheets, marking sheet

Description of Activity

The Pioneering activity is a 20 minute instructional with patrols given points on how well the whole patrol are able to demonstrate square lashings of two spars in the time given.

Activity Set Up Instructions

Select a good flat grass area that is clear from overhead and other obstructions.

Instructions for Participants

Lay out the two spars in a V-shape.

Start with a clove hitch on one spar

Wrap the spare loose end around the working rope before starting

Follow under-over-under-pattern for 5 turns keeping the rope tight-ish

Change direction and do 5 frapping turns, tightening every half-turn

Finish with a clove hitch

Test the lashing, untie for the next person to demonstrate

If Patrol already know how to lash get them to build an A frame and walk it (without Scouts) up the field a short way – if still not satisfied teach them some new knots

| Risk Assessment |  |  |  |
| --- | --- | --- | --- |
| What could go wrong? | What is Risk from it? | Who is at risk? | What are you going to do about it? |
| Slips, trips and falls | Slips, trips and fall related injuries | Young people, adult volunteers and visitors | Keep activity area tidy, be careful about loose ropes and spars. |
| Heavy equipment | Possible injuries from spars dropped on feet or by accidental impact when carrying. | Young people, adult volunteers and visitors | Encourage safe manual handling of spars, minimise the movements required with spars by keeping spar pairs in two locations. |
| Rope injuries | Abrasions from rope burn or cuts from pulling it through hands. Misuse of ropes. | Young people, adult volunteers and visitors | Brief Scouts on not misusing ropes. If rope being used in the wrong way the activity must be stopped. |