| ITEMS | CHECK | OWNER FOLLOW UP |
| --- | --- | --- |
| Water | - Under kitchen sink - signs of water leaks |  |
|  | - Dishwasher - shut off valve, signs of water leaks, mold |  |
|  | - Bathroom sinks - water taps, signs of leaks, mold |  |
|  | - Moisture/water around toilets, sweating tank |  |
|  | - Toilet tank connector hose – plastic or braided (plastic connector hoses are unsafe and should be replaced) - Functioning shut off valve for toilets; |  |
|  | - Hot water tank - water alarm in pan; signs of water - Tag with replacement date and location of unit shut off valve |  |
|  | - Ceiling/wall stains, window leaks – (dry/moist) |  |
|  | - Sink and shower drains |  |
| Electricity | - Signs of overheating around outlets, thermostats; overloaded extension cords; |  |
|  | - Thermostats compatible with room heater wattage – signs of overheating around thermostat |  |
|  | - Noted electrical hazards (panel overload, wiring issue) |  |
|  | - Ground fault interceptor (GFI) outlet for bathrooms and kitchen (outlet with reset button – indicate even if not mandatory) |  |
| Dryer | - Condition of lint trap above dryer - accessible for regular cleaning; |  |
|  | - Dryer vent duct connections – from dryer and lint trap |  |
| Clothes washer | - Water connection; dry hoses; dry drain hose; dry floor - Braided hoses should be used – not is not |  |
|  | - Water taps accessible to easily turn off/on |  |
| ITEMS | CHECK | OWNER FOLLOW UP |
| Doors | - Patio doors: slider; closure; locking; dirt in tracks - Condition of screen slider door |  |
|  | - Exterior of unit entry door – dents, paint chipped, permanent modifications (locks, handle etc) |  |
| Windows | - Leaks (signs of water penetration) describe |  |
|  | - Glass seals, slides ok; dirt in tracks |  |
|  | - Safety limiters (in place) – code issue |  |
| Exhaust Fans | - Bathrooms - works (noise; dirt) |  |
|  | - Kitchen - works; (noise, dirt) |  |
| Hazards | - Battery operated smoke alarm in 2 story units – installed, functions, expiry date |  |
|  | - CO alarm (where required); functions, expiry date |  |
|  | - Unauthorized fuel operated applicances in unit, on balcony |  |
|  | - Unsafe wiring and installations |  |
| Balcony | - Unauthorized materials including fuel, animal waste, used as storage - Visible rebar exposure or concrete disintegration |  |
| Clean Out | - Accessible as per plan – indicate location within unit |  |
| Other | - Smoke and other strong smells affecting common elements - Canabis grow op – when more than legally acceptable |  |