CONFINED SPACE ENTRY FORM

This form must be totally completed prior to any non-emergency entry.

All hazardous materials involved in this entry permit must have SDS attached to this form.

| Roles and Responsibilities |  | Name(s) |  |
| --- | --- | --- | --- |
| Incident Commander (Required) |  |  |  |
| Safety Officer (May be IC) (Required) |  |  |  |
| Site Supervisor (May be IC or Ops) (Required) |  |  |  |
| Site Attendant (On Comm Headset) (Required) |  |  |  |
| Metering (May be Attendant) (Required) |  |  |  |
| (The following are as needed) |  |  |  |
| Air Supply & Comms Set-up |  |  |  |
| Ventilation |  | Lock-Out/Tag-Out |  |
| Rigging |  | Rehab |  |

| Attendant Meter |  |  |  |  |  |  | Entrant Meter |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meter ID# ______ Meter Model: __________ |  |  |  |  |  |  | Meter ID# ______ Meter Model: __________ |  |  |  |  |  |  |  |  |
| Meter Serial # ___________________________ |  |  |  |  |  |  | Meter Serial # ___________________________ |  |  |  |  |  |  |  |  |
| Meter Last Calibration Date: _______________ |  |  |  |  |  |  | Meter Last Calibration Date: _______________ |  |  |  |  |  |  |  |  |
| Test Readings |  |  | Entry Level | 4' mark | 8' mark |  | 12' mark | 16' mark | 20' mark |  | 24' mark | 28' mark |  | At Victim |  |
| OXYGEN (19.5 - 23.5%) |  | Reading |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Time |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMBUSTIBLE (<10% LEL) |  | Reading |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Time |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CARBON MONOXIDE (<35 PPM) |  | Reading |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Time |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HYDROGEN SULFIDE (<10 PPM) |  | Reading |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Time |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional Entries on Last Page |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Must perform continuous monitoring at the various levels throughout the space. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Must download the meter files and attach to the NFIRS report following the incident or event. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Authorized Entrants | Authorized Entrant Signature |  |  |  |  | (Entry or Back-up) |  |  |  | Time Entered |  |  | Time Exited |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Hazards of Confined Space | Yes | No | Special Requirements | Yes | No |
| --- | --- | --- | --- | --- | --- |
| Oxygen Deficiency |  |  | Hot Work Permit Required |  |  |
| Combustible Gas/Vapor |  |  | Lockout/Tagout |  |  |
| Combustible Dust |  |  | Lines Broken, Capped, or Blanked |  |  |
| Carbon Monoxide |  |  | Purge-Flush and Vent |  |  |
| Hydrogen Sulfide |  |  | Secure Area (Post and Flag to prevent entry) |  |  |
| Toxic Gas/Vapor/Fumes |  |  | Other-List: __________________________ |  |  |
| Skin - Chemical Hazards |  |  | Special Equipment Needed |  |  |
| Electrical Hazard |  |  | SCBA or SAR? |  |  |
| Mechanical Hazard |  |  | Harness - Required |  |  |
| Engulfment Hazard |  |  | Tripod and/or haul system |  |  |
| Entrapment Hazard |  |  | Retrieval Lines - Required dorsal unless hazardous |  |  |
| Thermal Hazard |  |  | Lighting (explosive proof/low voltage) |  |  |
| Slip or Fall Hazard |  |  | PPE- Goggles, Gloves, Clothing, Etc. |  |  |
|  |  |  | Fire Extinguisher |  |  |
| Ventilation |  |  | Communication Method ________________________________ |  |  |
| Space Size in Cubic Feet ________________ |  |  | Notes: |  |  |
| Fan CFM Output ________________ |  |  |  |  |  |
| Ventilation Started Time ________________ |  |  |  |  |  |

| Prior to Entry | Yes | No | N/A |
| --- | --- | --- | --- |
| Has the Atmosphere been tested and found safe for entry? (Must be below LEL or PPE for) |  |  |  |
| Has the space been adequately vented? |  |  |  |
| Have all Electrical & Mechanical lines been shut off and locked out? |  |  |  |
| Are entrants wearing proper PPE and have the proper tools? (Hand tools, I.S. Lighting?) |  |  |  |
| Is there a trained attendant outside of the confined space to monitor entrants? |  |  |  |
| Have entrants been briefed on hazards, communication plan, exit procedures, and tasks? |  |  |  |
| Have all personal electronics been removed from entrants (phone, watch, etc.)? (MUST BE) |  |  |  |

I have inspected and verified each requirement of this permit. I certify that the work defined can be performed in compliance with the standards set forth by the state of Wisconsin.

| OPENED | ENTRY SUPERVISOR: (print) | SIGNATURE: | DATE AND TIME: |
| --- | --- | --- | --- |
| CLOSED | ENTRY SUPERVISOR: (print) | SIGNATURE: | DATE AND TIME: |

Entry Procedure Checklist: Complete the following steps before, during, and after a confined space entry:

Step 1

If possible, locate a site/facility responsible party and gather information.

Step 2

Complete a Confined Space Entry Permit.

Step 3

Verify Confined Space Meters has been calibrated and are in working order

Step 4

Monitor/Meter the confined space prior to entry.  The entrant and attendant should sign the permit authorization section on the bottom of the permit to ensure all actions and conditions necessary for safe entry have been performed.

Step 5

Ensure all rescue equipment (e.g. tripod, retrieval line(s)) are in place prior to entry

Step 6

Employee entering the confined space should wear the 4-Gas Detector after the pre-atmosphere test.  The employee should also have a full body harness, dorsal connected retrieval line, and main-line if using a tripod.  Employee shall have a radio (or other means of communications) and any other necessary personal protective equipment.

Step 7

Perform a pre-entry briefing with all personnel. It is the Supervisor and Attendant’s responsibility to ensure the entrants know potential and likely hazards, tasks, communications plan, evacuation signals, and other pertinent information.

Step 8

Employee can enter the confined space once Step 7 is completed.  The entrant and attendant should continue to complete the Hazards of Confined Space and Special Requirements Sections of the Permit-Confined Space Entry Form once the employee is within the confined space to ensure accuracy. The entrant should also periodically report the readings on their meter for % Oxygen, % Explosive Gases, Carbon Monoxide, and Hydrogen Sulfide to the attendant to place on the Permit Form.

Step 9

The attendant should maintain constant communication with the entrant(s) until the entrant has exited the confined space.

Step 10

The attendant should contact the Supervisor once the entrant has exited the confined space.

Step 11

Once the event is complete, the Supervisor must Close the permit and sign, date, and record the time accordingly.

Additional Entries

Notes:

| Authorized Entrants | Authorized Entrant Signature | (Entry or Back-up) | Time Entered | Time Exited |
| --- | --- | --- | --- | --- |
| 7. |  |  |  |  |
| 8. |  |  |  |  |
| 9. |  |  |  |  |
| 10. |  |  |  |  |
| 11. |  |  |  |  |
| 12. |  |  |  |  |

Additional Meter Readings

| Additional Test Readings |  | Entry Level | 4' mark | 8' mark | 12' mark | 16' mark | 20' mark | 24' mark | 28' mark | At Victim |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OXYGEN (19.5 - 23.5%) | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
| COMBUSTIBLE (<10% LEL) | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
| CARBON MONOXIDE (<35 PPM) | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
| HYDROGEN SULFIDE (<10 PPM) | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |
|  | Reading |  |  |  |  |  |  |  |  |  |
|  | Time |  |  |  |  |  |  |  |  |  |