Portland Golf Club

Water Rights

The Portland Golf Club has water rights to Fanno Creek dating back to 1923.  There are multiple certificates (68630, 68631, 68632) granting them various rights/use of the water.

They are allowed to fill and store water in their reservoir/pond.

They can only store a maximum of 7.2 acre-feet of water in their pond. They are also​ allowed to divert water to "maintain" their maximum storage.

- From October 1-->April 30, they can divert water from Fanno Creek for the purpose of the pond​ at a rate of 1.0 cubic feet per second (CFS)

- From May 1-->September 30, they can divert water from Fanno Creek for the purpose of the pond​ at a rate of 0.1 CFS

So, for the purpose of filling/storing water in their pond and​ keeping it full, they can do this year-round, the amount they are allowed to take changes depending on the time of the year.

They are allowed to divert water from Fanno Creek to irrigate their grounds.

- From October 1-->April 30, they cannot​ divert water from Fanno Creek for the purpose of irrigation​

- From May 1-->September 30, they can divert water from Fanno Creek for the purpose of irrigation​ at 1.21 CFS.

If need be, the state has the right to monitor/meter the water usage and/or compliance with the above-listed certificates. This could happen if someone downstream​ with more senior water rights​ complained.  Since the PGC has very senior water rights (1923) this has never happened. This could also happen if the state has reasonable suspicion to believe the PGC is enlarging their water right in some way.  This has also never happened. Thus, there is no monitoring, metering, oversight or measuring taking place to ensure that the PGC is, in fact, in compliance with the above-named Certificates.

​**Additionally, PGC receives water from the aquifer, they have 3 wells on site

​**They also receive 100% of the water from Woods Creek to the east, Woods Creek ends​ in their pond