Speeds and Feeds

## Variables / Abbreviations

- Speed – Surface Feet per Minute (SFM)

- Feed – Inches per Minute (IPM)

- Feed per Tooth (FPT)

- Adjusted Feed per Tooth – Chip Thinning (AFPT)

- Feed per Revolution (FPR)

- Depth of Cut (DOC)

- Width of Cut (WOC)

- Tool Diameter (D)

- # of Teeth in Cutter (Z)

- Metal Removal Rate – Cubic Inches per Minute (MRR)

## Milling Formulas

- Speed (RPM) = (SFM x 3.82) / D

- Feed (IPM) = RPM x IPT x Z

- SFM (Surface Feet per Minute) = (RPM x D) / 3.82

- IPT (Inches per Tooth) = (IPM / RPM) / Z

- MRR (Cubic Inches per Minute) = IPM * WOC * DOC

- AFPT (@ less than 1/2 dia. WOC) = IPM x sqroot of (D / WOC)

- HP (Horsepower Consumption) = MRR x mf

  - mf - steel = 1

  - mf - gray iron = .65

  - mf - aluminum = .3

  - * - 1.5% from total for every degree positive

High Speed Steel Endmill Speed and Feed’s

Carbide Endmill Speed and Feed’s

Ballmill Speed and Feed’s