Required Minimum Safety from USCG (Check Local and State Laws)

| Items | 16-26 | 26-40 | 40- 65 |
| --- | --- | --- | --- |
| Life Vest (Type I, II, III, or V) |  | X |  |
| Throwable Device (Type IV Immediately Available) |  | X |  |
| Fire Extinguisher (One B-1 or One B-2)* ** | One | Two* | Three** |
| Visual Distress Signals (Three Day/Night are Suffice) |  | X |  |
| Sound Producing Device (4-second blast audible for ½ mile) |  | X |  |
| Placards (must display oil and garbage placards) |  | X |  |
| Note USCG Does not require Smoke Alarms (Check State Laws) |  |  |  |

Recommendation From World Sailing

| Items |  |
| --- | --- |
| Life Lines (Uncovered) |  |
| Bilge pump handles secured against loss overboard |  |
| One pump be operated with cockpit lockers, hatches and companionways shut |  |
| At least two manual bilge pumps? |  |
| A second compass which may be hand-held and/or electronic |  |
| A softwood plug secured at or near every through-hull opening |  |
| Jackstays Tethers and PFDs |  |
| High intensity heavy duty search light immediately available |  |
| Each crew member carry their own waterproof torch/light at night |  |
| First Aid Manual that covers marine medical emergencies |  |
| Foghorn and Radar Reflector |  |
| Display the Safety Equipment Location Chart |  |
| Display the Through hole Location on a Chart |  |
| Emergency Steering |  |
| Boat's name on all miscellaneous buoyant equipment |  |
| Retro-reflective material on lifebuoys, recovery slings, liferafts and life jackets |  |
| 406 MHz EPIRB (Offshore) |  |
| Lifebuoy with a self-igniting light and drogue |  |
| Practice a Man Overboard Drill |  |
| Means of recording and monitoring a Man Overboard incident within 10 seconds |  |
| Recovery sling |  |
| Cockpit Knife |  |
| Reefing ability for 40 and 50% for Main and Jib |  |
| Ditch Bag - |  |
| The crew have undertaken safety training |  |