Safety Officer Card

Preparation

Safety Briefing for Crew - Each crew will be familiar with:

Sea Cocks – Insect for operational fitness and locations (Recommended Map)

First Aid – Location (Who is trained)

Life Jackets – Location (When are they used; rules)

Throwable Device/Recovery Line/MOM – Crew will understand the use of devices to aid in recovery operations.

Reefing – Crew will understand how to reef.

Crew List – Captains shall provide a daily crew list and emergency contact details.

Emergency Communications – Crew will understand the equipment and how to communicate in an emergency. Ensure local rescue and emergency call numbers are clearly posted in the vessel. If DSC is available, crew shall understand its use.

Man Over Board Button – The Crew will understand the use of the MOB Button.

De-watering Plan – Crew will understand procedures and equipment to de-water the vessel.

Fire – Crew will understand the location and use of firefighting equipment.

Visual & Audible Distress Signals – Crew will understand the location and use of VDS & ADS.

Crew Health – Understand medical conditions, medicines and symptoms.

Crew Overboard Drill – Crew will understand and practiced a Crew Overboard Drill

Weather

The captain will ensure a weather briefing informs the crew of expected conditions including the worst-case scenario based on relevant data.

Crew will understand exposure protection from hypothermia and sun exposure. Yes, 80-degree water temperature can cause hypothermia.

Routing

Identification of safe harbors.

Identification of rescue services locations and landlines to call for assistance

Proximity or Probability of Emergency Assistance shall be understood.

Make sure this is filled out with local knowledge

Material Condition

The material condition of the vessel - it is urged to always keep all bilges clean and dry. The attachment of the fittings to the fiberglass hull should be examined for signs of corrosion. All apparatus and through hull fittings must be visually examined for signs of wear, corrosion, strain, cracking, or potential failure.

The crew will be able to identify identification of pinch points and areas that contain high loads.

Vessels equipped with electronic safety systems and DSC will provide DSC and AIS information prior to competing.

Required Safety Equipment (Refer to World Sailing – Offshore Special Regulations)

Minimum USCG Requirements

3 SOLAS approved orange smoke distress signals
3 Flares
1 First Aid kit 
1 GMDSS approved handheld VHF radio
1 Throwable device
1 Rescue heaving line
1 Anchor least and appropriate chain/line for the area Raced
swivel between anchor and line.
1Baseball bat (required for crew morale)

Required Navigation Equipment

Magnetic Compass with current deviation card
Nautical charts maintained to current Notice to Mariners
Binoculars

PRIOR TO DEPARTURE

_____ Crew list given to shore side contact.

_____ Captain certifies a COB practice has occurred for all crew.

_____ Water and Food stocked

_____ Adequate number of PFDs, Mustang suits, or float coats aboard.

_____ Inspect/test EPIRB for proper operation

_____ Check fuel levels, record amount in log. Understand burn rate.

_____ Open all sea chests sea cocks and necessary sea valves

_____ Check oil, coolant, belts, and hoses for:

_____ Marine gear(s)/transmission(s)

_____ Start main engines and generator

_____ Check for cooling water discharge

_____ Check oil pressures, Oil Color

_____ Check generator is warmed, shift to generator power

_____ Take in or unplug land line and shore tie

While Sailing

In the event of an emergency, at the first opportunity, declare a PAN PAN or Mayday.  Note a ‘Crew Overboard is a Mayday’ (a life is in jeopardy).

First, try to control the situation. Second, ask for assistance.  Events multiply quickly and it is easier to turn off assistance on the way, rather than having a delayed call for help.

It is the captain’s responsibility to always maintain radio contact with other racers and the Race Committee.

Any situation – that may develop into an emergency – It is encourages to utilize the PAN PAN function to expedite awareness for responders if needed.

Final Considerations

Risk Assessment

The Risk Assessment may be considered the most important tool on the vessel.  See Appendix A for a recommended Go-No Go assessment tool.  This tool is intended to be utilized by each sailor as a self-assessment and the scores are to be averaged to assist in decision making.

Sailing Risk Assessment (Go – No Go)

| Risk Assessment (Vessel) |  |  |
| --- | --- | --- |
| Identify who is the Safety Officer |  |  |
| Identify the Boat Captain |  |  |
| Man-Overboard Drill |  | G/NG |
| Within 30 days – the entire crew has participated in a MOB drill for the vessel. A section of the log will be dedicated to all team members who have completed the MOB drill which includes: 1) vessel maneuverability, 2) attaching the person in the water to the vessel and, 3) simulated (weighted) recovery of a person onto the vessel. (all three must be included for Go) |  |  |
| Equipment |  |  |
| Day and/or Night Safety Equipment accounted for and noted in the log. |  |  |
| Weather |  |  |
| Forecasted Meteorological Conditions. Weather is: Low Risk (Score 1) – below the vessel capabilities with maximum sail plan. Medium Risk (Score 2) – meets the vessel capabilities with maximum sail plan, possible reef (medium risk of round up). (Note: Water temperature below 72f degrees PFDs Mandatory). High Risk (Score 3) – Exceeds vessel capabilities without reefing (high risk of round up). (Note: Night racing is High Risk - PFDs Mandatory) Low = Go, Medium = Go, High – Go (with DMA and/or individual risk average of 14 or greater. |  |  |
| Load and Pinch Point Awareness |  |  |
| Complete understanding objects with high loads and pinch points, briefed and/or identified with markings. (Safety Brief shall include Loads and Pinch Points). Controls shall be identified. |  |  |
| Head-Trauma Locations |  |  |
| Awareness of locations with the potential of head trauma are identified (bow, cockpit, other). Controls have been identified. Boom, Clews, Sheets, Blocks and Elbows. |  |  |
| Individual Risk Assessment |  |  |
| Days on Particular Vessel and Type: (per individual * Note Below “Individual Score”) 1: 0-14 days 2: 15-60 days 3: 61+ days |  |  |
| Personal Health and Athleticism: Physical condition and ability to perform under stress. 1: Limited physical ability, mobility and/or health concerns (must wear PFD & clipped-in) 2: Average physical ability, some limitations 3: Excellent physical ability and endurance |  |  |
| Experience Sailing in Weather Conditions: Ability to handle various weather conditions. 1: Limited experience (only low conditions) 2: Moderate experience (low to medium conditions) 3: Extensive experience (medium to high conditions and storms) |  |  |
| Days offshore: Total experience offshore (13 miles from land *for offshore only): 1: 0-50 days 2: 51-100 days 3: 101+ days |  |  |
| Positions While Sailing: Experience in different roles on the boat. 1: Unfamiliar with tasks and needs instruction 2: Familiar with tasks, but sometimes needs to think about technique 3: Mastered tasks, doesn't need to think about procedures |  |  |
| Spinnaker Experience: Experience with symmetrical and asymmetrical spinnakers. 1: Limited or no experience 2: Experience with one type (symmetrical or asymmetrical) 3: Experience with both types (Score of three without spinakers) |  |  |
| Individual Score * (Score from above) 18 – 14 Average in High Risk Go (PFDs, Tethers Mandatory) 13 – 9 Average in High Risk - DMA for Go (PFDs, Tethers Mandatory) 8 – 6 Any Sailor below 9, in any conditions (DMA PFDs, Tethers Mandatory) Mobility issues, PFD & Tethers Mandatory |  |  |
| Definitions DMA = Decision Making Authority, Crew needs Leadership Approval PFDs = Personal Flotation Devices, When reefed and at night PFDs and tethers shall be donned. Scoring – Each Sailors ‘Individual Score’ shall be averaged for a total voyage Go – No Go decision. |  |  |