# STANDARD INSPECTION PROCESS

# Scope of document

This document outlines the basic principles for determining the condition of Finished Vehicles, during the transportation chain.

The inspections are utilized to determine damage liability and provide information for damage elimination efforts.

# Inspection  Criteria’s and definition:

The basic inspection carried out will be a Standard Transport Inspection, which is an external inspection of the vehicle condition, and also to include a cursory interior inspection to identify occurrences of theft and or vandalism.  (This does not include a requirement to inspect trunk compartments, or the contents of sealed loose item bags or boxes)

The Inspector appointed, whether by the Contact Carrier or jointly with the OEM, has to be a qualified vehicle inspector of reputable standing. Where cost estimates of exceptions are required, the inspector must have knowledge of current repair procedures and cost levels for the account in question.

# VEHICLE  INSPECTION PROCEDURES

## General

The actual vehicle inspection is to be conducted so as to record all transport related or other types of exceptions to the vehicles, and in such way to minimize any physical contact with the vehicle in order to avoid damage.

It is important to maintain a consistent and high quality inspection. The inspection must be performed in a systematic and thorough manner and never vary from that routine.

- It is worthwhile noting that a single inspector requires between 1 minute and 1.5 minutes in order to complete a proper inspection.

- Remember/ensure that the inspector needs to adhere to the following:

### Record identified exceptions by listing and coding the specific part damaged.

### Name the type of exceptions according to the standard damage code list and definitions.

### Vehicles are not to be moved from FPR (first point of rest) position until units have been inspected.

## Preliminary Steps

### The inspection is to be performed during daylight hours where possible.

### Observe and make note of the condition of the vehicle staging area, i.e., spacing of vehicles, presence of debris, pavement condition.

### Observe and make note of the general cleanliness of the vehicles, and/or the presence of protective devices/packaging/coatings.

### Assessments shall always be entirely impartial.

### Use clothing suitable for the purpose.  Buttons, zippers and belts shall be properly covered. Tools, mobile phones etc., are not to be carried in pockets or exposed. All exposed jewellery, such as rings, bracelets and watches are to be taken off or shall be properly covered.

### Do not walk between vehicles if there is inadequate space to ensure that there is no contact with the adjacent vehicle(s)

### It is absolutely forbidden to sit on or lean against the vehicle.

### Do not smoke, eat or drink around the vehicles

### Do not place anything on top of a vehicle.

### Make sure that doors and windows are kept closed

### Do not use clipboards with metal clips and/or any sharp objects

### Do not apply markings of any kind to the vehicles (i.e. grease pencil)

## Automobile Inspections

The inspection of the vehicle typically starts at the Left Side or Drivers side of the vehicle.

Start your inspection at the drivers’ door.

### Inspect glass on the outside mirror of the door for scratches and chips.

### Check inside of vehicle including dash, making sure the audio system and GPS equipment is present. (Note any missing or vandalized items)

### Seats, carpet area, and door trim panels for soiling or other exceptions.

### Next inspect the outside of the driver’s door. Keep in mind that this is a high damage area due to the number of entries into the vehicle during the transportation process.

### While moving toward the rear of the vehicle, you should view all panels, roof doors, rocker panels etc. from at least two angles. During the inspection of suspected panel damages, occasional light touching with one’s finger on the vehicle is permitted to verify paint scratches and chips.

### Inspect the wheels, wheel covers, and tires.

### Step back from the vehicle at about a 30 degree angle to view panels from a different sight line.

Move from the left rear of the vehicle toward the trunk area.

Inspect:

### Rear glass, deck lid area, rear lamps, rear end panel, lower filler panels, bumper, bumper strips and bumper guards.

If the vehicle is a large utility vehicle, it is best to perform a second roof inspection from the rear.

### Also check the end of the exhaust system and under carriage for any damages.

Move from the rear or trunk area to the passenger side of the vehicle.

### Inspect the quarter panel, roof, doors, door glass, wheels, wheel covers, tires, rocker panel, front fender, all mouldings.

Move from the passenger side of the vehicle to the front of the vehicle.

### Inspect the windshield, hood, headlamps, fog lamps, all mouldings, emblems, bumper, bumper guards, bumper strips and splash panels.

While inspecting the vehicle it is good to step back and view the vehicle from different angles/sight lines.  A 30 degree angle, to identify certain damages that are not as apparent when viewing from right angles.

# ASSESSMENT OF TRANSIT DAMAGE

Initial Inspections should be viewed at > 1.0 meters (>3.2 feet) distance from eye to exception in question.

Wrap guard may be partly removed; only if the inspector suspects hidden damage underneath already damaged wrap guard/Plastic protection.  This is to be done with extreme care!

- * Gun-sighting	Viewing down the side of a vehicle at angles less than 30 degrees, usually up close and parallel to the vehicle. The inspector is moving his/her head up and down and from side to side to detect any reflection changes due to very small dents in the panels.  As a general rule, gun sighting should be a verification tool and not a detection tool.

- If inspection personnel discover repeated cases of the same types of damage or defect(s) this must be reported immediately to the OEM.

During the transportation chain when automobiles change possession the vehicles should be inspected immediately (as practical) with any damage or missing parts/options noted. This manual provides a standard up to date set of codes for use by all personnel responsible for handling vehicles during the vehicle shipping process. It is important that the damage codes be accurately detailed. The codes are recorded manually on delivery receipts or in portable data terminals for transfer into manufacturer’s claims systems. I The damage codes are composed of five digits as follows:

Damage Area Code – First and Second Digits

Damage Type Code – Third and Fourth Digits

Damage Severity Code – Fifth Digit

Damage Code Example: Left front door is scratched 4 inches in length.  The damage code describing this condition is:

Damage area = 10 (Left Front Door) Damage type = 12 (Scratch)  Damage Severity 3 (Over 3” up to 6”)

Right and left are determined as if sitting in the driver’s seat.

Multiple unrelated damages with the same damage area and type noted on the same panel should be entered separately.

The following is the latest AIAG Standard Inspection Damage Codes Listing:

- THE FOLLOWING LIST IS THE UPDATED SET OF CODES WITH DESCRIPTION

  - DAMAGE AREA/TYPE CODES

CODE»DESCRIPTION (New Descriptions In Italics)

| 1 | Antenna / Antenna Base |
| --- | --- |
| 2 | Battery/Box |
| 3 | Bumper/Cover/Ext-Front |
| 4 | Bumper/Cover/Ext-Rear |
| 5 | Bumper Guard/Strip-Front |
| 6 | Bumper Guard/Strip-Rear |
| 7 | Door-Back Cargo, Right |
| 8 | Door-Back Cargo, Left |
| 9 | Door, Right Carco |
| 10 | Door-Left Front |
| 11 | Door-Left Rear |
| 12 | Door-Right Front |
| 13 | Door-Right Rear |
| 14 | Fender-Left Front |
| 15 | Quarter Panel/Pick-Up Box-Left |
| 16 | Fender-Right Front |
| 17 | Quarter Panel/Pick-Up Box -Right |
| 18 | Front Floor Mats |
| 19 | Floor Mats Rear |
| 20 | Glass Windshield |
| 21 | Glass Rear |
| 22 | Grille |
| 23 | Accessory Bag / Box |
| 24 | Headlight/Cover/Turn Signal |
| 25 | Lamps-Fog/Driving/Spot Light |
| 26 | Headliner |
| 27 | Hood |
| 28 | Keys |
| 29 | Keyless Remote |
| 30 | Mirror-Outside Left |
| 31 | Mirror-Outside Right |
| 32 | Open |
| 33 | Audio/Video Player |
| 34 | TV/DVD Screen |
| 35 | Rocker Panel / Outer Sill - Left |
| 36 | Rocker Panel / Outer Sill - Right |
| 37 | Roof |
| 38 | Running Board / Step - Left |
| 39 | Running Board / Step - Right |
| 40 | Spare Tire/Wheel |
| 41 | Open |
| 42 | Splash Panel / Spoiler - Front |
| 43 | open |
| 44 | Gas Tank |
| 45 | Tail Light / Hardware |
| 46 | Open |
| 47 | Open |
| 48 | Trim Panel-Front Left |
| 49 | CD Changer Separate Unit |
| 50 | Trim Panel-Front Right |
| 51 | open |
| 52 | Deck Lid/Tailgate/Hatchback |
| 53 | Sunroof/T-Top |
| 54 | Undercarriage-Other |
| 55 | Cargo Area-Other |
| 56 | Vinyl/Convertible Top/Tonneau Cover |
| 57 | Wheel Covers/Caps/Rings |
| 58 | Radio Speakers |
| 59 | Wipers, all |
| 60 | special use |
| 61 | Box Interior, pickup |
| 62 | open |
| 63 | Rails, Truckbed/Lightbar |
| 64 | Spoiler/Deflector-Rear |
| 65 | Luggage Rack (Strips) / Drip Rail |
| 66 | Dash/Instrument Panel |
| 67 | Cigarette Lighter/Ashtray |
| 68 | Carpet-Front |
| 69 | center post, right |
| 70 | center post left |
| 71 | corner post |
| 72 | Left Front Tire |
| 73 | Left Front Wheel / Rim |
| 74 | Left Rear Tire |
| 75 | Left Rear Wheel / Rim |
| 76 | Right Rear Tire |
| 77 | Right Rear Wheel / Rim |
| 78 | Right Front Tire |
| 79 | Right Front Wheel / Rim |
| 80 | Cowl |
| 81 | Gas/Cap Cover |
| 82 | Fender-Rear Left |
| 83 | Fender-Rear Right |
| 84 | Tools/Jacks/Spare-Tire Mount + Lock |
| 85 | Communication/GPS Unit |
| 86 | Parking Sonar System |
| 87 | Open |
| 88 | Open |
| 89 | Trailer Hitch, Wiring Harness, Tow Hooks |
| 90 | Frame |
| 91 | Exhaust System |
| 92 | License-Bracket |
| 93 | Steering Wheel/ Airbag |
| 94 | Seat-Front Left |
| 95 | Seat-Front Right |
| 96 | Seat-Rear |
| 97 | Carpet-Rear |
| 98 | Interior Other |
| 99 | Engine Compartment-Other |

  - Damage Type Codes.

| 1 | Bent |
| --- | --- |
| 2 | Broken |
| 3 | Cut |
| 4 | Dented |
| 5 | Chipped |
| 6 | Cracked |
| 7 | Gouged |
| 8 | Missing |
| 9 | Scuffed |
| 10 | Stained or soiled |
| 11 | Punctured |
| 12 | Scratched |
| 13 | Torn |
| 14 | Dented Paint not damaged |

Damage Severity Codes

| 18 | Moulding/emblem.weatherstrip damaged |
| --- | --- |
| 19 | Moulding/emblem.weatherstrip loose |
| 20 | Glass cracked |
| 21 | Glass broken |
| 22 | Glass chipped |
| 23 | Glass scratched |
| 24 | Marker light / additional turn light damage |
| 25 | Decal/paint stripe damaged |
| 29 | Contamination, exterior |
| 30 | Fluid spillage, exterior |
| 34 | Chipped panel edge |
| 36 | Incorrect part or option not as invoiced |
| 37 | Hardware - damaged |
| 38 | Hardware - loose, missing |

| 1 | Damage up to and including 1 inch in length/diameter - less than 3 cm |
| --- | --- |
| 2 | Damage over 1 inch up to and including 3 inches in length/diameter - 3cm up to 8 cm |
| 3 | Damage over 3 inch up to and including 6 inches in length/diameter - over 8 cm up to 15 cm |
| 4 | Damage over 6 inches up to and including 12 inches in length/diameter - over 15 cm to 30 cm |
| 5 | Damage over 12 inches in length/diameter - over 30 cm |
| Severity Codes 6 | Missing |

The first document (Section 1) is the AIAG “Non-Carrier” Transportation Damage Liability Guideline, which can be used to assist the inspector in determining if an exception is transportation related or warranty / plant related.  This is an advice to the carriers that they will not be held liable for these items regardless of whether these types of damages are noted or not.

The second document (Section 2) is the AIAG Similarity Matrix, which can be used by the claims processor to determine which damage codes are interchangeable with other codes in the same category (i.e., Damage Area, Damage Types, and Severity). This document is not intended to influence the way damages are coded. It is for use by persons who are filing and adjudicating claims, not by persons who are recording damages.

- AIAG Non-Carrier Transportation Damage Liability Guideline

THE FOLLOWING CONDITIONS ARE NOT CONSIDERED TRANSPORTATION DAMAGE

- All exterior paint damage resulting from environmental fallout or fluids, unless clear evidence supports carrier responsibility.

- Sheet metal dents, restricted to severity 1, with no paint damage or evidence of physical impact, abrasion, or forced entry, except to the left front door or as identified by specific manufacturers’ policy.

- Sheet metal protrusions or misalignment of panels, moldings, decals, weather stripping, emblems, etc., indicative of plant or installation problems.

- Missing moldings, emblems, decals, etc., when there is clear evidence of no installation (i.e., holes not drilled for installation, or holes w/ no screws installed).

- Peeling, runs, sags, blisters, or foreign material in paint or chrome.

- Stress cracks in glass originating from under molding without signs of impact.

- Minor damage, as identified by the manufacturer, to painted surfaces protected by shipping film, where the shipping film shows no obvious signs of disturbance.

- Missing contents of sealed plant-provided loose-part packages.

- Incorrect parts or options claims – miss built vehicles

CONDITIONS’s NOTED BY DEALERS TO BE ASSIGNED BY CLAIMS CENTER

- Damages noted at factory gate inspection.

- Plant-authorized known quality problems or pattern damage (Vehicle Quality Group or divisional directives to charge plant).

- Vehicle interior damages other than driver area, as identified by the manufacturer, unless there is clear evidence of theft / vandalism.

- Battery charge and test / replace as a result of failure not due to carrier negligence.

GM-SPECIFIC NOTATIONS

- Plant failure to install basic protective devices to prevent damage during the normal shipping process, for example, seat or carpet protection.

- Port Claims by damage area/type/severity including 09-1, 12-1, 14-1, 14-2, 18-1, 25-1, 32-6, 37-1.

CHRYSLER SPECIFIC NOTATIONS

- Panel edge chips – other than driver’s door

Note:  This document is not intended to influence the way damages are coded. It is for use by persons who are filing and adjudicating claims, not by persons who are recording damages.

- AIAG FINISHED VEHICLE, Claim Settlement, Damage CODE SIMILARITY MATRIX

| Similar Damage Area Matrix |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table Logic Works from Left to Right |  |  |  |  |  |  |  |  |
| Description | Area | Matrix Suggestion |  |  |  |  |  |  |
| Antenna/Antenna Base | 01 |  |  |  |  |  |  |  |
| Battery/Box | 02 | 99 |  |  |  |  |  |  |
| Bumper/Cover/Ext., Front | 03 | 05 | 42 | 86 |  |  |  |  |
| Bumper/Cover/Ext., Rear | 04 | 06 | 86 |  |  |  |  |  |
| Bumper Guard/Strip, Front | 05 | 03 | 42 | 86 |  |  |  |  |
| Bumper Guard/Strip, Rear | 06 | 04 | 86 |  |  |  |  |  |
| Door Back Cargo-Right | 07 | 52 |  |  |  |  |  |  |
| Door Back Cargo- Left | 08 | 52 |  |  |  |  |  |  |
| Door, Right Cargo | 09 | 13 |  |  |  |  |  |  |
| Door, Left Front | 10 |  |  |  |  |  |  |  |
| Door, Left Rear | 11 |  |  |  |  |  |  |  |
| Door, Right Front | 12 |  |  |  |  |  |  |  |
| Door, Right Rear | 13 | 9 |  |  |  |  |  |  |
| Fender, Left Front | 14 |  |  |  |  |  |  |  |
| Qtr. Panel or PU Box, Left | 15 | 82 |  |  |  |  |  |  |
| Fender, Right Front | 16 |  |  |  |  |  |  |  |
| Qtr. Panel /Pick-Up Box Right | 17 | 83 |  |  |  |  |  |  |
| Front Floor Mats | 18 | 98 | 19 | 68 |  |  |  |  |
| Rear Floor Mats | 19 | 98 | 18 | 97 |  |  |  |  |
| Glass Windshield | 20 |  |  |  |  |  |  |  |
| Glass Rear | 21 |  |  |  |  |  |  |  |
| Grille | 22 |  |  |  |  |  |  |  |
| Accessory Bag/Box | 23 | 55 | 98 |  |  |  |  |  |
| Headlight/Cover/Turn Signal | 24 | 25 |  |  |  |  |  |  |
| Lamps, Fog/Driving/Spot Lt. | 25 | 24 |  |  |  |  |  |  |
| Headliner | 26 | 98 |  |  |  |  |  |  |
| Hood | 27 |  |  |  |  |  |  |  |
| Keys | 28 | 29 |  |  |  |  |  |  |
| Keyless Remote | 29 | 28 |  |  |  |  |  |  |
| Mirror, Outside, Left | 30 |  |  |  |  |  |  |  |
| Mirror, Outside, Right | 31 |  |  |  |  |  |  |  |
| Open | 32 |  |  |  |  |  |  |  |
| Audio/Video Player | 33 | 34 | 49 | 85 |  |  |  |  |
| TV/DVD Screen | 34 | 33 | 85 |  |  |  |  |  |
| Rocker Panel /Outer Sill Left | 35 |  |  |  |  |  |  |  |
| Rocker Panel /Outer Sill Right | 36 |  |  |  |  |  |  |  |
| Roof | 37 | 53 | 56 | 65 | 71 |  |  |  |
| Running BD/Step, Left T | 38 |  |  |  |  |  |  |  |
| Running BD/Step, Right T | 39 |  |  |  |  |  |  |  |
| Spare Tire/Wheel | 40 | 72 | 73 | 74 | 75 | 76 | 77 |  |
|  |  | 78 | 79 | 84 |  |  |  |  |
| Open | 41 |  |  |  |  |  |  |  |
| Splash Panel/Spoiler Front | 42 | 03 | 05 |  |  |  |  |  |
| Open | 43 |  |  |  |  |  |  |  |
| Gas Tank | 44 | 54 |  |  |  |  |  |  |
| Tail Light/Hardware | 45 |  |  |  |  |  |  |  |
| Open | 46 |  |  |  |  |  |  |  |
| Open | 47 |  |  |  |  |  |  |  |
| Trim Panel, Left Front | 48 | 98 |  |  |  |  |  |  |
| CD Player Separate Unit | 49 | 33 |  |  |  |  |  |  |
| Trim Panel, Right Front | 50 | 98 |  |  |  |  |  |  |
| Open | 51 |  |  |  |  |  |  |  |
| Deck Lid/Tailgate/Hatchback | 52 | 64 |  |  |  |  |  |  |
| Sun roof/T-Top | 53 | 37 | 56 |  |  |  |  |  |
| Undercarriage/Other | 54 | 90 | 91 | 44 | 84 | 89 |  |  |
| Cargo Area, Other | 55 | 23 | 98 |  |  |  |  |  |
| Vinyl/Convertible Top/Tonneau Cover | 56 | 37 | 53 |  |  |  |  |  |
| Wheel Covers/Cap/Rings | 57 | 23 | 55 | 98 |  |  |  |  |
| Radio Speakers | 58 |  |  |  |  |  |  |  |
| Wipers, All | 59 |  |  |  |  |  |  |  |
| Special Use | 60 |  |  |  |  |  |  |  |
| Box Interior, PU T | 61 | 63 |  |  |  |  |  |  |
| Open | 62 |  |  |  |  |  |  |  |
| Rails, Truckbed/Light Bar | 63 | 61 |  |  |  |  |  |  |
| Deflector/Spoiler, rear | 64 | 52 |  |  |  |  |  |  |
| Luggage Rack /Strips/Drip Rail | 65 | 37 | 71 |  |  |  |  |  |
| Dash/Instrument Panel | 66 | 33 | 34 | 85 |  |  |  |  |
| Cigarette Lighter/Ashtray | 67 |  |  |  |  |  |  |  |
| Carpet, Front | 68 | 18 | 19 | 98 |  |  |  |  |
| Center Post, Right | 69 |  |  |  |  |  |  |  |
| Center Post, Left | 70 |  |  |  |  |  |  |  |
| Corner Post | 71 |  |  |  |  |  |  |  |
| Left Front Tire | 72 | 40 |  |  |  |  |  |  |
| Left Front Wheel/Rim | 73 | 40 |  |  |  |  |  |  |
| Left Rear Tire | 74 | 40 |  |  |  |  |  |  |
| Left Rear Wheel/Rim | 75 | 40 |  |  |  |  |  |  |
| Right Rear Tire | 76 | 40 |  |  |  |  |  |  |
| Right Rear Wheel/Rim | 77 | 40 |  |  |  |  |  |  |
| Right Front Tire | 78 | 40 |  |  |  |  |  |  |
| Right Front Wheel/Rim | 79 | 40 |  |  |  |  |  |  |
| Cowl | 80 |  |  |  |  |  |  |  |
| Gas/Cap Cover | 81 |  |  |  |  |  |  |  |
| Fender, Left Rear T | 82 | 15 |  |  |  |  |  |  |
| Fender, Right Rear T | 83 | 17 |  |  |  |  |  |  |
| Tools/Jacks/Spare-Tire Mount & Lock | 84 |  |  |  |  |  |  |  |
| Communication/GPS Unit | 85 |  |  |  |  |  |  |  |
| Parking Sonar System | 86 | 54 |  |  |  |  |  |  |
| Open | 87 |  |  |  |  |  |  |  |
| Open | 88 |  |  |  |  |  |  |  |
| Trailer Hitch, Wiring Harness Tow Hooks | 89 | 54 |  |  |  |  |  |  |
| Frame | 90 | 54 |  |  |  |  |  |  |
| Exhaust System | 91 | 54 |  |  |  |  |  |  |
| License Bracket | 92 | 55 |  |  |  |  |  |  |
| Steering Wheel/Airbag | 93 | 98 |  |  |  |  |  |  |
| Seat, Left Front | 94 | 98 |  |  |  |  |  |  |
| Seat, Right Front | 95 | 98 |  |  |  |  |  |  |
| Seat, Rear | 96 | 98 |  |  |  |  |  |  |
| Carpet, Rear | 97 | 98 |  |  |  |  |  |  |
| Interior | 98 | 18 | 19 | 23 | 26 | 48 | 50 | 57 |
|  |  | 58 | 68 | 94 | 95 | 96 | 97 |  |
| Engine Compartment, Other | 99 | 02 |  |  |  |  |  |  |

| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Type | Description | Matrix Suggestion |  |  |  |  |  |  |  |  |  |  |
| 01 | Bent | Deformed surface or part due to impact. Different from broken or dented | 02 | 03 | 04 | 06 | 07 | 11 | 13 |  |  |  |  |
| 02 | Broken | Inoperable. Also means to separate into two or more parts as a result of impact | 03 | 06 | 11 | 13 |  |  |  |  |  |  |  |
| 03 | Cut | A smooth-edged serration (as if cut by a knife). Not a break, crack or tear | 02 | 06 | 11 | 13 |  |  |  |  |  |  |  |
| 04 | Dented - Paint or Chrome damaged | An inward depression of a painted of chrome surface with damage to the paint or chrome present | 01 | 02 | 03 | 06 | 07 | 11 | 13 |  |  |  |  |
| 05 | Chipped - Does not apply to glass or panel edge | An area missing paint caused by impact. Do not use to describe chips caused by poor panel alignment during assembly | 01 | 02 | 03 | 04 | 06 | 07 | 09 | 11 | 12 | 13 | 34 |
| 06 | Cracked - Does not apply to glass | A narrow opening of flaw as a result of impact; the pieces remain together. | 02 | 02 | 11 | 13 |  |  |  |  |  |  |  |
| 07 | Gouged | A groove or cavity causing damage to metal or plastic surface | 02 | 03 | 04 | 06 | 11 | 13 |  |  |  |  |  |
| 08 | Missing | Part or option is not present at time of inspection | 38 |  |  |  |  |  |  |  |  |  |  |
| 09 | Scuffed | A scrape mark that does not break the surface material | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 11 | 12 | 13 | 34 |
| 10 | Stained or Soiled | Discoloration of an interior surface by a foreign substance. |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Punctured | a hole caused by being pierced | 02 | 03 | 06 | 13 |  |  |  |  |  |  |  |
| 12 | Scratched - Does not apply to glass | A linear mark or cut in painted or chrome surfaces. | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 09 | 11 | 13 | 34 |
| 13 | Torn | Similar to cut but edges of damage area are ragged | 02 | 03 | 06 | 11 |  |  |  |  |  |  |  |
| 14 | Dented Paint not Damaged - Paint / Chrome not damaged | An inward depression of a painted of chrome surface with no damage to pain or chrome. | 01 | 02 | 03 | 04 | 06 | 07 | 11 | 13 |  |  |  |
| 18 | Molding/Emblem/Weather-strip Damaged | Damage to the molding or emblem of a specific damage area resulting from impact to that part or to a directly adjacent part. | 19 | 25 | 37 | 38 |  |  |  |  |  |  |  |
| 19 | Molding/Emblem/Weather-strip Loose | Loosening of the molding or emblem of a specific damage area resulting from impact to that part or an adjacent part. Do not use to describe molding or emblems improperly installed at the assembly plant | 18 | 25 | 37 | 38 |  |  |  |  |  |  |  |
| 20 | Glass Cracked | Cracked as a result of impact, but pieces remain together. | 02 | 21 | 22 | 23 |  |  |  |  |  |  |  |
| 21 | Glass Broken | Glass has been broken as a result of impact to the glass or surrounding panel or molding | 02 | 20 | 22 | 23 |  |  |  |  |  |  |  |
| 22 | Glass Chipped | A small fragment of glass removed as a result of impact. | 02 | 20 | 21 | 23 |  |  |  |  |  |  |  |
| 23 | Glass Scratched | A narrow linear exception. | 02 | 20 | 21 | 22 |  |  |  |  |  |  |  |
| 24 | Marker Light Damaged | Damage to the marker light lens or bezel mounted on a specific area of the vehicle |  |  |  |  |  |  |  |  |  |  |  |
| 25 | Decal/Paint Stripe Damaged | Damage to a decal, wood grain transfer or paint stripe on a specific area of the automobile | 18 |  |  |  |  |  |  |  |  |  |  |
| 29 | Contamination, Exterior |  | 30 |  |  |  |  |  |  |  |  |  |  |
| 30 | Fluid Spillage, Exterior | Discoloration of an exterior painted or bright metal surface by a fluid substance or airborne material. | 29 |  |  |  |  |  |  |  |  |  |  |
| 34 | Chipped Panel Edge | The same as chipped, but along the edge of a panel, such as a door panel. | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 09 | 11 | 12 | 13 |
| 36 | Incorrect Part or Option not as Invoiced | Part is incorrect or option is incorrect. Not considered transportation damage. |  |  |  |  |  |  |  |  |  |  |  |
| 37 | Hardware - Damaged | Damage type not described by other codes. Door handles, key locks, air horns, grab handles, etc. | 18 |  |  |  |  |  |  |  |  |  |  |
| 38 | Hardware - Loose, Missing | Damage type not described by other codes. Door handles, key locks, air horns, grab handles, etc. | 19 |  |  |  |  |  |  |  |  |  |  |

Please note the table logic works from the left column (or the type code) to the codes on the right. This does not work in reverse. The philosophy is damage will not cure itself in transit.

"14-1" Damage type / severity code is not interchangeable with any other code.

| Similar Damage Codes For Severity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Severity | Matrix Suggestion |  |  |  |
| Damage up to an including 1" in length / diameter - less than 3 cm | 1 | 2 | 3 | 4 | 5 |
| Damage over 1" up to and including 3" in length / diameter - 3 cm up to 8 cm | 2 | 1 | 3 | 4 | 5 |
| Damage over 3" up to and including 6" in length / diameter - over 8 cm up to 15 cm | 3 | 2 | 4 | 5 |  |
| Damage over 6" up to and including 12" in length / diameter - over 15 cm up to 30 cm | 4 | 3 | 5 |  |  |
| Damage over 12" in length / diameter - over 30 cm | 5 | 4 |  |  |  |
| Missing | 6 |  |  |  |  |