# SECTION 09 30 00 – ARCHITECTURAL CLAY SURFACE SYSTEMS

EconiClay™ Architectural Surface System
Full-Length CSI Specification
California WUI / SFM 12-7A Oriented

## PART 1 – GENERAL

1.01 SUMMARY
A. Furnish and install mineral-based architectural clay surface system manufactured from natural modified clay and active soil.
B. Applications include interior and exterior walls, façades, and specialty architectural surfaces.
C. Work includes substrate preparation, accessories, installation, and protection.

1.02 RELATED SECTIONS
A. Section 06 16 00 – Sheathing
B. Section 07 27 00 – Air Barriers
C. Section 07 42 00 – Wall Panels
D. Section 09 29 00 – Gypsum Board

1.03 REFERENCES
A. ASTM E84 – Surface Burning Characteristics
B. ASTM E96 – Water Vapor Transmission
C. ASTM D4060 – Abrasion Resistance
D. ASTM D2047 – Slip Resistance (where applicable)
E. California Building Code (CBC)
F. California Fire Code (CFC)
G. OSFM SFM 12-7A (WUI)

1.04 SUBMITTALS
A. Product Data
B. Installation Instructions
C. Fire Performance Documentation
D. Sustainability Documentation
E. Samples: Minimum 12 x 12 inches

1.05 QUALITY ASSURANCE
A. Manufacturer Qualifications: Minimum five years experience.
B. Installer Qualifications: Approved by manufacturer.
C. Mockups: As directed by Architect.

1.06 DELIVERY, STORAGE, AND HANDLING
A. Deliver in original packaging.
B. Store dry and protected.
C. Handle to prevent damage.

1.07 WARRANTY
A. Manufacturer’s 30-Year Limited Warranty.

## PART 2 – PRODUCTS

2.01 MANUFACTURER
A. Basis of Design: Elementum Surfaces – EconiClay™
B. Substitutions subject to approval.

2.02 MATERIAL DESCRIPTION
A. Mineral-based, inorganic architectural surface.
B. Composed of natural active soil and modified clay.
C. Non-combustible.

2.03 PERFORMANCE REQUIREMENTS
A. Fire Performance: Class A per ASTM E84.
B. Water Absorption: 3–8% typical.
C. Thermal Performance: Contributes to reduced surface heat gain.
D. Seismic Performance: Lightweight and flexible.
E. Durability: Resistant to scratching and fading.

2.04 SUSTAINABILITY
A. 100% mineral-based.
B. Near-zero embodied carbon.
C. No added VOCs.

2.05 WUI / SFM 12-7A COMPLIANCE
A. Intended for use in WUI zones when part of approved assemblies.
B. Compliance is assembly-dependent and subject to AHJ approval.

2.06 ACCESSORIES
A. Manufacturer-approved adhesives.
B. Trims, joint treatments, primers.

## PART 3 – EXECUTION

3.01 EXAMINATION
A. Verify substrates are sound, clean, and dry.
B. Do not proceed until conditions are acceptable.

3.02 PREPARATION
A. Prepare substrates per manufacturer instructions.
B. Ensure proper layout and alignment.

3.03 INSTALLATION
A. Install EconiClay™ in accordance with manufacturer’s published requirements.
B. Panels may be installed on flat or curved surfaces.
C. Maintain uniform joints and appearance.

3.04 WUI INSTALLATION REQUIREMENTS
A. Coordinate with fire-resistive sheathing and barriers.
B. Maintain integrity of wildfire-resistive envelope.
C. Comply with CBC, CFC, and OSFM requirements.

3.05 FIELD QUALITY CONTROL
A. Inspect completed work.
B. Replace defective materials.

3.06 CLEANING AND PROTECTION
A. Clean with manufacturer-approved methods.
B. Protect from damage.

# APPENDIX A – FIRE & WUI BASIS-OF-DESIGN

This appendix defines the fire-resistance and Wildland–Urban Interface (WUI) design intent for the EconiClay™ Architectural Surface System. 
This information is provided for coordination with architectural drawings, code sheets, and Authority Having Jurisdiction (AHJ) review.

## A.1 Fire Performance Design Intent

EconiClay™ is a mineral-based, inorganic architectural surface specified for its non-combustible characteristics. 
The material is intended to achieve Class A surface burning characteristics when tested in accordance with ASTM E84, 
supporting enhanced building fire and life-safety performance through limited flame spread and smoke development.

## A.2 Wildland–Urban Interface (WUI) Design Intent

For projects located within California-designated Wildland–Urban Interface (WUI) Fire Hazard Severity Zones, 
EconiClay™ is intended to be used as part of an approved, code-compliant exterior wall assembly. 
Its mineral-based composition supports resistance to ember exposure, radiant heat, and flame propagation 
when incorporated into assemblies evaluated in accordance with California Office of the State Fire Marshal (OSFM) SFM 12-7A criteria.

## A.3 Assembly-Based Compliance Statement

Compliance with wildfire-resistance requirements is dependent on the complete exterior wall assembly, including 
sheathing, weather-resistive barriers, insulation, attachment methods, and detailing. 
EconiClay™ functions as a non-combustible surface component within such assemblies and does not constitute standalone certification 
for OSFM SFM 12-7A without assembly-level evaluation and AHJ approval.

## A.4 Coordination Requirements

Coordinate EconiClay™ installation with fire-resistive sheathing, air and weather barriers, flashings, and adjacent exterior components 
to maintain continuity of the wildfire-resistive envelope. 
Any deviation from approved assemblies shall be reviewed by the Architect, manufacturer, and AHJ prior to installation.

## A.5  Drawing Note

“EconiClay™ architectural surface system is a mineral-based, non-combustible finish intended for use in exterior wall assemblies 
within California Wildland–Urban Interface (WUI) zones. When installed as part of an approved, code-compliant assembly, 
the system supports wildfire-resilient design objectives and Class A fire performance, subject to AHJ approval.”

# APPENDIX B – WUI WALL ASSEMBLY MATRIX (TYPE III / TYPE V)

This matrix provides representative exterior wall assembly configurations for projects located in California Wildland–Urban Interface (WUI) zones.
Assemblies are provided for design coordination purposes only. Final acceptance is subject to project-specific testing, documentation, and approval by the Authority Having Jurisdiction (AHJ).

## B.1 Assembly Matrix Overview

EconiClay™ functions as a non-combustible exterior surface component within code-compliant wall assemblies evaluated in accordance with OSFM SFM 12-7A criteria.
The following assemblies are typical for Type III and Type V construction and must be coordinated with structural, fire-resistive, and weather-resistive requirements.

| Construction Type | Typical Wall Assembly (Exterior to Interior) | WUI / Fire Notes | Coordination Requirements |
| --- | --- | --- | --- |
| Type III (Non-Combustible Exterior) | EconiClay™ surface panel Fire-resistive WRB Exterior gypsum sheathing (Type X) Metal studs with insulation Interior gypsum board | Assembly to be evaluated per OSFM SFM 12-7A. EconiClay™ acts as non-combustible exterior finish. | Coordinate WRB continuity, joint detailing, and fire-resistive sheathing. |
| Type V (Combustible Construction) | EconiClay™ surface panel Fire-resistive WRB Exterior gypsum sheathing (Type X) Wood studs with insulation Interior gypsum board | Assembly compliance is assembly-dependent. EconiClay™ provides non-combustible exterior surface. | Strict coordination of fastening, WRB, and detailing required. |

## B.2 Notes

1. EconiClay™ does not constitute standalone compliance with OSFM SFM 12-7A.
2. All penetrations, joints, and transitions must be detailed to maintain wildfire-resistive performance.
3. Substitute materials or alternate assemblies require review and approval by the Architect, manufacturer, and AHJ.

# APPENDIX C – WUI WALL ASSEMBLY MATRIX (TYPE I / TYPE II – MID-RISE & PODIUM)

This appendix provides representative exterior wall assembly configurations for Type I and Type II construction,
commonly used in mid-rise, podium, and mixed-use developments within California Wildland–Urban Interface (WUI) zones.
Assemblies are conceptual and must be coordinated with project-specific fire-resistance ratings, testing, and AHJ approval.

## C.1 Assembly Matrix Overview

For Type I and Type II construction, EconiClay™ is intended to function as a non-combustible exterior surface component
within rated wall assemblies. The mineral-based composition supports wildfire-resilient design objectives when incorporated
into assemblies evaluated in accordance with OSFM SFM 12-7A criteria.

| Construction Type | Typical Wall Assembly (Exterior to Interior) | WUI / Fire Notes | Coordination Requirements |
| --- | --- | --- | --- |
| Type I (Fire-Resistive Construction) | EconiClay™ surface panel Fire-resistive WRB Exterior gypsum sheathing (Type X) Continuous insulation Concrete or steel backup wall Interior rated gypsum board | Wall assembly to maintain required fire-resistance rating. EconiClay™ acts as non-combustible exterior finish. Assembly subject to OSFM SFM 12-7A evaluation where required. | Coordinate fire ratings, WRB continuity, insulation attachment, and detailing at slab edges. |
| Type II (Non-Combustible Construction) | EconiClay™ surface panel Fire-resistive WRB Exterior gypsum sheathing (Type X) Metal studs with insulation Interior gypsum board | Non-combustible wall assembly. EconiClay™ provides non-combustible exterior surface. WUI compliance dependent on complete assembly. | Coordinate stud spacing, WRB, fire stopping, and transitions. |

## C.2 Notes

1. EconiClay™ does not provide fire-resistance rating by itself and must be used within rated wall assemblies.
2. Fire-resistance ratings, WUI compliance, and podium conditions shall be verified by the design team.
3. All deviations from listed assemblies require review and approval by the Architect, manufacturer, and AHJ.

# APPENDIX D – DETAIL CALLOUT REFERENCES (SECTIONS & ELEVATIONS)

This appendix establishes standard detail callout numbers for use on architectural sections, wall sections, and exterior elevations.
Callout numbers are intended to coordinate EconiClay™ wall assemblies with drawing details and specifications.

## D.1 Callout Legend

D-1  EconiClay™ Exterior Wall Assembly – Type III (WUI)
D-2  EconiClay™ Exterior Wall Assembly – Type V (WUI)
D-3  EconiClay™ Exterior Wall Assembly – Type I (Podium / Mid-Rise)
D-4  EconiClay™ Exterior Wall Assembly – Type II (Mid-Rise)
D-5  EconiClay™ Typical Panel Joint & Edge Detail
D-6  EconiClay™ Base / Termination Detail
D-7  EconiClay™ Window & Opening Interface Detail

## D.2  Drawing Notation

On architectural sections and elevations, reference the applicable detail callout number as follows:

“EconiClay™ Exterior Wall System – See Detail D-1 / A-5.20”
“EconiClay™ Podium Wall Assembly – See Detail D-3 / A-5.21”

Detail numbers shall correspond to the project drawing index.

## D.3 Coordination Notes

1. Detail callouts shall be coordinated with fire-resistive ratings, WUI requirements, and assembly notes.
2. All EconiClay™ details shall maintain continuity of the weather-resistive barrier, fire-resistive components, and wildfire-resistive envelope.
3. Deviations from referenced details require review by the Architect, manufacturer, and AHJ.

# APPENDIX E – WUI COMPLIANCE STATEMENT (SPECS / DRAWINGS)

The following compliance statement is  for inclusion in project specifications, architectural drawings, 
code sheets, and permit submittal documents for projects located within California Wildland–Urban Interface (WUI) zones.

EconiClay™ is a mineral-based, non-combustible architectural surface intended for use in exterior wall assemblies within California Wildland–Urban Interface (WUI) zones. When installed as part of an approved, code-compliant wall assembly, the product supports wildfire-resilient design objectives and may be incorporated into assemblies evaluated in accordance with California Office of the State Fire Marshal (OSFM) SFM 12-7A requirements, subject to approval by the Authority Having Jurisdiction (AHJ).

Drawing Sheet Alignment:
• Reference this WUI Compliance Statement on General Notes and Code Sheets (A-0.xx).
• Reference this statement on Exterior Elevations and Wall Sections (A-5.xx) in conjunction with EconiClay™ detail callouts (Details D-1 through D-7).
• Coordinate this appendix with Appendix D (Detail Callout References) and Appendices B and C (WUI Wall Assembly Matrices).

# APPENDIX F – FIRE-RESISTANCE-RATED WALL ASSEMBLY REFERENCES (1-HR / 2-HR)

This appendix provides typical fire-resistance-rated wall assembly reference language for coordination with architectural drawings and specifications. EconiClay™ is a non-combustible exterior surface component and does not independently provide a fire-resistance rating. Final rated assembly selection shall be based on tested and listed systems and subject to project requirements and Authority Having Jurisdiction (AHJ) approval.

## F.1 General

EconiClay™ shall be incorporated only as an exterior finish component within complete fire-resistance-rated wall assemblies tested in accordance with ASTM E119 / UL 263 (or equivalent). Fire-resistance ratings are provided by the complete wall assembly, including framing, sheathing, insulation, and interior layers.

## F.2 1-Hour Rated Wall Assembly – Typical Coordination Reference

Provide a 1-hour rated exterior wall assembly (as required by code and design) utilizing a tested/listed system. Typical components may include (exterior to interior):

EconiClay™ non-combustible exterior surface (finish component)

Approved weather-resistive barrier / fire-resistive WRB as required by assembly

5/8 in Type X exterior gypsum sheathing (or as required by listed assembly)

Steel or wood stud framing with insulation as required by listed assembly

5/8 in Type X interior gypsum board (or as required by listed assembly)

Reference a tested/listed 1-hour wall system from one of the following sources (or equivalent): UL Design listings (e.g., U300/U400 series as applicable), GA File (WP) assemblies, or other AHJ-accepted listings matching project conditions.

## F.3 2-Hour Rated Wall Assembly – Typical Coordination Reference

Provide a 2-hour rated exterior wall assembly (as required by code and design) utilizing a tested/listed system. Typical components may include (exterior to interior):

EconiClay™ non-combustible exterior surface (finish component)

Approved weather-resistive barrier / fire-resistive WRB as required by assembly

Two layers 5/8 in Type X exterior gypsum sheathing (or as required by listed assembly)

Non-combustible backup wall or stud framing with insulation as required by listed assembly

Two layers 5/8 in Type X interior gypsum board (or as required by listed assembly)

Reference a tested/listed 2-hour wall system from UL Design listings (as applicable), GA File (WP) assemblies, or other AHJ-accepted listings matching project conditions.

## F.4 Required Disclaimer (Use on Drawings and Specs)

Fire-resistance ratings are based on complete tested/listed wall assemblies. EconiClay™ is a non-combustible exterior finish component and does not independently provide a fire-resistance rating. Assembly selection and compliance shall be verified by the Design Professional and accepted by the AHJ.