Mid Valley Air Park Property Owners Association CC&Rs

Addendum A Set Back Matrix

| Property Boundary Abuts | Use / Structure Type | Governing Setback Restriction | Notes |
| --- | --- | --- | --- |
| Auto-Plane Drive (Centerline) | Any Obstruction (Vehicle, motorhome, camper, boat, trailer, fence, wall, tree, etc.) | 25 feet from the centerline of any auto-plane drive. | Applies to all properties. Additionally, objects within 10 feet of the edge of the auto-plane drive cannot be greater than 2 feet in height. |
| Auto-Plane Drive (Any side bordered by an Auto-Plane Drive) | Any Structure (Does not include Fences or Walls) | 30 feet from the edge of the right-of-way OR 30 feet from the centerline, whichever is furthest. | This setback is the required minimum for yards in both Rural Residential (RR) and Neighborhood Commercial (C-1) districts. |
| Private Property Line (Standard Side/Rear Yard) | Any Structure (RR-1, RR-2 Districts) | 15 feet from the property line. | The minimum requirement for Rural Residential Districts. |
| Private Property Line (Standard Side/Rear Yard) | Any Structure (C-1 District) | 25 feet from the property line. | This is the most stringent setback for standard side/rear yards (Neighborhood Commercial District). |
| Runway (Centerline) | Residential Structure (Heated Living Space) | 200 feet from the centerline of the North-South runway. | This is the most stringent setback. Only applies to properties adjacent to the North-South runway taxiway. |
| Runway (Centerline) This replaces the 1999 Runway Moratorium and is policy when considering architectural plans. | Any new or modified Structure – house, hangar, or other. | Height Restriction – shall placed closer than 160 feet from the centerline of the paved runway, and at that distance shall not exceed 20 feet in height. | Only applies to properties adjacent to the North-South runway taxiway. |
| East-West Landing Area (Centerline) | Any Obstruction (Fence, wall, tree, structure, vehicle, motorhome, camper, boat, trailer, etc.) | 90 feet from the centerline of the East-West Landing Area. | Only applies to properties adjacent to the East-West Landing Area. |
| Taxiway (Paved Edge) | Any Obstruction exceeding 6 feet in height (including but not limited to walls, fences, trees, boat, trailer, vehicle, camper, motorhome, etc.) | 30 feet from the closest paved edge of the North-South taxiway. | Applies to properties adjacent to the North-South runway taxiway. |