DESIGNER SPECIFICATION SECTION

SECTION 13 11 00

CONTENTS OF PRODUCT and NOTES to SPECIFICATION PREPARER

- Perimeter Overflow Gutter Grating (Concrete Gutters)

  - Parallel – parallel straight and parallel radius

  - Perpendicular – perpendicular straight and radius

- Perimeter Overflow Systems – (for Stainless Steel Gutters)

- Area Drain Covers – Slot Drain Drop-Out Box and Grating

- Deck Drain System

- Suction Re-circulation Fittings – Main Drain Sump, Frame & Grates–(SOFA’s)

- Custom Large Area Suction Grates

- Perimeter Custom Width Straight Grate

- Custom Non-Suction Grates

- Wall Weir (straight and radius) with Grate

- DalLUX Series - Tile/Stone Paver Support Grate

- Current Flow Fitting

- Notes to Project Specifier -

  - NOTES required in other Sections of the Project Specifications.

  - Document Requirements for SOFA

  - Project Design Requirements for Tile/Stone Paver Support Grate

SWIMMING POOL SPECIFICATION

SECTION 13 11 00

Perimeter Overflow System – Concrete Gutter

- A perimeter overflow system consisting of a continuous concrete and overflow channel as detailed and shown on the design documents shall be installed on the pool(s).

- The sidewalls, floor of a concrete gutter shall be coated with an approved waterproofing application.

- (PARALLEL GRATING) The grating shall be formed of molded PVC sections, certified NSF/ANSI 50, that are modular, interlocking pieces, UV stabilized. The top section shall have a raised herringbone design and certified to IBC Classification C for Wet-Barefoot inclining platform (ramp) test for swimming pool surroundings and communal shower rooms. Grating shall be 0.47” (12mm) wide and an outside depth of 1.0” (25.4mm) with a middle depth of 1.83” (46.6mm) for load strength requirements. The grating shall be certified under ANSI/AS3996 class A for covers and grates with load exceeding 10kN (2,248lb). The space between the bars shall not exceed 0.315” (8mm) to comply with IBC child finger/toe entrapment guidelines. The grating surface bars shall run parallel to the pool wall and with the gap providing at least 40% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and be provided by the manufacturer. The width of the grating shall allow the insertion of the touchpad holding brackets between the grating and the gutter lip.

All inside and outside corners and custom radii sections under 3-feet inside diameter shall be custom fabricated and strengthened by manufacturer.

Grate width to be 12” (14”; 16”) and supported each side by the pool gutter lip no less than 1.5” in width.

- Grate color to be selected by the Architect/Owner. Colors available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating shall be manufactured by Daldorado LLC. Ph: (888) 509-8128.  No known equal.

(PERPENDICULAR GRATING) The grating shall be formed of molded PVC sections, certified NSF/ANSI 50, that are modular, interlocking pieces, UV stabilized. The top section shall have a raised herringbone design and certified to IBC Classification C for Wet-Barefoot inclining platform (ramp) test for swimming pool surroundings and communal shower rooms. Grating shall be 0.47” (12mm) wide and an outside depth of 1.0” (25.4mm) with a middle depth of 1.83” (46.6mm) for load strength requirements. The grating shall be certified under ANSI/AS3996 class A for covers and grates with load exceeding 10kN (2,248lb). The space between the bars shall not exceed 0.315” (8mm) to comply with IBC child finger/toe entrapment guidelines. The grating surface bars shall run perpendicular to the pool wall and with the gap, providing at least 40% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and be provided by the manufacturer. The width of the grating shall allow the insertion of the touchpad holding brackets between the grating and the gutter lip. All inside and outside corners and custom radii sections under 3-feet inside diameter shall be custom fabricated and strengthened by manufacturer.

Grate width to be 10” (12”;14”; 16”) and supported each side by the pool gutter lip no less than 1.5” in width.

- Grate color to be selected by the Architect/Owner. Colors available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating shall be manufactured by Daldorado LLC. Ph: (888) 509-8128.  No known equal.

Perimeter Overflow Systems – Custom Width Parallel Grate - Straight

The grating shall be formed of molded PVC sections, certified NSF/ANSI 50, that are modular, interlocking sections, UV stabilized, with the top section having a raised herringbone design. Grating shall be 0.625” (15.9mm) wide and an outside depth of 1.0” (25.4mm) with the space between the bars not exceeding 0.375” (9.5mm). The grating will have 4-15mm FRP rods and 5 connector pins for load strength requirements.  FRP rods to be held in place with PVC caps that are glued on the ends. The grating surface bars shall run parallel to the pool wall and with the gap, provide at least 37.5% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and provided by the manufacturer. The width of the grating shall allow the insertion of the holding brackets between the grating and the gutter supports. All inside and outside corners custom fabricated and strengthened by manufacturer.

Grate width to be supported each side by the pool gutter lip no less than 1.5” in width.

(Gutters without outside gutter lip, the grate shall be structurally supported across the width, by alternate support method, at 30” maximum on center, by others.)

- Grate color to be selected by the Architect/Owner. Colors available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating as manufactured by Daldorado LLC. Ph: (888) 509-8128, or approved equal.

Area Drain Covers – Slot Drain Drop-Out Box and Grating

Slot Drain Drop-Out Drain Box Grating (12” x12”) in all pool deck areas shall be custom ordered and supplied with same style of grate to match swimming pool grating along with a custom PVC frame. The grating shall be formed of molded PVC, certified NSF/ANSI 50, UV stabilized, with raised herringbone design and certified to IBC Classification C for Wet-Barefoot inclining platform (ramp) test for swimming pool surroundings. Grating shall be 0.47” (12mm) wide and an outside depth of 1.0” (25.4mm) with a middle depth of 1.83” (46.6mm) for load strength requirements. The grating shall be certified under ANSI/AS3996 class A for covers and grates with load exceeding 10kN (2,248lb). The space between the bars shall not exceed 0.315” (8mm) to comply with IBC child finger/toe entrapment guidelines.

- Grate color to be selected by the Architect/Owner. Colors available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating shall be manufactured by Daldorado LLC. Ph: (888) 509-8128, or approved equal.

Deck Drain System

- The deck drain shall be 4” wide, shall have a 4” (2”) inside depth and be formed of extruded PVC 0.2” (5mm) thick in lengths of 10’-0” in straight or custom radii down to a 6’-0” (2-meter) inside diameter. Each deck drain section shall be provided with EPDM joiners that also perform as expansion joints. The deck drain base shall have 5/8” pre-drilled holes at 18” OC both sides. These holes are provided for the use of threaded rods with lock nuts, for ease of leveling. Size, location and number of storm water (sanitary sewer) drain outlets to be determined by designer and/or local rain/sewer water requirements. The deck drain shall be strength tested for light vehicle traffic. Color: gray only.

- The deck drain grating shall be formed of molded PVC sections, certified NSF/ANSI 50, that are modular, interlocking sections, UV stabilized. The top section shall have a raised herringbone design and certified to IBC Classification C for Wet-Barefoot inclining platform (ramp) test for swimming pool surroundings and communal shower rooms. Grating shall be 0.47” (12mm) wide and an outside depth of .83” (21.1 mm) with a middle depth of 1.1” (27.7 mm) for load strength requirements. The grating shall be certified under ANSI/AS3996 class A for covers and grates with load exceeding 10kN (2,248lb). The space between the bars shall not exceed 0.315” (8mm) to comply with IBC child finger/toe entrapment guidelines. The grating surface bars gap, provide at least 40% open space per foot for unrestricted water capture.

Color to be selected by the Architect/Owner. Available in colors white, bone or gray.

- Dropout drain boxes ~ 12”x12”x12” pre-manufactured, corrosive resistant, non-conductive, non-magnetic. Deck Drain flow inlets to be provided on sides of box to suit project requirements with bottom or side exit drain for connection to storm water (sanitary sewer) system. Color: gray only.

Each drain box to be supplied with grate.

(Each drain box to be supplied with strainer for easier cleanout.)

The 12” x 12” grating shall be formed of molded PVC, certified NSF/ANSI 50, UV stabilized, with the top section having a raised herringbone design. with raised herringbone design and certified to IBC Classification C for Wet-Barefoot inclining platform (ramp) test for swimming pool surroundings. Grating shall be 0.47” (12mm) wide and an outside depth of 1.0” (25.4mm) with a middle depth of 1.83” (46.6mm) for load strength requirements. The grating shall be certified under ANSI/AS3996 class A for covers and grates with load exceeding 10kN (2,248lb). The space between the bars shall not exceed 0.315” (8mm) to comply with IBC child finger/toe entrapment guide lines.

- Grate color to be selected by the Architect/Owner. Colors available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- CAP-SHURE Deck Drain System as manufactured by Daldorado LLC. Ph: (888) 509-8128. No known equal.

Suction Re-circulation Fittings – Main Drain Sump Frame and Grates – (SOFA’s)

- Main suction outlets – SOFA (main drain sumps, frames and grates), sizes as indicated on the drawings, with “25-Year Service Life under VGB, as manufactured, and supplied, by Daldorado LLC.

  - Sumps/drains shall be a fiberglass pre-manufactured SOFA (Suction Outlet Fitting Assembly) that is corrosion resistant, non-conductive, non-magnetic, high strength 8oz fiberglass matt with marine grade white gel coat. SOFA shall have a 2.6” (65mm) FRP angle around the outside perimeter 5” below the top rim and a 2 x 2 FRP T-bar around the perimeter for additional strength. The sump/drain shall be supplied complete with frame and grate. The complete unit shall have been tested and labeled by the manufacturer in compliance with APSP-16 2011, Suction-Outlet Fitting Assemblies for Use in Swimming Pools.

All sumps/drains and grates/covers to be USPC® certified.

Sizes as shown on the drawings. (24x24x30, 18x36x24, 18x36x28, 18x36x34, 18x54x24, 18,54,28, 18x54x34)

- DalMAX-SG/FG-24x24x30 - (max. 10” suction port size)

FLOOR sump/drain - flow rate of 825gpm.

- DalMAX-SG/FG-18x36x24 - (max. 8” suction port size)

FLOOR sump/drain - certified max. flow rate of 1,984gpm.

WALL sump/drain - certified max. flow rate of 1,440gpm.

- DalMAX-SG/FG-18x36x28 - (max. 10” suction port size)

FLOOR sump/drain - certified max. flow rate of 2,480gpm.

WALL sump/drain - certified max. flow rate of 1,909gpm.

- DalMAX-SG/FG-18x36x34 - (max. 12” suction port size)

FLOOR sump/drain - certified max. flow rate of 4,412gpm.

WALL sump/drain - certified max. flow rate of 2,080pm.

- DalMAX-SG/FG-18x54x24 - (max. 10” suction port size)

FLOOR sump/drain - certified max. flow rate of 2,872gpm.

WALL sump/drain - certified max. flow rate of 2,872pm.

- DalMAX-SG/FG-18x54x28 - (max. 12” suction port size)

FLOOR sump/drain - certified max. flow rate of 2,944gpm.

WALL sump/drain - certified max. flow rate of 2,872gpm.

- DalMAX-SG/FG-18x54x34 - (max. 14” suction port size)

FLOOR sump/drain – certified max. flow rate of 4,412gpm.

  - Grate/covers to be supplied complete with the sump/drain and in corresponding sizes to the sump. Grate/cover shall be formed of molded PVC sections that are UV stabilized and certified NSF-50. The top section shall have a raised design and certified to APSP-16 2011. Grating shall be 0.20” (5mm) wide and an outside depth of 1.0” (25.4mm) with a middle depth of 1.45” (36.8mm) for load strength requirements. The grating shall be certified under APSP-16 2017. The space between the bars shall not exceed 0.39” (10mm). The grating shall provide at least 62% open space for unrestricted water flow. Fasteners shall be SS-316 or Brass as tested to APSP-16 2011 and provided by the manufacturer with complete sump, frame and grate unit, no others shall be permitted. Grate/covers shall be USPC certified unblockable in 18”x18” multiples (24”x24”). Color: White (Black)

- DalMAX-GO-1818 (single) x 2 or x 3.

- DalMAX-GO-2424 (single)

  - SOFA (Sump, frame and grate) shall not exceed the flow rates as stated by the manufacturer in compliance with APSP-16 2011. Suction outlets, size and numbers can be changed and specified and ordered through the manufacturer provided design flow rates as certified by APSP-16 2011 are not exceeded.

- Each main drain suction outlet sump shall be supplied and shipped with removable internal stabilizing supports in the sump to protect it from external pressure during the concrete pour, and must allow for easy extraction and disposal following construction.

- Each main drain suction outlet sump shall be supplied and shipped with removable ply-board top cover protector to shield the internals of the sump from debris during concrete pour and throughout the construction period. The cover shall be secured for easy removal and disposal prior to the final securing of the suction grate cover.

- All sump suction ports 12” or less in diameter, shall be supplied with (threaded / slip /flanged) adaptor connectors.

- Optional Winterization: Each large area main drain suction outlet sump shall be supplied with removable ¼” PVC top cover protector to shield the internals of the sump from debris during the winterization period. The cover shall be secured in place of the grates, for the duration of winterization, and be shipped with 3/16” SS screws for seasonal fixing and removal for storage for repeat annual use.

- Each Main Drain Suction Outlet (SOFA) grate cover shall have a manufacturers minimum service life of 25-years. Each Manin Drain Suction Outlet (SOFA) sump body shall have a “facility” life time warranty.

- Main Drain Suction Outlets (SOFA) shall be Dal-MAX manufactured by Daldorado LLC. Ph: (888) 509-8128. No known equal.

- Custom Large Area Custom Suction Grate

- The grate shall be formed of molded PVC sections that is UV stabilized and certified to NSF 50 with the top section having a raised herringbone design. Grating shall be 0.625” (15.9mm) wide and an outside depth of 1.0” (25.4mm) with the space between the bars not exceeding 0.375” (9.5mm). The grating shall have 4-15mm FRP rods and 5-connector pins for load strength requirements.  FRP rods shall be held in place with PVC caps that are glued on the ends. The grating gap, shall provide at least 37.5% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and be provided by the manufacturer. The grating shall allow the insertion of the holding brackets between the grating and the gutter supports.

- 24 x 24 tested and certified to APSP-16 2011, all other sizes greater than 18 inches in one direction and/or 23 inches in the other direction shall have the maximum flow rate calculated using the “Q” formula as stated in the standard.  This flow rate to be laser engraved on 2-ply acrylic and fastened to grate with 316 SS drive screws.  Widths up to 30” do not need additional support.

- Color to be selected by the Architect/Owner. Available only in white. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating as manufactured by Daldorado LLC. Ph: (888) 509-8128, or approved equal.

- Perimeter Custom Width Straight Grate

- The grate shall be (X-inches (6 to 30) wide) formed of molded PVC sections that are modular, interlocking 24” sections, UV stabilized and certified NSF-50 with the top section having a raised herringbone design. Grating shall be 0.625” (15.9mm) wide and an outside depth of 1.0” (25.4mm) with the space between the bars not exceeding 0.375” (9.5mm). The grating shall have 4-15mm FRP rods and 5-connector pins for load strength requirements.  FRP rods shall be held in place with PVC caps that are glued on the ends. The grating gap, shall provide at least 37.5% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and be provided by the manufacturer. The grating shall allow the insertion of the holding brackets between the grating and the gutter supports.

- Color to be selected by the Architect/Owner. Color: available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating as manufactured by Daldorado LLC. Ph: (888) 509-8128 or approved equal.

- Custom Non-Suction Flat Grate – (rectangular, square, triangular, circular, ect.)

- The grate shall be (X-inches (12” minimum wide) (X-inches in diameter (18” minimum dia)) formed of molded PVC sections that are modular, interlocking 24” sections, UV stabilized and certified NSF-50 with the top section having a raised herringbone design. Grating shall be 0.625” (15.9mm) wide and an outside depth of 1.0” (25.4mm) with the space between the bars not exceeding 0.375” (9.5mm). The grating shall have 4-15mm FRP rods and 5-connector pins for load strength requirements.  FRP rods to be held in place with PVC caps that are glued on the ends. The grating gap, shall provide at least 37.5% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and provided by the manufacturer. The grating shall allow the insertion of the holding brackets between the grating and the gutter supports.

- Color to be selected by the Architect/Owner. Color: available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating shall be manufactured by Daldorado LLC. Ph: (888) 509-8128.

- Note to Specifier:

- Grate cover rectangular or square from 12-inches and up in width.

- Grate, greater than a 30-inch span shall require additional structural support.

- Grate cover triangular or circular from 18-inch min. and up.

- Circular and Triangular grates to have 1-inch PVC edging welded/glued to provide a stable and secure edge finish.

- Possible Application for large area grate covers – including but not limited to;

- catch basins

- water-slide run-out over flow grate

- lazy river seat opening cover grate

- water play area gravity flow grate

- bulk heads

- Wall Weir with Grate

- Lazy River straight or radius Wall Weir with Grate shall be pre-manufactured, corrosive resistant. Lengths up to 8-feet.  Radii can be either concave or convex and pre-plumbed per project design detail.

- Weir shall have tabs welded onto the body with predrilled holes provided for the use of threaded rods with lock nuts, for ease of levelling and installation.  Additional tabs with predrilled holes to be utilized for tying into rebar for stability during concrete pour.  The weir shall be provided with internal PVC supports for added strength.

- The weir to be supplied with plywood installed across the opening of the weir to keep debris out of the weir throughout construction.  The front face surface of the weir shall be provided with stainless steel mesh to provide a base that will allow application of pool finish material to the face of the weir.

- Weir Grating shall be parallel grate 6-inch wide. The grating shall be formed of molded PVC sections that are modular, interlocking sections, UV stabilized and certified NSF-50. The top section shall have a raised herringbone design and certified to IBC Classification C for Wet-Barefoot inclining platform (ramp) test for swimming pool surroundings and communal shower rooms. Grating shall be 0.47” (12mm) wide and an outside depth of 1.0” (25.4mm).  The space between the bars shall not exceed 0.315” (8mm) to comply with IBC child finger/toe entrapment guidelines. The grating surface bars shall run parallel to the pool wall and with the gap, provide at least 40% open space per foot for unrestricted water flow. Fasteners shall be SS-316 and be provided by the manufacturer.

- Color to be selected by the Architect/Owner. Color: available in white, bone and gray. The Weir and Grate shall have a minimum 10-year manufacturer’s warranty.

- Weir and Grate shall be manufactured by Daldorado LLC. Ph: (888) 509-8128, or approved equal.

- Custom Drop-Out Box with Strainer

- Dropout Drain Boxes shall be 12”x12”x12” pre-manufactured, corrosive resistant. Drain outlets shall be provided, per project design requirements, to storm water drainage connections. Each drain box shall be supplied with matching grate.

- Optional strainer basket shall be mounted inside drop out box for easy removal and cleaning.

- Grate Color to be selected by the Architect/Owner. Colors available in white, bone and gray. The grating shall have a minimum 10-year manufacturer’s warranty.

- Grating shall be manufactured by Daldorado LLC. Ph: (888) 509-8128, or approved equal.

- DalLUX Tile/Stone Support Grate

Tile/Stone Paver Support Grate shall be provided in interlocking 12” x 12” (305mm x 305mm) PVC sections with recessed groves providing 3.58” (91-mm) openings separated by 5/16” (8-mm) spacers to secure, cut sections of tile or stone paver, as specified by the architect, and allow free water flow through each opening. Across the span of the Tile/Stone Paver Support Grate there shall be 3-sections of tile/stone paver offering four (4), 5/16” (8-mm) openings for water flow. (4-sections of tile/stone paver offering five (5), 5/16” (8-mm) openings for water flow). The open 5/16” (8-mm) open slots for water flow shall be parallel to the pool edge. Tile/Stone Paver Support Grate shall be designed for minimal deflection and tested under AS3996 at over 10Kn (2,000-lbs).

Tile/Stone Pavers (as specified by the architect and supplied and installed by the tile contractor), shall be designed and specified for approval in wet, barefoot areas, in accordance with DIN 51097 (Pendulum DCOF Test) CTIOA, HB 198/AS 4663-2013, EN 13036-4, 15676, EN 14231, etc.), guidelines of minimum wet Pendulum Test Slip Resistance or equal as approved by the Ceramic Tile Institute of America (CTIOA). Each tile section must have beveled sides and ends to avoid bare foot injury. Each Tile/Stone Paver shall be adhered to the support grate bars with a PU based adhesive as recommended by the manufacturer.

All Tile/Stone Paver Support Grate corners shall be custom assembled by the manufacturer with NO exceptions.

Tile/Stone Paver Support Grate: Color: black.

Support Grate shall have a minimum 10-year manufacturer’s warranty.

Tile/Stone Paver Support Grate system shall be provided in interlocking 12” x 12” (305mm x 305mm) PVC sections with recessed groves providing 3.58” (91-mm) openings separated by 5/16” (8-mm) spacers to secure the cut sections of tile or stone paver and allow for water flow through each opening. Across the span of the Tile/Stone Paver Support Grate there shall be 3-sections (4-sections) of tile/stone paver offering four (4), 5/16” (8-mm) openings for water flow (4-slimmer sections of tile/stone paver offering five (5), 5/16” (8-mm) openings for water flow). The open 5/16” (8-mm) slots for water flow shall be parallel to the pool edge.

Tile/Stone Paver Support Grate shall be designed for minimal deflection and tested under AS3996 at over 10Kn (2,000-lbs).

Tile/Stone Pavers (specified by the architect and supplied and installed by the contractor), shall be approved slip-resistance and must have bevelled sides and ends to avoid bare foot injury.

Tile/Stone Pavers shall be adhered to the support grate bars with a PU based adhesive as supplied by the manufacturer.

Tile/Stone Paver Support Grate corners are to be custom assembled by manufacturer and must be used. No exceptions.

- Tile/Stone Paver Support Grate: Color shall be black. Support Grate shall have a minimum 10-year manufacturer’s warranty.

- Support Grate shall be DalLUX as manufactured by Daldorado LLC. Ph: (888) 509-8128. No known equal.

Current Flow Fitting

Lazy River/Current Channel, Current Flow Fitting - shall be pre-manufactured, corrosive resistant, 30” long, 6” wide, 10” deep, with one (1) 4” schedule 40 PVC threaded coupling to allow for a maximum flow rate of 350 to 400 GPM per fitting.

Current Flow Fitting shall be manufactured using Type 1 PVC board, thermal seamed and welded at all joints. The sump shall have a continuous 1.5” PVC water-stop on the perimeter exterior wall placed 3” down from the top of the face cover.

Current Flow Fitting – Face Cover; shall have two (2) x slots x .315 wide to allow for maximum return water velocity of 20 feet/second per Flow Fitting. The face cover slots shall be IBC finger and toe entrapment compliant. The face cover shall have a ¼"D soft durometer rubber gasket around the underside perimeter to create a tight seal when fixed to the sump unit, following hydrostatic pressure testing, on-site using manufacturer supplied tamper proof 316SS flat sheet metal screws into factory installed brass inserts for additional strength and safety.

Current Flow fitting complete with sump and face cover has been IAPMO tested to confirm out-flow of 20 ft/second, with a net pressure drop (head loss) of 3.1 psi.

Current Flow Fitting sump and face cover, shall have a minimum 10-year manufacturer’s warranty.

Current Flow Fitting as manufactured by Daldorado LLC. Ph: (888) 509-8128, no known equal.

- Note to specifier re: Current flow fitting: Manufacturer to determine final length and number of slots in face cover.

(ADDITONAL NOTES TO SPECIFIER)

Information required in other Sections in the Project Specifications are as follow:

SUBMITTAL Documents for Daldorado Product

- Certifications support the QUALITY of a product, NSF-50, STRENGTH of a product, SLIP RESISTANCE of a product.

If, as a designer, or customer, you require a QUALITY product then specify and request the appropriate Laboratory Testing Certifications for each specified product to be submitted during the submittal process.

Daldorado Submittal and Certification documents can be easily obtained and downloaded from the Daldorado LLC website at:    www.daldorado.com

WARRANTY and CERTIFICATIONS for Daldorado Product

- Warranty information supports the manufacturer’s warranty on life expectancy of the product.

- Certifications support the QUALITY of a product, NSF-50, STRENGTH of a product, SLIP RESISTANCE of a product.

If, as a designer, or customer, you require a QUALITY product then request the appropriate Laboratory Testing Certifications for each product to be submitted during the submittal process.

Daldoraro Warranty information, Certifications and Submittal supporting documents can be easily obtained from the Daldorado LLC website at:    www.daldorado.com

OPERATION and MAINTENANCE Manuals

Please request and ensure that the Swimming Pool Contractor supply all documentation as supplied by the SOFA (sump, frame and grate) manufacturer and additional documentation as required by APSP-16 2017 at completion and sign-off of the finished project to the Owner of the facility.

This documentation shall include but not be limited to General Certificate of Conformity (GCC) for each SOFA as required by 16 CFR Part 1450, this must include attachment points and installation of compliant fasteners as supplied by the SOFA manufacturer and as certified under the requirements of APSP-16 2018.

SOFA (Suction Outlet Fitting Assembly) – Labelling and Documentation

MAIN DRAIN’s (SOFA’s) Installation Information - (manufactured sump, frame, grate).

- Each manufactured SOFA (Main Drain) cover/grate shall be supplied complete and packaged with a label made from materials designed to exceed the service life of the cover/grate, and be of sufficient size and format to be legibly filled-in by the contractor/installer at the time the cover/grate is installed.

Instructions shall be provided with the label requiring the cover/grate contractor/installer to fill-in the balance of the information required before giving the label to the pool owner.

The label instructions shall be permanently posted as near as feasible to the pump control, and that an additional copy of this information shall be provided to the pool owner, to be kept with other important pool related documents and/or included in the Operations and Maintenance Manuals at completion of construction.

This Documentation shall include:

- Manufacturers name.

- Cover/grate part number.

- Service life of the cover/grate.

- The certified Cover/grate flow rating(s), as they are permanently marked on the grate by the manufacturer.

- The certified flow rating specific to the Main Drain (SOFA) configuration and installed orientation (wall or floor) for cover/grates permanently marked “For Flow Ratings”.

- Location of the installed Main Drain (SOFA), such that it can be successfully identified in the future.

- The date the cover/grate was installed in the pool, where the month and year indicators on the label are easily identified in the field.

- Equivalent Markings - Field Built - Registered Design Professional Certified Main Drain (SOFA)

# The Registered Design Professional (RDP) that certifies a Main Drain (SOFA) shall either apply the applicable markings to the Main Drain (SOFA) in conformance with APSP-16 2011, Suction-Outlet Fitting Assemblies for Use in Pools, Spas, and Hot Tubs, Sections 8.1 through 8.3 and provide an information label in conformance with Section 8.4, or the equivalent information shall be posted on a permanently mounted sign, that is adjacent to the pool circulation pump(s).

# When posting this sign adjacent the pump(s) is not feasible, the sign shall be posted in the equipment area and clearly indicate which pump(s) to which the Main Drain’s (SOFA’s) is installed.

DalLUX TILE/STONE PAVER Support Grate PROJECT DESIGN REQUIREMENTS

# Above grade or elevated perimeter overflow swimming pools require that the ‘Building Structural Engineer’ design the swimming pool support structure (platform or vault) so that the maximum long term deflection shall not exceed 1/4” in the vault and/or supporting platform floor.

# Tile or Stone Pavers – as specified by the architect, shall be designed and specified for approval in wet, barefoot areas in accordance with DIN 51097 (Pendulum DCOF Test /CTIOA, HB 198/AS 4663-2013, EN 13036-4, TS 15676, EN 14231, etc.), recommendations/guidelines of minimum wet Pendulum Test Slip Resistance or equal as approved by the Ceramic Tile Institute of America.

# DalLUX Stone Tile support grate is supplied either in 3 or 4 stone or tile supports, (4 or 5 slot openings), specify the number of openings according to water capture preferences.

# Tile Contractor cutting and laying the tile or stone pavers shall be prequalified by the general contractor to be proficient in completing similar work on and around pool deck areas. Following the cutting of tile and stone pavers, all edges of each tile/stone paver must be bevelled on all sides, including ends, after cutting, so that they will not cause wet bare foot injury. Each tile or stone paver must be adhered to the tile/stone support grate with a PU based adhesive as recommended by the manufacture of the support grate.

# Tile/Stone Paver Grate support load/deflection testing chart – at the onset of load/deflection details at the onset of cracking in ceramic tile. (15.7 kN equals 3,529 pounds force).

END OF DOCUMENT