Builders Responsibilities for Waterproofing

Shower Pans

- Drains must be same level as floor and anchored (typically plumber’s responsibility). Armored Star can provide this service for a fee.

- Blocking to be placed in-between shower wall studs on top of sole plates.  2”x 8” boards min., set flush with inside edge of wall.

- Benches require nailer boards placed adjacent to and 5” from the lateral vertical edges to allow placement of barrier fins.

- Recessed pans require a flush edge to the shower pan floor.  Chip-out by Armored Star $25 every 15 min.

Decks

- Builder installs flashing.  Typical flashing is 6”x 6”.  Drip edge flashing should have a 4”-6” footing on deck surface.

- Flashing should be tied into the sub-wall.  If house is wrapped, it will be removed approx. 1’ above deck surface.  Builder is responsible for replacement.

- Waterproofing is not suitable for decks to be carpeted.

- Decks should be covered with lt. weight concrete or other protective measures after application.

Basement/Retaining Walls

- Walls to be clean and dry.

- Honeycombing to be filled and trowelled.

- Walls accessible and free sharp objects, i.e. Boards with nails.  Sharp items damage our high pressure hoses.

- Walls less than 30 days cured require primer.  Primer applied for a fee.  Walls require a min. of one week drying time after removal of forms.  More time is required in cooler/rainy weather.