| Valley Fliers |  | Phase 1: Pre-Flight |  |  |
| --- | --- | --- | --- | --- |
| 1402 Auburn Way North, #223 Auburn WA 98002 | Name: |  | _______________________ |  |
|  | Certificate Number: |  | _________________________ |  |
| Instructor: ____________________ | Certificate Type: |  | _________________________ |  |
|  | Ratings: |  | _________________________ |  |
|  |  |  | _________________________ |  |
| Check Out Date: _______________ | Total Flight Time: |  | ________ Last 90 Days: _____ |  |
| Club check out is in two phases: Phase 1 – Pre-flight. Phase 2 – Flight. |  |  |  |  |
| This is an open book check out. The aircraft’s documentation will be required to complete the Phase 1 check out. Read the question and the possible answers, then print the letter of the most correct answer on the line next to the question. |  |  |  |  |

| 1. _____ | The gross weight of C182 N759MA is: |  | - 2800 pounds - 3000 pounds - 2650 pounds |
| --- | --- | --- | --- |
| 2. _____ | The empty weight of C182 N759MA is: |  | - 1829 pounds - 1730 pounds - 1672 pounds |
| 3. _____ | The useable fuel capacity of N759MA is: |  | - 60 gallons total, 55 gallons useable - 70 gallons total, 65 gallons useable - 80 gallons total, 75 gallons useable |
| 4. _____ | What is the total engine oil capacity and, except for an extended flight, when should oil be replenished? |  | - 12 qts, 9 qts minimum, fill to 10 qts. - 10 qts, 8 qts minimum, fill to 9 qts. - 12 qts, 10 qts minimum, fill to 11 qts. |
| 5. _____ | The maximum allowable weight that can be placed in the baggage areas in normal category operations is: |  | - 150 pounds - 200 pounds - 225 pounds |
| 6. _____ | Excessively high engine temperatures, either in the air or on the ground will: |  | - Increase fuel consumption - Result in damage to hoses and fittings - Cause loss of power, excessive oil use and possible internal engine damage |
| 7. _____ | Which statement regarding the use of the cowl flaps is most correct? |  | - Only open cowl flaps during ground operation - Cowl flaps should be open for ground operations and climbs-during cruise they should be adjusted to maintain CHT at approximately two thirds of the normal operating range (green arc) - Cowl flaps are a nuisance and can always remain closed |
| 8. _____ | The first indication of carburetor ice in an aircraft with a constant speed propeller is: |  | - A decrease in RPM - A decrease in manifold pressure - Rough running engine and RPM loss |
| 9. _____ | The normal leaning procedure, using the exhaust temperature gage (EGT), is to set the engine up for cruise power, lean the mixture until the EGT peaks, then: |  | - Enrich the mixture until EGT drops 75 degrees - Leave the mixture at peak EGT - Lean the mixture another 25 degrees |
| 10. ____ | Before the engine is started you notice the manifold pressure gage is indicating about 29” hg. This is because the: |  | - Gage is stuck at the full power indication - Throttle is closed, trapping air in manifold - Manifold pressure equal atmospheric pressure when engine is not running |
| 11. ____ | What is the primary advantage of a constant speed propeller? |  | - To maintain a specific engine speed - To obtain a suitable pitch setting for each situation and power setting - To obtain a selected pitch angle regardless of the flight situation or power setting |
| 12. ____ | When operating an aircraft with a constant speed propeller, which procedure places the least stress on cylinder components? |  | - When decreasing power, 1st reduce RPM - When decreasing power, keep the RPM constant-reduce manifold pressure only - When decreasing power, reduce manifold pressure before RPM |
| 13. ____ | The airspeed indicator in N759MA registers in: |  | - KPH - KTS - MPH |
| 14. ____ | The maximum cross wind component is generally accepted to be 0.25 x Vso. What is the computed maximum cross wind component for this aircraft? |  | - 0.25 x 45 = 11.25 - 0.25 x 50 = 12.50 - 0.25 x 55 = 13.75 |
| 15. ____ | The distance to clear a 50 ft. obstacle in a maximum effort, no wind, 2950 lb. gross weight from a paved runway at sea level with a temperature of 15 C (59 F) and altimeter setting of 29.92” is: |  | - 705 feet - 1430 feet - 1350 feet |
| 16. ____ | If the runway in question 15 was a dry grass strip, the take-off data would have to be adjusted by: |  | - Increase the total distance by 15% - Increasing the ground run by 15% - Increase both the ground run and total distances by 7% of the total to clear a 50 foot obstacle |
| 17. ____ | What engine setting would produce the closest performance to 75% brake horse power at 6,000 feet on a standard day? |  | - 2400 RPM, 22” hg for 12.8 gallons/hour - 2300 RPM, 22” hg for 12.6 gallons/hour - 2200 RPM, 22” hg for 12.3 gallons/hour |
| 18. ____ | With a normally aspirated engine what happens to the manifold pressure during a climb? |  | - The manifold pressure increases approximately 1” hg per 1000’ of climb - The manifold pressure decrease approximately 1” hg per 1000’ of climb - The manifold pressure remains constant due to the variable pitch propeller |
| 19. ____ | What power setting and airspeed is recommended for a cruise climb? |  | - 2400 RPM and 25” hg - 2400 RPM and 24” hg - 2400 RPM and 23” hg |
| 20. | Using the Cessna manual’s loading graph and center of gravity moment envelope for C182 N759MA, calculate the CG for the following: |  |  |
| Item |  | Weight | Moment/1000 |
| Empty Weight |  |  |  |
| Fuel – 40 gallons |  |  |  |
| Pilot and Co-Pilot |  | 320 |  |
| Rear Seats |  | Empty |  |
| Baggage Area |  | Empty |  |
| Total Weight and Moment |  |  |  |
| 21. | Using the Cessna manual’s loading graph and center of gravity moment envelope for C182 N759MA, calculate the CG for the following: |  |  |
| Item |  | Weight | Moment/1000 |
| Empty Weight |  |  |  |
| Fuel – 75 gallons |  |  |  |
| Pilot and Co-Pilot |  | 320 |  |
| Rear Seats |  | 212 |  |
| Baggage Area |  | 70 |  |
| Total Weight and Moment |  |  |  |
| 22. ____ | What is the recommended tire pressure for the main gear tires? |  | - 32 psi on 6.00 x 6 tires - 42 psi on 6.00 x 6 tires - 49 psi on 6.00 x 6 tires |
| 23. ____ | What is the recommended tire pressure for the nose gear tire? |  | - 32 psi on 5.00 x 5 tire - 42 psi on 6.00 x 6 tire - 49 psi on 5.00 x 5 tire |
| 24. ____ | When securing the aircraft, the fuel selector valve should be: |  | - Turned to either the left or right tank to prevent siphoning - Remain on “Both” - Turned to the fullest tank |

| 25. | Provide the following airspeeds (IAS): |  |  |
| --- | --- | --- | --- |
|  | Va | Design maneuvering speed | _______ |
|  | Vno | Maximum cruise speed | _______ |
|  | Vne | Never exceed speed | _______ |
|  | Vy | Best rate of climb speed | _______ |
|  | Vx | Best angle of climb speed, Flaps 20 | _______ |
|  | Vfe | Maximum flap extension speed | _______ |
|  | Vs1 | Power off stall, flaps up | _______ |
|  | Vso | Power off stall, flaps 40 | _______ |
| 26. ____ | What is the recommended final approach speed with the flaps extended? |  | - 70 to 80 MPH - 60 to 70 MPH - 50 to 60 MPH |
| 27. ____ | What airspeed produces the best glide for an engine out glide from 8,000 feet and how far could you glide in a zero wind condition? |  | - 80 MPH IAS, glide distance 16 SM - 70 MPH IAS, glide distance 13 SM - 60 MPH IAS, glide distance 9 SM |
| 28. ____ | What is the stalling speed in a 60 degree bank with full flaps? (forward CG) |  | - 70 MPH IAS - 46 MPH IAS - 64 MPH IAS |
| 29. ____ | What is the approved flap range for take-off? |  | - 0 to 10 degrees - Flaps not recommended for take-off - 0 to 20 degrees |
| 30. ____ | The easiest way to ground handle this aircraft is to use the tow bar. What is the tow bar turning angle limits? |  | - Do not exceed 29 degrees from center - To the limit where it will turn no further - Do not exceed 60 degrees from center |
| 31. ____ | Where is the emergency locator transmitter (ELT) located? |  | - Behind the instrument panel on the left side - In the baggage compartment, near door - Behind the baggage compartment |
| 32. ____ | How is the ELT armed in this aircraft? |  | - A switch on the unit’s case - A switch on the instrument panel, left side - It is always armed |
| 33. ____ | Where is the battery located? |  | - Behind the baggage compartment - Behind the engine, on the firewall - Under the rear seats |
| 34. ____ | What is the voltage of the electrical system? |  | - 14 volt generator, 12 volt battery - 28 volt alternator, 24 volt battery - 38 volt alternator, 36 volt battery |
| 35. ____ | Does C182 N759MA have a radio master switch? |  | - No, shut off each radio individually - Yes, for comm radios only - Yes, individual switches can be left on |
| 36. ____ | What engine RPM is used for a magneto check? |  | - 2000 RPM and 23” hg - 1700 RPM and flat pitch - 2300 RPM and 23” hg |
| 37. ____ | The aircraft is certificated in the normal category. Can you practice spins in this airplane? |  | - Yes, use slow deceleration - No, spins are prohibited - No, unless the rear seats are removed |
| 38. ____ | What is the normal tachometer operating range at sea level? |  | - 2100-2400 RPM - 2000-2500 RPM - 1800-2400 RPM |
| 39. ____ | What is the manifold gage operating range? |  | - 17-30” hg - 15-23” hg - 15-29” hg |
| 40. ____ | What minimum fuel grades (octane) are authorized for use in N759MA and how many fuel drain locations are there? |  | - AV 80/87, AV100LL – 4 drains - AV 80/87, MoGas – 3 drains - AV100LL, AV100 – 3 drains |
| 41. ____ | Who is responsible for the aircraft documentation that must be on board the airplane before flight? |  | - The maintenance officer - The pilot in command - The safety officer |
| 42. ____ | Who is responsible for cleaning the airplane interior and windows after a flight? |  | - The next user - The cleaning crew - The pilot who just completed the flight |
| 43. ____ | Who is responsible for installing the control lock, securing the tie-downs and locking the aircraft’s doors and windows? |  | - The maintenance officer - The person assigned to ramp duty - The pilot who just completed the flight |
| 44. ____ | Who can perform any maintenance or modifications on club aircraft? |  | - Any Club member on the active list - Anyone with a pilot’s license, except students - The maintenance officer or his designee |
| 45. ____ | In order to complete a club check out in the C182 our insurance carrier requires a minimum of 5 hours of logged C182 time. Which of the following statements is most correct: |  | - Pilots that have more than 10 hours in a C182 do not need to complete a club check out - Pilots with less than 100 hours total time require a total of 10 logged C182 hours - Student pilots cannot check out in the C182 |
| 46. ____ | You notice some oil streaks on the cowl and windscreen. When you wipe it off you notice that the oil is very red. This indicates: |  | - The propeller dye pack is activated. The airplane is grounded. - The propeller hydraulic fluid is leaking - The oil contains Marvin’s Mystery Oil |
| 47. ____ | Is a log book endorsement required to act as PIC in a C182 with a 230 horsepower engine? |  | - No. As of August 4, 1997 not considered as a “complex” aircraft - Yes. Considered a “complex” aircraft since November 1, 1973 - Yes. Requires a “high performance” endorsement as of August 4, 1997 |

| Valley Fliers |  | Phase 2: Flight |  |  |
| --- | --- | --- | --- | --- |
| 1402 Auburn Way North, #223 Auburn WA 98002 | Name: |  | _______________________ |  |
|  | Certificate Number: |  | _______________________________ |  |
| Instructor: ____________________ | Certificate Type: |  | _______________________________ |  |
|  | Ratings: |  | _______________________________ |  |
|  |  |  | _______________________________ |  |
| Check Out Date: _______________ | Total Flight Time: |  | __________ Last 90 Days: _________ |  |
| Club check out is in two phases: Phase 1 – Pre-flight. Phase 2 – Flight. |  |  |  |  |
| The Phase 1 check out should be completed and discussed prior to the Phase 2 check out. |  |  |  |  |

| Flight Review |  | Instructor initials indicating satisfactory performance – refer to applicable PTS |
| --- | --- | --- |
| - Pre-Flight Inspection |  |  |
| - Check List and Pre-Start Procedures |  |  |
| - Starting Engine |  |  |
| - Normal Departure Operations - Taxing - Pre Take-off Checks - Normal Take-off - Climb – appropriate power settings - Cruise – appropriate power settings |  | a. ______ b. ______ c. ______ d. ______ e. ______ |
| - Air Work - Steep Turns - Flight at Minimum Controllable Airspeed - Stall Recognition and Recovery - Recovery from Unusual Attitudes by reference to instruments - Simulated Emergency Descent |  | a. ______ b. ______ c. ______ d. ______ e. ______ |
| - Normal Arrival Operations - Descent and check list procedures - Normal landings |  | a. ______ b. ______ |
| - Pattern Work - Cross wind take-off and landing - Short field take-off and landing - Soft field take-off and landing - Go arounds - Zero Flap landing |  | a. ______ b. ______ c. ______ d. ______ e. ______ |
| - After Landing and Post-Flight Procedures |  |  |
| - Remarks: |  |  |
| Overall Completion of Transition or Original Aircraft Check Out |  |  |
| Phase 1 – Ground Instruction | Phase 2 – Flight Review |  |
| Hours of ground instruction completed: _____________ | Hours of flight instruction completed: _______________ |  |
| Instructors signature: ___________________________ | Instructors signature: ___________________________ |  |
| Certificate number: _____________________________ | Certificate number: _____________________________ |  |
| Expiration date: ________________________________ | Expiration date: ________________________________ |  |
| I have received training to operate a Cessna C182 aircraft and completed the ground and flight training noted above. |  |  |
| Pilots signature: _______________________________ | Date: _________________ |  |