COCKPIT			C-172M

- Check paperwork - Remove Control Lock

- Master switch - ON

    - Check Fuel Gauges – Lower Flaps

    - Rotating Beacon – ON

    - Check Nav lights, Strobes, & Pitot heat, if required

- Master OFF / Lights OFF / Pitot Heat OFF

# EXTERIOR INSPECTION

- Check actual fuel levels, fuel caps securely on.

- Baggage door – LOCKED

Tail, check:

- Fuselage and upper wing

- Horiz stabilizer, elevator, trim tab

- Rudder

- Remove tail tie-down

Right Wing, check:

- Fuel sample from wing, if required

- Flap and Aileron

- Wing tip

- Remove wing tie-down, chocks

- Tire & Brake

- Passenger door condition and hinges

Right Front, check:

- Condition of cowling

- Oil level and engine condition: minimum of 6 quarts

- Fuel sample from engine sump

- Nose tire and strut

- Exhaust pipe security and condition

Nose, check:

- Propeller & spinner

- Engine compartment

- Front of cowling

Left Front, check:

- Static Air source

- Engine cowling

- Nose tire and strut

Left Wing, check:

- Pitot tube / Stall warning / fuel vent line

- Remove wing tie down

- Wing tip

- Aileron and Flap

- Fuel sample from wing, if required

- Pilot’s door condition and hinges

- Tire & Brake

# BEFORE STARTING ENGINE

- Seats / seat belts / shoulder harness – ADJUST

- Passenger briefing – COMPLETED

- Cabin Doors – LATCHED

- Brakes – SET or HOLD

- Fuel Selector Valve – BOTH

- Electrical equipment – OFF / Rotating beacon - ON

- Master Switch – ON

# STARTING ENGINE

- Carb Heat – COLD

- Mixture – RICH

- Primer – As Required (2 to 6 strokes, none if engine is warm)

- Throttle – Open ¼”

- Propeller Area – CLEAR

- Ignition Switch – Start, 1000 RPM after starting

- Oil Pressure – green arc (30 sec normally, 60 sec when cold)

# BEFORE TAXI

- Avionics switch – ON

- Intercom and Radios – SET

- Transponder – ALT (1200 for VFR)

- Flaps – UP

- Lights – As Required

- ATIS and Clearance – As Required

# TAXI

- Brakes – TEST

- Turn and Bank – Check operation during turns

- Heading Indicator and Compass – Check operation during turns

# BEFORE TAKE-OFF

- Cabin Doors and Windows – CLOSED AND LOCKED

- Mixture – RICH

- Throttle – 1700 RPM

  - Engine instruments – green arc

  - Suction Gauge – Check (4.6" – 5.4")

  - Ammeter – Slight Charge

  - Magnetos – Check (max 125 RPM drop, 50 RPM difference)

  - Carb Heat – Check

  - Mixture – Rich below 3000 ft, otherwise lean for max RPM

  - Engine – Idle, then 1000 RPM

- Flight Controls – Check for freedom of movement and correctness

- Trim – “Take-Off” setting

- Flight Instruments – SET (heading indicator and artificial horizon)

- Radios and Transponder – SET, Transponder to “ALT”

- Lights – Strobes – ON; Landing/Taxi Lights – As Required

- Clearance – If Required

# NORMAL TAKE-OFF

- Flaps – UP / Carb Heat – COLD

- Throttle – FULL

- Elevator – Lift nose at 55 KIAS (VR)

- Climb speed 70-80 KIAS until clear of obstacles (VY =78 at sea level)

# CLIMB

- Airspeed – 80-90 KIAS (VY =78 KIAS at sea level)

- Power – Full throttle

- Mixture – Rich below 3000 ft, otherwise lean for max RPM

# CRUISE

- Power – 2200 to 2700 RPM (no more than 75% power)

- Mixture – Rich below 3000 ft, otherwise lean for max RPM

# BEFORE LANDING

- Fuel Selector – BOTH

- Mixture – RICH

- Carb Heat – ON (apply full heat prior to closing throttle)

- Lights – As Required; Landing/Taxi Light - ON

- Airspeed – 60-70 KIAS (flaps up)

- Flaps – AS DESIRED (below 85 KIAS)

# AFTER LANDING (WHEN CLEAR OF RUNWAY)

- Flaps – UP

- Carb Heat – COLD

- Radios – As Required

- Lights – As Required (Strobes – OFF)

- Pitot Heat – OFF

- Taxi Clearance – As Required

# ENGINE SHUTDOWN

- Radios & Electrical Equipment - OFF

- Engine – 1000 RPM

- Mixture – Idle cut-off

- Master and Mags – OFF

- All Switches – OFF

- Fuel selector – Right Tank

- Air Vents – CLOSED

# SECURE AIRCRAFT

- Parking Brake – SET

- Control Lock – INSTALLED

- Tie down airplane / set wheel chocks / pitot cover

- Lock Aircraft

- Flight Plan – CLOSE (if filed)

## SHORT FIELD (Max Performance)

## Take-Off

- Flaps – UP

- Carb Heat – COLD

- Brakes – HOLD

- Throttle – Full Open

- Check engine instruments – Green arc

- Brakes – RELEASE

- Elevator – slightly tail low

- Climb speed (VX) 59 KIAS (55 KIAS with 10 deg of flaps)

## Landing

- Mixture – RICH

- Carb Heat – Apply FULL prior to closing throttle

- Flaps – Full Down

- Airspeed – 60 KIAS (until flare)

- Brakes – APPLY HEAVILY

- Wing Flaps – RETRACT (increases traction for braking)

## SOFT FIELD

## Take-Off

- Flaps – 10 deg.

- Carb Heat – COLD

- Rolling takeoff from taxi position

- Elevator – Full Nose Up

- Throttle – Full Open

- Lift off at minimum airspeed – accelerate in ground effect

- Climb speed 60 KIAS with Flaps 10 until clear of obstacles, 
             then 78 (VY) and Flaps UP

## Landing

- Mixture – RICH

- Carb Heat – Apply FULL prior to closing throttle

- Flaps – Full Down

- Airspeed – 60 KIAS with full flaps

- Maintain engine power during landing flare

- Immediately after touchdown – hold full back elevator

- Do not come to a complete stop until ready to park.

EMERGENCY PROCEDURES

# ENGINE FIRE ON START

- Continue cranking engine

- If engine starts, run at 1700 RPM for 1 minute and shut down

- If start is unsuccessful: Crank with Mixture – IDLE CUT-OFF and 
Throttle – FULL OPEN to attempt to suck flames back into engine

- Shut down engine

  - Mixture – Idle Cut-Off

  - Fuel Selector – OFF

  - Master – OFF

  - Mags – OFF

- Exit aircraft and use Fire Extinguisher As Required

# ENGINE FAILURE AFTER TAKE-OFF

- Airspeed – 	65 KIAS (flaps UP)
		60 KIAS (flaps DOWN)

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – As Required (40 deg. Recommended)

- Master – OFF

- Cabin Doors – UNLATCH prior to touchdown

# ENGINE FAILURE DURING FLIGHT

- Carb Heat - ON

- Slow to Best Glide Speed: 65 KIAS

- Determine, and head for, an Emergency Landing area

If Engine Restart is Feasible:

- Fuel Selector Valve – BOTH

- Mixture – RICH

- Ignition Switch – BOTH (or START if propeller is stopped)

- Primer – IN and LOCKED

If Engine Cannot (or Should Not) be Restarted:

- Transmit MAYDAY, Squawk 7700 
(emergency frequency: 121.5, or last ATC frequency you were on)

- Airspeed – 	65 KIAS (flaps UP)
		60 KIAS (flaps DOWN)

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – As Required (40 deg. Recommended)

- Master – OFF prior to touchdown

- Doors – UNLATCH prior to touchdown

# ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Master – OFF

- Cabin Heat and Air Vents – CLOSED (overhead vents open)

- Airspeed – 110 KIAS (or faster) to blow out fire

- Land as soon as possible

# ELECTRICAL FIRE IN FLIGHT

- Master – OFF

- All Radio / Electrical Switches – OFF

- Vents / Cabin Heat and Air vents – CLOSED

If fire appears out and power is necessary for the continuance of flight:

- Master – ON

- Circuit Breakers – CHECK for faulty circuit, do not reset

- MINIMUM required electrical equip. – ON ONE AT A TIME, 
   delay after each until short circuit is found (then that switch OFF)

- Cabin Air and Heat, vents open after fire is extinguished.

# PRECAUTIONARY LANDING WITH ENGINE POWER

Inspection Pass:

- Fly over selected field with flaps 20O at 60 KIAS

- After inspection, go-around

Landing Pass:

- Radio, Electrical Switches – OFF

- Flaps – 40O (on final approach)

- Airspeed – 60 KIAS

- Master – Off

- Doors – UNLATCH prior to touchdown

# DITCHING

- Transmit MAYDAY on 121.5, Squawk 7700

- Secure or Jettison heavy objects (expect aircraft to flip upside down
   and the cabin to flood immediately upon landing)

- Approach into the wind if winds are high and seas heavy. With light 
  winds, land parallel to swells

- Flaps – 20O - 40O (Maintain 300 ft/min descent at 55 KIAS)

- Doors – UNLATCH prior to touchdown

- Do NOT inflate life vests or raft until AFTER evacuation of aircraft

| Light Gun Signals | On Ground | In Flight |
| --- | --- | --- |
| Steady Green | Cleared for Take-off | Cleared to land |
| Flashing Green | Cleared for Taxi | Return for landing (to be followed by steady green) |
| Steady Red | STOP | Give way to other aircraft, continue circling |
| Flashing Red | Taxi clear of runway | Airport unsafe – do not land |
| Flashing White | Return to starting point | Not applicable |
| Alternating Red & Green | Exercise caution | Exercise caution |