COCKPIT					C-182Q N759MA

- Check paperwork

- Remove Control Lock

- Master switch - ON

    - Check Fuel gauges – Lower Flaps

    - Cowl Flaps – OPEN

    - Rotating Beacon – ON

    - Check NAV lights, Strobes, & Pitot Heat, if required

- Master OFF / Lights OFF / Pitot Heat OFF

# EXTERIOR INSPECTION

- Check actual fuel levels, fuel caps securely on.

- Baggage door – LOCKED

Tail, check:

- Fuselage and upper wing

- Autopilot Static Air source (both sides of fuselage)

- Horiz stabilizer, elevator, trim tab, rudder

- Remove tail tie-down

Right Wing, check:

- Flap and Aileron

- Wing tip

- Remove wing tie-down, chocks

- Tire & Brake / Wheel pant security

- Passenger door condition and hinges

- Fuel sample from wing, if required

Right Front, check:

- Static Air source

- Condition of cowling

- Nose tire and strut / Wheel pant security

- Exhaust pipe security and condition

Nose, check:

- Propeller & spinner

- Engine compartment

- Front of cowling

Left Front, check:

- Static Air source

- Fuel sample from engine sump

- Oil level and engine condition: minimum of 10 quarts

- Engine cowling

- Nose tire and strut / Wheel pant security

Left Wing, check:

- Pitot tube / Stall warning / fuel vent line

- Remove wing tie down

- Wing tip

- Aileron and Flap

- Fuel sample from wing, if required

- Pilot’s door condition and hinges

- Tire & Brake / Wheel pant security

# BEFORE STARTING ENGINE

- Seats / seat belts / shoulder harness – Adjust

- Passenger briefing – COMPLETED

- Cabin Doors – LATCHED

- Brakes – SET or HOLD

- Fuel Selector Valve – BOTH

- Cowl flaps - OPEN

- Electrical equipment – OFF / Rotating beacon - ON

- Master Switch – ON

# STARTING ENGINE

- Carb Heat – COLD

- Mixture – RICH

- Propeller – HIGH RPM (full in)

- Primer – As Required (2 to 6 strokes, none if engine is warm)

- Throttle – Open 1/2 INCH

- Propeller Area – CLEAR

- Ignition Switch – Start, 1000 RPM after starting

- Oil Pressure – green arc

# BEFORE TAXI

- Avionics switch – ON

- Intercom and Radios – SET

- Transponder – ALT (1200 for VFR)

- Flaps – UP

- Lights – As Required

- ATIS and Clearance – As Required

# TAXI

- Brakes – TEST

- Turn and Bank – Check operation during turns

- Heading Indicator and Compass – Check operation during turns

# BEFORE TAKE-OFF

- Cabin Doors and Windows – CLOSED AND LOCKED

- Throttle – 1700 RPM

  - Engine instruments – green arc

  - Suction Gauge – Check (4.6" – 5.4")

  - Ammeter – Slight Charge

  - Magnetos – Check (max 150 RPM drop, 50 RPM difference)

  - Carb Heat – Check

  - Propeller – CYCLE (HIGH to LOW, then full in)

  - Mixture – Rich below 3000 ft, otherwise lean for max RPM

  - Engine – Idle, then 1000 RPM

- Autopilot – TEST, then OFF (If desired for use during flight)

- Flight Controls – Check for freedom of movement

- Trim – “Take-Off” setting

- Flight Instruments – SET (heading indicator and artificial horizon)

- Radios and Transponder – SET, Transponder to “ALT”

- Lights – Strobes – ON; Landing/Taxi Lights – As Required

- Clearance, if required

# NORMAL TAKE-OFF

- Flaps – 0 – 20 DEG

- Carb Heat – COLD

- Throttle – FULL

- Elevator – Lift nose at 50 KIAS (VR)

- Climb speed 70 KIAS (flaps 20 deg), 80 KIAS (flaps UP)

# SHORT FIELD TAKE-OFF

- Flaps – 20 DEG

- Carb Heat – COLD

- Brakes – HOLD

- Throttle – FULL

- Brakes – Release

- Climb Speed – 57 KIAS (until clear of obstacles0

- Wing Flaps – Retract slowly after reaching 70 KIAS

# NORMAL CLIMB

- Airspeed – 85-90 KIAS

- Power – 23 INCHES manifold pressure

- Propeller – MAX of 2380 RPM

- Mixture – Rich below 3000 ft, otherwise lean for max RPM

- Cowl Flaps – OPEN as required

# MAX PERFORMANCE CLIMB

- Airspeed – 78 KIAS at sea level to 72 KIAS at 10,000 feet

- Power – FULL Throttle and MAX RPM

- Mixture – Rich below 3000 ft, otherwise lean for max RPM

- Cowl Flaps – OPEN

# CRUISE

- Power – Max of 75% power, 15-23 inch MP / 2100-2380 RPM

- Mixture – Rich below 3000 ft, otherwise lean for max RPM

- Cowl Flaps – CLOSED

# BEFORE LANDING

- Fuel Selector – BOTH

- Mixture – RICH (Lean for high altitude operations)

- IFR Approach:

  - Heading Indicator – SET to Magnetic Compass

  - NAV Radios – SET for Approach

  - Tower / CTAF – SET as STANDBY

  - Approach – BRIEF Minimums / Timing / Missed Approach

- Carb Heat – ON (apply full heat prior to closing throttle)

- Propeller – FULL FORWARD

- Cowl Flaps – CLOSED

- Lights – As Required

- At Night or IMC:

  - Landing/Taxi Light – ON

  - Pilot Controlled Runway Lights – ON

- Airspeed – 70-80 KIAS (flaps up)

- Flaps – AS DESIRED (10 below 140 KIAS, 10-40 below 95 KIAS)

# SHORT FIELD LANDING

- Mixture – RICH (Lean for high altitude operations)

- Carb Heat – ON (apply full heat prior to closing throttle)

- Propeller – FULL FORWARD

- Cowl Flaps – CLOSED

- Flaps – 40 (below 95 KIAS)

- Lights – As Required

- Airspeed – 60-70 KIAS (flaps down)

- Brakes – Apply Heavily

- Wing Flaps – Retract for maximum braking

# AFTER LANDING (WHEN CLEAR OF RUNWAY)

- Flaps – UP

- Carb Heat – COLD

- Cowl Flaps – OPEN

- Transponder – ALT

- Radios – As Required

- Lights – As Required (Strobes – OFF)

- Taxi Clearance – As Required

# ENGINE SHUTDOWN

- Radios & Electrical Equipment - OFF

- Engine – 1000 RPM

- Mixture – Idle cut-off

- Master and Mags – OFF

- All Switches – OFF

- Air Vents – CLOSED

# SECURE AIRCRAFT

- Parking Brake – SET

- Control Lock – INSTALLED

- Cowl Flaps – CLOSED

- Tie down airplane / set wheel chocks / pitot cover

- Lock Aircraft

- Flight Plan – CLOSE (if filed)

## EMERGENCY PROCEDURES

# ENGINE FIRE ON START

- Continue cranking engine

- If engine starts, run at 1700 RPM for 1 minute and shut down

- If start is unsuccessful: Crank with Mixture – IDLE CUT-OFF and 
Throttle – FULL OPEN to attempt to suck flames back into engine

- Shut down engine

  - Mixture – Idle Cut-Off

  - Fuel Selector – OFF

  - Master – OFF

  - Mags – OFF

- Exit aircraft and check for damage

# ENGINE FAILURE AFTER TAKE-OFF

- Airspeed – 70 KIAS (flaps UP)
	      65 KIAS (flaps DOWN)

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – As Required (40 Deg. Recommended)

- Master – OFF

- Cabin Doors – UNLATCH prior to touchdown

# ENGINE FAILURE DURING FLIGHT

- Slow to Best Glide Speed: 70 KIAS

- Carb Heat - ON

- Determine, and head for, an Emergency Landing area

If Engine Restart is Feasible:

- Fuel Selector Valve – BOTH

- Mixture – RICH

- Ignition Switch – Left / Right / BOTH (START if propeller is stopped)

- Primer – IN and LOCKED

If Engine Cannot (or Should Not) be Restarted:

- Transmit MAYDAY, Squawk 7700 
(emergency frequency: 121.5, or last ATC frequency you were on)

- Airspeed – 70 KIAS (flaps UP)
	      65 KIAS (flaps DOWN)

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – As Required (40 deg. Recommended)

- Master – OFF prior to touchdown

- Doors – UNLATCH prior to touchdown

# ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Master – OFF

- Cowl Flaps – OPEN

- Cabin Heat and Air Vents – CLOSED (overhead vents open)

- Airspeed – 100 KIAS (or faster) to blow out fire

- Land as soon as possible

# ELECTRICAL FIRE IN FLIGHT

- Master – OFF

- All Radio / Electrical Switches – OFF

- Vents / Cabin Heat and Air vents – CLOSED

If fire appears out and power is necessary for the continuance of flight:

- Master – ON

- Circuit Breakers – CHECK for faulty circuit, do not reset

- MINIMUM required electrical equip. – ON ONE AT A TIME, 
   delay after each until short circuit is found (then that switch OFF)

- Cabin Air and Heat, vents open after fire is extinguished.

PRECAUTIONARY LANDING WITH ENGINE POWER

Inspection Pass:

- Fly over selected field with flaps 20O at 65 KIAS

- After inspection, go-around

Landing Pass:

- Radio, Electrical Switches – OFF

- Flaps – 40O (on final approach)

- Airspeed – 65 KIAS

- Master – Off

- Doors – UNLATCH prior to touchdown

DITCHING

- Transmit MAYDAY on 121.5, Squawk 7700

- Secure or Jettison heavy objects (expect aircraft to flip upside down
   and the cabin to flood immediately upon landing)

- Approach into the wind if winds are high and seas heavy. With light 
  winds, land parallel to swells

- Flaps – 10O (Maintain 300 ft/min descent at 65 KIAS)

- Doors – UNLATCH prior to touchdown

- Do NOT inflate life vests or raft until AFTER evacuation of aircraft

| Light Gun Signals | On Ground | In Flight |
| --- | --- | --- |
| Steady Green | Cleared for Take-off | Cleared to land |
| Flashing Green | Cleared for Taxi | Return for landing (to be followed by steady green) |
| Steady Red | STOP | Give way to other aircraft, continue circling |
| Flashing Red | Taxi clear of runway | Airport unsafe – do not land |
| Flashing White | Return to starting point | Not applicable |
| Alternating Red & Green | Exercise caution | Exercise caution |