PA28-180

COCKPIT

- Check paperwork - Remove Control Lock

- Master switch - ON

    - Check Fuel gauges – Lower Flaps

    - Rotating Beacon – ON

    - Check Nav lights, Strobes, & Pitot heat, if required

- Master OFF / Lights OFF / Pitot Heat OFF

# EXTERIOR INSPECTION

Right Wing, check:

- Flap and Aileron

- Wing tip

- Remove wing tie-down, chocks

- Tire & Brake

- Fuel Level / Fuel sample if required

Right Front, check:

- Condition of cowling

- Oil level and engine condition: minimum of 6 quarts

- Nose tire and strut

- Exhaust pipe security and condition

Nose, check:

- Propeller & spinner

- Engine compartment

- Front of cowling

- Air Inlet clear of obstructions

Left Front, check:

- Engine cowling

- Fuel sample from engine sump if required

- Nose tire and strut

Left Wing, check:

- Fuel Level / Fuel sample if required

- Pitot tube / Static Source / Stall warning

- Tire & Brake

- Remove wing tie down

- Wing tip

- Aileron and Flap

Tail, check:

- Horiz stabilizer, elevator, trim tab

- Rudder

- Remove tail tie-down

Right Fuselage check:

- Baggage compartment secure

- Baggage door locked

# BEFORE STARTING ENGINE

- Flaps – UP

- Seats / seat belts / shoulder harness – Adjust

- Passenger briefing – COMPLETED

- Cabin Door – LATCHED

- Brakes – Set or Hold

- Fuel Selector Valve – Fullest Tank

- Electrical equipment – OFF / Rotating beacon - ON

- Master Switch – ON

# STARTING ENGINE

- Carb Heat – COLD

- Mixture – RICH

- Primer – As Required (2 to 6 strokes)

- Throttle – Open ¼”

- Electric Fuel Pump – ON (confirm fuel pressure)

- Propeller Area – CLEAR

- Ignition Switch – Start, 1000 RPM after starting

- Oil Pressure – green arc (30 sec normally, 60 sec when cold)

- Ammeter – Charging

# BEFORE TAXI

- Fuel Pump – OFF, Check Fuel Pressure

- Avionics switch – ON

- Intercom and Radios – SET

- Transponder – ALT (1200 for VFR)

- Lights – As Required

- ATIS and Clearance – As Required

# TAXI

- Brakes – TEST

- Turn and Bank – Check operation during turns

- Heading Indicator and Compass – Check operation during turns

# BEFORE TAKE-OFF

- Cabin Door and Window Vent – CLOSED AND LOCKED

- Throttle – 2000 RPM

  - Engine instruments – green arc

  - Suction Gauge – Check (4.6" – 5.4")

  - Ammeter – Shows Electrical Load

  - Magnetos – Check (max 175 RPM drop, 50 RPM difference)

  - Carb Heat – Check

  - Mixture – Rich below 5000 ft, otherwise lean for max RPM

  - Engine – Idle, then 1000 RPM

- Flight Controls – Check for freedom of movement

- Trim – “Take-Off” setting

- Flight Instruments – SET 
	(heading indicator, artificial horizon, Altimeter)

- Radios and Transponder – SET, Transponder to “ALT”

- Lights – Strobes – ON; Landing/Taxi Lights – As Required

- Fuel Pump – ON

- Clearance, if required

# NORMAL TAKE-OFF

- Flaps – UP

- Carb Heat – COLD

- Throttle – FULL

- Elevator – Lift nose at 60 MPH (VR)

- Climb speed 85 MPH (VY ) until clear of obstacles

# CLIMB

- Airspeed – 85-100 MPH (VY )

- Power – Full throttle

- Mixture – Rich below 5000 ft, otherwise lean for max RPM

- Fuel Pump – OFF

# CRUISE

- Power – 2200 to 2700 RPM (no more than 75% power)

- Mixture – Rich below 5000 ft, otherwise lean for max RPM

- Switch Fuel Tanks – As Required 
	(Fuel Pump – ON when switching tanks)

# BEFORE LANDING

- Fuel Selector – Fullest Tank

- Fuel Pump – ON

- Mixture – RICH

- Carb Heat – OFF (unless Carb Ice is suspected)

- Lights – As Required

- Airspeed – 85 MPH (flaps up)

- Flaps – AS DESIRED (below 115 MPH)

# AFTER LANDING (WHEN CLEAR OF RUNWAY)

- Flaps – UP

- Carb Heat – COLD

- Fuel Pump – OFF

- Radios – As Required

- Lights – As Required (Strobes – OFF)

- Taxi Clearance – As Required

# ENGINE SHUTDOWN

- Radios & Electrical Equipment – OFF

- Engine – IDLE

- Mixture – IDLE CUT-OFF

- Master and Mags – OFF

- All Switches – OFF

- Air Vents – CLOSED

# SECURE AIRCRAFT

- Control Lock – INSTALLED

- Tie down airplane / set wheel chocks / pitot cover

- Lock Aircraft

- Flight Plan – CLOSE (if filed)

SHORT FIELD (Max Performance)

## Take-Off

- Flaps – 25 Deg

- Carb Heat – COLD

- Fuel Pump – ON

- Brakes – HOLD

- Throttle – Full Open

- Check engine instruments – Green arc

- Brakes – RELEASE

- Elevator – slightly tail low

- Climb speed (VX) 74 MPH

## Landing

- Mixture – RICH

- Carb Heat – COLD (unless Carb Ice is suspected)

- Fuel Pump – ON

- Flaps – Full Down

- Airspeed – 74 MPH (until flare)

- Brakes – APPLY HEAVILY

- Wing Flaps – RETRACT (increases traction for braking)

- Fuel Pump – OFF

## SOFT FIELD

## Take-Off

- Flaps – 25 deg.

- Carb Heat – COLD

- Fuel Pump – ON

- Rolling takeoff from taxi position

- Elevator – Full Nose Up

- Throttle – Full Open

- Lift off at minimum airspeed – accelerate in ground effect

- Climb speed (VX) 74 MPH with Flaps 25 until clear of obstacles, 
then 85 (VY) and Flaps UP

- Fuel Pump – OFF

## Landing

- Mixture – RICH

- Carb Heat – COLD (unless Carb Ice is suspected)

- Fuel Pump – ON

- Flaps – Full Down

- Airspeed – 74 MPH with full flaps

- Maintain engine power during landing flare

- Immediately after touchdown – hold full back elevator

- Do not come to a complete stop until ready to park.

- Flaps – UP

- Fuel Pump – OFF

EMERGENCY PROCEDURES

# ENGINE FIRE ON START

- Continue cranking engine

- Throttle – OPEN

- Mixture – IDLE CUT-OFF

- Fuel Pump – OFF

- Fuel Selector – OFF

- Exit aircraft and use Fire Extinguisher As Required

# ENGINE FAILURE AFTER TAKE-OFF

- Airspeed –  80 MPH or as appropriate

If Engine Re-Start is Feasible

- Fuel Selector Valve – SWITCH TANKS

- Fuel Pump – ON

- Mixture – RICH

- Carb Heat – ON

# ENGINE FAILURE DURING FLIGHT

- Carb Heat - ON

- Slow to Best Glide Speed: 80 MPH

- Determine, and head for, an Emergency Landing area

If Engine Restart is Feasible:

- Fuel Selector Valve – SWITCH TANKS

- Mixture – RICH

- Engine Gages – Check

- Primer – IN and LOCKED

- Ignition Switch – LEFT / RIGHT / BOTH 
	(or START if propeller is stopped)

- Vary Throttle and Mixture settings

POWER OFF LANDING

If Engine Cannot (or Should Not) be Restarted:

- Airspeed –  80 MPH

- Transmit MAYDAY, Squawk 7700 
(emergency frequency: 121.5, or last ATC frequency you were on)

- Ignition Switch – OFF

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Wing Flaps – As Required (FULL Recommended)

- Master – OFF prior to touchdown

- Door – UNLATCH prior to touchdown

- Seat Belt and Shoulder Harness – TIGHTEN

# ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Fuel Pump – OFF

- Master – OFF

- Magneto Switch / Ignition – OFF

- Throttle – CLOSED

- Airspeed – Dive to blow out fire

- Land as soon as practicable

# ELECTRICAL FIRE IN FLIGHT

- Master – OFF

- Vents – OPEN

- Cabin Heat – OFF

- Land as soon as practicable

If fire appears out and power is necessary for the continuance of flight:

- All Electrical Switches – OFF

- Master –  ON

- Circuit Breakers – CHECK for faulty circuit, DO NOT RESET

- MINIMUM required electrical equip. – ON ONE AT A TIME, 
   delay after each until short circuit is found (then that switch OFF)

- Cabin Air and Heat – OPEN after fire is extinguished

ALTERNATOR FAILURE

- Detected through a Zero reading on the Ammeter, confirm by turning 
   on an electrical device and seeing if the load increases.

- Reduce Electrical Load

- Alternator Circuit Breakers – CHECK

- ALT Switch (part of the Master Switch) – OFF for 30 sec, then ON

DITCHING

- Transmit MAYDAY on 121.5, Squawk 7700

- Secure or Jettison heavy objects (expect aircraft to flip upside down
   and the cabin to flood immediately upon landing)

- Approach into the wind if winds are high and seas heavy. With light 
  winds, land parallel to swells

- Flaps – 25O - 40O (Maintain 300 ft/min descent at 80 MPH)

- Door – UNLATCH prior to touchdown

- Do NOT inflate life vests or raft until AFTER evacuation of aircraft

| Light Gun Signals | On Ground | In Flight |
| --- | --- | --- |
| Steady Green | Cleared for Take-off | Cleared to land |
| Flashing Green | Cleared for Taxi | Return for landing (to be followed by steady green) |
| Steady Red | STOP | Give way to other aircraft, continue circling |
| Flashing Red | Taxi clear of runway | Airport unsafe – do not land |
| Flashing White | Return to starting point | Not applicable |
| Alternating Red & Green | Exercise caution | Exercise caution |