| Condensate Handling |  |  |
| --- | --- | --- |
| SEQUENCE OF JOB STEPS: | HAZARDS | RECOMMENDATIONS/ COMMENTS |
| 1. Assess and survey area for any potential hazards fill out JSA | Uncontrolled release of hazardous energy Working area clear of hazards | Implement control measures before proceeding. Document pre-job work plan |
| 2. Position truck for job | Obstacles in the area Environmental release Exposure to hazardous liquids | Position truck to see the load valve. This allows you to see the complete job while you are doing it. |
| 3. Hook up grounding cable to wellhead | Worn spots or wickers in grounding cable Tripping hazard | Make sure to pull out extra cable and lay it flat on the ground, so you won't trip on it. Scratch off paint with clamp when installing to get a good connection |
| 4. Connect hose to tank valve with camlock fitting | Environmental release Untethered camlock ears may become loose due to vibration from loading | Make sure to secure camlock fitting and tie hose ears shut Inspect all hoses and fittings for excessive wear or abrasions |
| 5. Open valve at tank and check for any leaks in connections | Environmental release Pinch points Untethered camlock fittings may become loose due to vibration from loading | Clean and lube all fittings when installing Be sure to secure camlock fitting shut while loading Check all hoses and fittings for leakage before engaging pump |
| 6. Open vent valve on truck tank and open fire valves | Failure to open vent valve may cause damage to truck tank Hazardous release of vapor gas from tank | Inspect all plumbing for leakage before engaging pump to load |
| 7. Open valve on truck and engage roper pump to load product | Venting gas from truck tank can be hazardous | Check tank vent line to make sure its venting do not breathe while checking the vent Make sure to have your 4 gas meter on and functioning |
| 8. When loaded shut valve at tank and open hose ears slowly to give it air to suck out hose | Environmental release Muscle strain | After venting hose lift it and walk it out to truck to suck leftovers out of hose and close valve at truck Drain any leftovers into metal bucket not on the ground |
| 10. Disengage pump remove hose and put cap on camlock at truck | Environmental release Pinch point | Always install caps on camlocks of truck before travel |
| 11. Close vent valve and fire valves on truck | Release of flammable, toxic and LEL hazards | Always keep tank shut in when traveling |
| 12. Put away hoses and fittings | Muscle strain Protective gloves | Always wear PPE, gloves, eye protection, FR clothing, etc. |
| 13. |  |  |
| 14. |  |  |
| CAUTION: Flammable liquid (i.e., any liquid with a flashpoint of 140 degrees fahrenheit or less) shall require the use of a combustibility monitor on a continuous basis during fluid transfer to ensure the safe disbursement of vapors. The use of FRC clothing is mandatory. Operations shall be terminated immediately anytime the LEL exceeds 10% in the work environment. Avoid lines of fire. SDS is required on location prior to transfer operations. |  |  |