VILLAGE OF

MILLINGTON ILLINOIS

ZONING

1995

COMPREHENSIVE AMENDMENT 2006

AMENDMENTS AND CORRECTIONS

2019

TABLE OF CONTENTS

SECTION												PAGE

1.00		TITLE										   	    1

2.00		INTENT AND PURPOSE							    	    1

2.01		Definitions										    2

3.00		GENERAL PROVISIONS							    	    8

3.01		Construction Words								     	    8

3.02		Interpretation									   	    9

3.03		Scope of Regulations								    	    9

3.04		Permit or License Pending Changes in Zoning				   	  10

3.05		Accessory Uses									  10

3.06		Building Height, Bulk and Lot Coverage					  	  11

3.07		Lot Area and Dimension								  12

3.08		Access to Public Street								  13

3.09		Number of Buildings on a Zoning Lot					   	  13

3.10		Existing Special Uses								  	  13

3.11		Use Not Specifically Permitted in Districts					  	  13

4.00		ESTABLISHMENT OF ZONING DISTRICTS				   	  14

4.01		Classifications of Zones								  14

4.02		Zoning Maps									 	  15

4.03		Abandonment of Designated Use							  15

4.04		Zoning of Annexed Land								  15

4.05		Boundary Lines									  15

5.00		AGRICULTURAL DISTRICT							  16

5.01		A-1 Agricultural District								  16

6.00		RESIDENTIAL DISTRICT								  17

6.01		Purpose										  17

6.02		R-1A Traditional Single Family Residence District			 	 	  18

6.03		R-1B Single Family Residence District						  20

6.04		R-2 General Residence District							  21

7.00		GENERAL BUSINESS DISTRICT							  23

7.01		Purpose									 	  23

7.02		B-1 Local Business District								  23

7.03		B-2 General Business District							  27

8.00		MANUFACTURING DISTRICTS							  29

8.01		Purpose										  29

8.02		M-1 Manufacturing Districts								  29

Village of Millington

Zoning Ordinances 2019				                     ii

9.00		SIGNS											  33

9.01		Purpose and Intent									  33

9.02		Definitions										  33

9.03		Administration									  35

9.04		Violations										  35

9.05		Permitted and Non Permitted Signs 							  36

9.06		Signs on Residential Property							  37

9.07		Signs on Charitable, Educational and Similar Organizations			  37

9.08		Business Signs									  37

9.09		Temporary Signs									  37

9.10		Temporary Subdivision Advertising Signs						  39

9.11		Pennants or Banners									  39

9.12		Construction Requirements								  40

9.13		Inspections										  40

9.14		Nuisances										  41

9.15		Maintenance										  41

9.16		Removal of Certain Signs								  41

9.17		Unsafe and Unlawful Signs								  42

9.18		Exemptions and Zoning Restrictions							  42

9.19		Building and Electrical Code								  42

9.20		Permits and Fees									  42

9.21		Variations										  42

9.22		Legal Non Conforming Signs							  43

9.23		Grievance Procedure									  43

Table 9-1											 	  43

10.00		OFF STREET PARKING AND LOADING						  44

10.01		Purpose										  44

10.02		General Provisions Parking and Loading						  44

10.03		Additional Regulations Parking 							  46

10.04		Additional Regulations Loading							  46

10.05		Location of Parking Requirements							  47

10.06		Schedule of Parking Requirements							  47

10.07		Off Street Loading and Unloading Facilities						  49

11.00		NON CONFORMING BUILDERS AND USES					  50

11.01		Continuance of Use									  50

11.02		Discontinuance of Use								  50

11.03		Change of Use									  50

11.04		Termination and Removal of Non Conforming Uses Building and Structures          51

11.05		Repairs and Alterations								  51

110.6		Damage and Destruction								  51

Village of Millington

Zoning Ordinances 2019

iii

11.07		Additions and Enlargements								  52

11.08		Use District Changed									  52

12.00		SPECIAL USES									  52

12.01		Purpose										  52

13.00		VARIATIONS										  55

13.01		Granting										  55

13.02		Applications										  56

13.03		Hearing										  56

13.04		Report on Variations									  56

13.05		Standards										  56

13.06		Conditions										  57

13.07		Time Limit on Variations								  57

14.00		ADMINISTRATIVE PROVISIONS							  58

14.01		Enforcement Officer									  58

14.02		Creation and Membership								  58

14.03		Meetings										  58

14.04		Appeals										  58

14.05		Hearing an Appeal									  58

14.06		Vote Required on Appeals								  59

14.07		Fees											  59

14.08		Amendments										  59

14.09		Separability										  59

14.10		Penalty										  59

Village of Millington

Zoning Ordinance 2019

iv

Village of Millington

SECTION 1.00 TITLE

This comprehensive amendment shall be known and may be cited and referred to as the 	ZONING ORDINANCE for the Village of Millington, Illinois.

SECTION 2.00—INTENT AND PURPOSE

This comprehensive amendment to the Millington Zoning Ordinance is adopted for the following purposes:

To promote and protect the public health, safety, morals, comfort and general welfare of the people.

To divide the Village and the surrounding one and one half mile jurisdictional area into zones or districts restricting and regulating therein the location, erection, construction, reconstruction, alteration and use of buildings structures and land for residence, business and manufacturing and other specified uses.

To protect the character and the stability of the residential, business and manufacturing areas within the Village and the one and on half mile surrounding area and to promote the orderly and beneficial development of such areas.

To provide adequate light, air, privacy and convenience of access to property;

To regulate the intensity of use of lot areas and to determine the area of open space surrounding buildings necessary to provide adequate light and air to protect the public health.

To establish building lines and the location of buildings designed for residential, business, manufacturing or other uses within such areas.

To fix reasonable standards to which buildings or structures shall conform therein.

To prohibit uses, buildings or structures incompatible with the character of development of intended uses within specified zoning districts.

To prevent additions to or alterations or remodeling of existing buildings or structures in such a way as to avoid the restrictions and limitations imposed here under.

To protect against fire, explosion, noxious fumes and other hazards in the interest of the public health, safety, comfort and general welfare.

To conserve the taxable value of land and buildings throughout the Village and surrounding one and one half (1 1/2)  mile area.

To provide for the elimination of non conforming uses of land, buildings and structures which are adversely affecting the character and value of desirable in each district.

And to define and limit the powers and duties of the administrative officers and bodies as provided herein.

Zoning Ordinance Amendment 2018

1

2.01 DEFINITIONS

ACCESS: The right to cross between public and private property allowing pedestrians and vehicles to enter and leave property.

ACCESSORY USE OR ACCESSORY STRUCTURE: A use or structure incidental and subordinate to the main use for the property and located on the same lot as the main use.

ALLEY: A street which affords only a secondary means on access to property.

APARTMENT: A room or suite of rooms in a multiple family structure, which is arranged, designed, used or intended to be used as a single housekeeping unit. Complete kitchen and bath facilities, permanently installed, must be included for each apartment.

AUTOMOBILE LAUNDRY: A building or portion thereof where automobiles are washed with the use of a chain conveyor and blower or steam cleaning device is employed.

AUTOMOBILE REPAIR, MAJOR: Engine rebuilding or major reconditioning of worn or damaged motor vehicles or trailers: collision service, including body, frame straightening or repair and over all painting of vehicles.

AUTOMOBILE REPAIR, MINOR: Incidental repairs, replacement of parts and motor service to automobiles but not including any operation specified under “Automobile Repair, Major”.

AUTOMOBILE SERVICE STATIONS: A facility where gasoline, stored only in underground tanks, kerosene, lubricating oil or grease, for operation of automobiles, are offered for sale directly to the public, on the premises and including minor accessories and the services for automobiles and washing automobiles where no chain conveyor, blower or steam cleaning device is employed.

AUTOMOBILE WRECKING YARD: A premises used for the storage or sale of used automobile or truck parts or for the storage, dismantling or abandonment of junk, obsolete automobiles, trailers, trucks, machinery or parts thereof.

AWNING: A roof like veering, which projects from the wall of a building or overhangs the public way.

BASEMENT: A story partly or wholly underground. Where more than one half (½) of it is above the average level of the adjoining ground.

BOARDING HOUSE: An establishment other than a hotel or restaurant where meals are provided for compensation to at least four (4) persons but not more than twelve (12) persons who are not members of the keeper’s family residing on the premises.

BLOCK: A tract of land bounded by streets or in lieu of a street or streets, by a public park, cemetery, railroad right of way, bulkhead lines, a shoreline of waterway or boundary line of the Village.

Zoning Ordinance Amendment 2018

2

BUILDING: Any structure with a substantial walls and roof, securely affixed to the land, which is designed or intended for the shelter enclosure or protection of persons, animals, chattels or property of any kind.

BUILDING AREA: he space remaining on a lot after the minimum open space requirements of this ordinance or any other has been complied with.

BUILDING HEIGHT: The vertical distance measured from the established grade opposite the middle of the front of the building to the highest point of the roof in the case of a flat roof: to the deck line of a mansard roof and to the mean height level between eaves and ridge of a gable, hip or gambrel roof provided that where buildings are set back from the street line, the height of the building may be measured from the average elevation of the finished lot grade at the front of the building.

BUILDING NONCONFORMING: Any building which does not conform to the regulations of this ordinance prescribing the maximum floor area ratio, required yards, coverage, height and setbacks, minimum required spacing between buildings on a single lot and minimum required usable open space for the district in which such building is located.

BUILDING PRINCIPAL: A building in which is conducted the main use of the lot on which it is situated.

BUILDING LINE: A line parallel to the street line at a distance from it, regulated by the front yard requirements, set up in this ordinance.

CELLAR: A cellar is a story having more than one half of its height below the curb level or below the highest level of adjoining ground. A cellar shall not be counted as a story for the purpose of height measurements.

CLUB OR LODGE, PRIVATE: A “private club” or “lodge” is a non profit association of persons, who are bona fide members paying annual dues, which owns, hires or leases a building or portion thereof.

CURB LEVEL: The level of the established curb in front of the building measured at the center of such front. Where a building faces on more than one street the “curb level” shall be the average of the level of the curbs at the center of the front of each street. Where no curb elevation has been established, the mean level of the land immediately adjacent to the building shall be considered the “curb level”.

DWELLING, MULTI FAMILY: A building or portion thereof, designed for occupancy by three (3)) or more families living independently of each other.

DWELLING, SINGLE FAMILY: A detached building containing one (1) dwelling unit and designed for occupancy by one (1) family only.

DWELLING, TOWN HOME (TOWNHOUSE): Not more than four(4) attached one family party wall dwellings, nor more than two and one half (2 ½) stories in height.

Zoning Ordinance Amendment 2018

3

DWELLING, TWO FAMILY: A detached building containing two (2)  dwelling units and designed for occupancy by two (2) families.

DWELLING UNIT: One (1) or more rooms in a building designed for occupancy by one (1) family and having not more than one (1) cooking facility.

FAMILY: An individual or two or more persons related by blood, marriage, legal adoption or legal guardianship, living together as one housekeeping using one kitchen and providing meals or lodging to not more than two additional persons, excluding servants or a group of not more than five unrelated persons living together as one housekeeping unit using one kitchen.

FENCE: A structure or natural growth used as a boundary, screen, separation, means of privacy, protection or confinement:

Fence Natural: A natural fence is a fence of natural growth, such as trees, deciduous shrubs, 	evergreens, etc.

Fence Open: An open fence is one where visibility at right angles to any surface thereof is not 	reduced by more  sixty seven  (67%) percent opacity at right angles to the fence.

Fence Solid: A fence including gates which conceals from view of adjoining properties, open 	storage materials and/or operations conducted behind the fenced area, at greater than sixty seven 	(67%) percent opacity. Less than thirty three (33%) percent of the face area of the solid fence 	consists of openings (two solid per one open)  at right angles to the fence.

FLOOR AREA RATIO (FAR):  The numerical value obtained by dividing the total floor area of a building or buildings by the gross lost area o which such building or buildings are located.

FUEL BULK STATION: A place where crude petroleum, gasoline, naphtha, benzene, benzyl, kerosene or other flammable liquid which has a flash point at or below two (200) hundred degrees Fahrenheit (closed cup tester) is stored for wholesale purposes, where the aggregate capacity of all storage tanks is more than six (6000) thousand gallons.

GARAGE PRIVATE: A detached accessory building or portion of the principal building designed arranged use or intended to be used for the storage of automobiles owned or used by the occupants of the premises.

GARAGE, PUBLIC: A building used for the care, incidental servicing or sale of automobile supplies or where motor vehicles are parked or stored for remuneration, hire or sale within the structure, but not including trucks, tractors, truck trailers and commercial vehicles exceeding one and one half (1 1/2) ton capacity.

GARAGE BUS OR TRUCK: A building which is used or intended to be used for the storage of motor trucks, truck trailers, tractors and commercial vehicles exceed one and one half (1 1/2) ton capacity.

GRADE GROUND LEVEL: The average elevation of the finished ground elevation at the centers of all walls of a building, except that if a wall is parallel to and within five (5) feet of a sidewalk, the sidewalk elevation nearest the center of the wall shall constitute the ground elevation.

Zoning Ordinance Amendment 2018

4

GUEST HOUSE: Living quarters within a detached accessory building located on the same premises with the principal building, for use by temporary guests of the occupants on the premises.

HEIGHT OF BUILDING: The vertical distance from the grade to the highest point of the coping of a flat roof, to the deck line of a mansard roof or to the center height between the highest and lowest points on other types of roofs.

HOME OCCUPATIONS: A gainful occupation or profession carried on by an occupant of a dwelling unit carried on wholly within the principal building or within a building accessory thereto, by only members of the family occupying the premises. No article shall be sold or offered for sale on the premises except such as is produced by the occupation on the premises and no mechanical or electrical equipment shall be installed or maintained other than such as is customarily incidental to domestic use. There shall be no exterior display, no exterior sign except as allowed by the sign regulations for the district in which such “home occupation” is located, no exterior storage of materials, no other exterior display, no exterior sign except as allowed by the sign regulations for the district in which such “home occupation” is located, no exterior indication of the “home occupation” or variation from the residential character of the principal building. Offices, clinics, doctors offices, hospitals, barber shops, beauty parlors, dress shops, millinery, shops, tearooms, restaurants, tourist homes, animal hospitals and kennels, among others, shall not be deemed to “home occupations”.

HOSPITAL OR SANITARIUM:   An institution devote primarily to the maintenance and operation of facilities for the diagnosis, treatment or are for not less than twenty four (24) hours in any week of the three (3) or more non related individuals suffering from illness, disease, injury, deformity or other abnormal physical conditions. The term “hospital” as used in this ordinance does not apply to institutions operating solely for the treatment of insane persons, drug addicts, liquor addicts or other types of cases necessitating restraint of patients and the term “hospital” shall not include convalescents, nursing shelter or boarding homes.

HOTEL, APARTMENT: A building containing dwelling units or individual guest rooms, the majority of which are for permanent guests.

HOTEL OR MOTEL:  A building in which more than five (5) rooms or suites are reserved to provide living and sleeping accommodations for temporary guests, with no provisions in such rooms for cooking in any individual room or suite.

JUNK YARD: Any place where waste, scrap metal, paper, rags or similar materials are bought, sold, exchanged, stored, baled, packaged, dissembled or handled, including auto and building wrecking yards, outside a completely enclosed building.

KENNEL, COMMERCIAL: Any lot or premises or portion thereof on which more than four (4) dogs, cats and other household domestic animals, over four (4) months of age, are kept or on which more than two (2) such animals are boarded for compensation.

LABORATORY: A place devoted to experimental study or testing and analyzing substances.

Zoning Ordinance Amendment 2018

5

LINE OF BUILDING:  A line parallel to the nearest lot line drawn through the point of a building or a group of buildings nearest to such lot line, exclusive of such features specified as being permitted to extend into a yard.

LIVESTOCK: Domestic animals of types customarily raised or kept on farms for profit or other purposes.

LIVESTOCK FEEDING YARD:  An enclosure designed or used for the purpose of the concentrated feeding or fattening of livestock for market.

LIVESTOCK SALES YARD: An enclosure designed or used for holding livestock for the purposes of sales or transfer by auction, consignment or other means.

LOADING AND UNLOADING SPACE: An open hard surfaced area of land other than a street or a public way the principal use of which is for the standing, loading, and unloading of motor vehicles, tractors and trailers.

LOT: The Word lot: When used alone shall mean a tract designated as such on a plat properly recorded.

LOT AREA: The total horizontal area within the lot lines of a lot exclusive of streets and easements of access to other property.

LOT CORNER: A lot abutting on two or more right of ways other than an alley, at their intersection.

LOT COVERAGE: The area of a lot occupied by the principal building or buildings and accessory buildings.

LOT DEPTH: The horizontal distance between the front and rear lot lines measured in the main direction of the side lot lines.

LOT FRONTAGE:  The front of a lot shall be that boundary of a lot along a public street: for a corner lot the owner may elect either street as the front lot line.

LOT LINE:  The property line bounding lot.

LOT LINE INTERIOR: A side lot line common with another lot.

LOT LINE REAR:  The rear lot line is the lot line or lot lines most nearly parallel to and most remote from the front lot line. Lot lines other than front or rear lot lines are side lines.

LOT LINE SIDE:  Any lot line not a front or rear lot line.

LOT REVERSE CORNER:  A corner lot the rear of which abuts upon the side of another lot whether across an alley or not.

Zoning Ordinance Amendment 2018

6

LOT THROUGH: A lot having frontage on two parallel or approximately parallel streets and which is not a corner lot.

LOT WIDTH: The horizontal distance between the side lot lines measured at right angles to the lot depth at the established front building line.

MOBILE HOME: A trailer house not less than thirty two (32) feet in body length designed for long term occupancy and containing a flush toilet, a tub or shower bath and kitchen facilities with water supply, electrical supply and sewage disposal and connected to outside systems.

MOTOR FREIGHT TERMINAL: A building in which freight, brought to said building by motor truck is assembled and sorted for routing in intrastate and interstate shipment by motor truck.

NONCONFORMING STRUCTURE OR USE: A lawful existing structure or use at the time this ordinance or any amendment thereto becomes effective, which does not conform to the requirement of the zone in which it is located.

NURSING HOME OR REST HOME:  A private home for the care of children or the aged or infirm or a place for those suffering bodily disorders, but including facilities for the treatment of sickness or injuries or for surgical care.

NURSERY, DAY CARE CENTER:  Place where care is provided for compensation for three or more children under the age of four (4) years, for periods of more than four (4) hours but not exceeding twenty four(24) hours.

NURSERY SCHOOL: A place where care provided for compensation for children from four (4) to six (6) years of age.

OWNER: An owner of property or the authorized agent of an owner.

PARCEL DELIVERY STATION:  A building in which commodities, sold at retail, are assembled and routed for delivery to retail customers located within the area.

PARKING AREA PRIVATE: An open, hard surfaced area, other than a street or public way, designed, arranged and made available for the storage of private passenger automobiles only, of occupants of the building or buildings for which the parking area developed and is accessory.

PARKING AREA PUBLIC: An open hard surfaced area, other than a street or public way used for the storage of passenger automobiles and commercial vehicles under one and one half (1 ½ ) ton capacity and available to the public, whether compensation, free or as an accommodation to the clients or customers.

PARKING SPACE, AUTOMOBILE: Space within a public or private parking area of not less than one hundred and sixty two (162)square feet (eight and one half (8 1/2) feet by nineteen (19) feet), exclusive of access drive, or aisles, ramps, columns, or office and work areas, for the storage of one (1) passenger automobile or commercial vehicle under one and one half (1 1/2) ton capacity.

Zoning Ordinance Amendment 2018

7

PERSONAL WIRELESS SERVICE FACILITY: See definition 47 USC 332. this definition includes a tower and/or antenna incidental to providing personal wireless services.

PORCH: A permanent roofed over structure, projecting from the  walls of a main structure.

RADIO TOWER OR TELEVISION TOWER: Tower and/or antenna to transmit commercial radio or televising signals. This definition does not include a tower and/or antenna incidental to providing  personal wireless services.

SIGN: All definitions for signs are contained in Section 9.02 of this Ordinance.

STORY: That portion of a building included between the surface of any floor and the surface of the floor above it, the space between the floor and the ceiling next above it. Any portion of a story exceeding fourteen (14) feet in height shall be considered as an additional story for each fourteen(14) feet or fraction there of.

STORY, HALF:  A half story is that portion of a building under a gable, hip or mansard roof, the wall plates of which on at least two opposite exterior walls, are not more than four and one (4 ½) half feet above the finished floor of each story. In the case of one family dwellings, two family dwellings and multiple family dwellings less than three (3) stories in height, a half story in a sloping roof shall not be counted as a story for the purpose of this ordinance. In the case of multiple family dwellings or more stories in height, a half story shall be counted as a story.

STREET: A street is a public thoroughfare which affords traffic circulation and principal means of access to abutting property, including avenue, place, way, drive, lane, boulevard, highway, road and any other thoroughfare, except an alley.

1. Arterial Street:  An arterial street is a major or minor street of greater continuity which is 	intended to serve as a large volume traffic way for both the immediate area and region beyond 	and may be designated on the Villages' Comprehensive Plan, as a principal or minor arterial, 	parkway, tollway, freeway, expressway or equivalent term to identify those streets comprising 	the basic structure of the street plan.

2. Collector Street: A collector street is a secondary street used primarily to carry traffic from 		minor streets to arterial streets.

3. Minor Street: A minor street of limited continuity used primarily for access to abutting 	residential properties.

4. Other Streets:

a. Cul-de-sac Street is a minor street of short length, having one (1) end open to traffic 			and being permanently terminated at the other end by a vehicular turn around.

b. Private Street is an un dedicated street which is privately owned and maintained, or 			an easement of access benefiting a dominant tenant.

Zoning Ordinance Amendment 2018

8

STRUCTURE: Something constructed or built and having a fixed base on, or fixed connection to the ground or another structure.

STRUCTURE ALTERATION: Any change to the supporting members of a structure including foundations, bearing walls or partitions, columns, beams, girders, or any structural change in the roof or in the exterior walls.

TRAILER HOUSE: A vehicle or similar portable device originally designed or presently constructed to permit human occupancy for living or sleeping purposes.

TRAILER PARK:  A lot upon which two (2) or more trailer houses occupied for living or sleeping purposes are located, regardless of whether a charge is made for such accommodations.

USE: The purpose for which land or a structure is designed, arranged, or intended, or for which it is occupied or maintained.

USED CAR LOT:  An area on which used care, trailers, or trucks are displayed for sale or trade.

YARD: An open space on a lot with a principal building or group of buildings which is unoccupied and unobstructed from its lowest level upward, except as otherwise permitted in this ordinance.

YARD FRONT: A yard extending across the full width of the lot and lying between the front line of the lot and the nearest line of the building.

YARD REAR: A yard extending across the full width of the lot and lying between the rear line of the lot and nearest line of principal building.

YARD SIDE: The part of the yard lying between the nearest line of the principal building and a side lot line and extending from the required front yard (or from the front line, if there is no required front yard) to the required rear yard.

SECTION 3.00- GENERAL PROVISIONS

3.01	CONSTRUCTION OF WORDS

Whenever any word in the ordinance importing the plural is used, any single matter, party or 	person shall be deemed to be included, although distributive words may not have been used. 	When any subject matter, party, or person is referred to in this ordinance by words importing the 	singular number only, or the masculine gender, several matters,  parties,  or persons females as 	well as males and bodies corporate shall be deemed to be included. The words "person, firm or 	corporation" shall be deemed to include any association or organization of any kind. Words in 	the present shall include the future. The words "this ordinance" shall be held and taken to mean 	the entire ordinance as amended, including each and every section thereof. The word "village" 	whenever used in this ordinance shall be held and taken to mean the Village of Millington.  The 	words  "written" or "in writing" may include printing. Provided that these rules of construction 	shall not be applied to any section of this ordinance which contained any express provisions

Zoning Ordinance Amendment 2018

9

excluding such construction or where the subject matter or content of such section may be 	repugnant thereto.

3.02	INTERPRETATION

A. Minimum Requirements. The provisions of this zoning ordinance shall be held to be 	minimum requirements for the promotion of public health, safety, morals and welfare.

B. Relations With Other Laws.   Where the conditions imposed by any provision of the 	amended zoning ordinance upon the use of land or buildings or upon the bulk or building are 	either more restrictive or less restrictive than comparable conditions imposed by any other 	provision of this ordinance or other law, ordinance, resolution, rule or regulation of any kind, 	the regulations which are more restrictive (or which impose higher standards or requirements) 	shall govern.

C. Effect of Existing Agreements.  This ordinance is not intended to abrogate any easement, 	covenant or any other private agreement provided that where the regulations of this amended 	ordinance are more restrictive (or impose higher standards or requirements) than such easement, 	covenants or other private agreements, the requirements of this amended ordinance shall 	govern.

3.03 	SCOPE OF REGULATIONS

A. Change in Structures or Use.  Except as may otherwise be provided in Section 11.00, 		"Nonconforming Buildings, Structures and Uses", all buildings erected hereafter, all uses of 	land or buildings established hereafter, all structural alteration or relocation of existing buildings 	occurring hereafter and all enlargements of or additions to existing uses occurring hereafter 	shall be subject to all regulations of this amended ordinance which are applicable to the zoning 	district such buildings, uses or land shall be located.

B. NONCONFORMING BUILDINGS STRUCTURE AND USES.  Any lawful building, 	structure or use existing at the time of the enactment of this amended ordinance may be 		continued even though such building, structure or use does not conform to the provisions of this 	amended ordinance for the district in which it it located and whenever a district shall be 	changed here after, the then existing lawful use may be continued subject to the provisions of 	Section 11.00 "Nonconforming Buildings, Structures and Uses".

C. Building Permits. When a building permit for a building or structure has been issued in 	accordance, with law prior to the effective date of this amended ordinance, and provided that 	construction is begun within six(6) months of such effective date and diligently prosecutes to 	completion, said building or structure may be completed in accordance with the approved plans 	on the basis of which the building permit has been issued and further may be upon completion 	be occupied under certificate of occupancy by the use for which originally designated, subject 	thereafter to the provisions of Section 11.00 "Nonconforming Building, Structures and Uses.

Zoning Ordinance Amendment 2018

10

D. Utilities No permit for connection with or service from any municipally operated water 	distribution system or other utility shall be connection with or service to any building or 	premises used or maintained in violation of the provisions of this ordinance.

3.04	PERMIT OR LICENSE PENDING CHANGES IN ZONING

No permit for erection of any building or structure or license or permit for the conduct of any use, shall be issued for a period of three (3) months after question of amending the zoning ordinance, so as to prohibit the use of building contemplated in the area concerned, has been referred by the Village Board to a board or commission to hold a public hearing on the question of adopting such amendment. Provided that if final action by the Village Board is not taken on the question within three (3) months of the time the matter is so referred to hold a public hearing, the permit shall be issued. If within such three (3) month period the governing body of the municipality shall pass an ordinance amending the zoning ordinance so as to prohibit such building, structural or use no such permit shall be issued.

If a permit for any such building or structure, or license for the conduct of any such business or use, has been issued prior to such reference, but the business or use has not been established, or no substantial part of the construction has been completed at the time of such reference such license or permit shall be

suspended and no action taken there under for a period of three (3) months after the question of amending the zoning ordinance has been to so referred. If final action by the Village Board is not taken on the question within three (3) months of the time of reference, the rights under the permit or license may be exercised. If within such three (3) month period the governing body of the municipality shall pass an ordinance prohibiting the use or structure on the site involve, such prohibition shall be applicable to of such permit or license.

3.05	ACCESSORY USES

An accessory use shall comply with all requirements for a principal use, except as this ordinance specifically allows to the contrary and shall comply with the following limitations.

1. An accessory building may not be located nearer to any interior lot line than that permitted 	for the principal building, if extended. However, when an accessory building is located in the 	rear yard, it may then be located within three (3) feet of the interior lot line, but not nearer 	than five (5) feet of the rear lot line.

2. An accessory building shall not be erected prior to the establishment or construction of the 	principal building to which it is accessory or to which it is intended to be accessory.

3. No accessory building shall be located on a reversed corner lot beyond the front yard required 	on the adjacent lot to the rear, nor located nearer than five (5) feet to the side lot line of the 	adjacent lot.

4. An "accessory use " includes, but is not limited to the following:

a.  A Cildren's playhouse, garden house, and/or private greenhouse.

b.  A garage, shed, carport or building for domestic storage.

Zoning Ordinance Amendment 2018

11

c.  Incinerators incidental to residential use.

d.  Storage of merchandise normally carried in stock on the same lot with any retail 			service or business use, unless such storage is excluded by the district regulations.

e.  Storage of goods used in or produced by manufacturing activities on the same lot or 			     parcel of ground with such activities, unless such storage is excluded by the district 			     regulations.

f.  Off street motor vehicle parking areas and loading and unloading facilities.

g.  Public utility communications, electric, gas, water and sewer lines, their supports and

incidental equipment.

h. Private swimming pools.

i.  Where a substantial part of the wall of an accessory building is a part of the wall of

the main building or where an accessory building is attached to the main building in a 		    substantial manner as by a roof, such accessory building shall be counted as part of 			    the main building.

5. An accessory building or accessory portion of a principal building shall be included when 	determining Floor Area Ratio but not when determining required area for residential structures.

3.06	BUILDING HEIGHT, BULK AND LOT COVERAGE

No building shall be erected, reconstructed, relocated, or structurally altered so as to have a 	greater height, a higher floor area ratio or smaller open space about it than permissible under the 	limitations set forth herein for the district in which such building is located, except that a 	parapet walls, chimneys, cooling towers, elevator bulkhead, fire towers, stacks, stage towers or 	scenery lofts and necessary mechanical appurtenances shall be permitted to exceed the 	maximum height provisions when erected in accordance with all other ordinances of the 	Village.

No space allocated to a building or dwelling group for the purpose of complying with the side, 	rear, front yard or court or other open space or lot area requirements of this ordinance shall 	thereafter, by reason of change in ownership or for any other building or dwelling group.

An open terrace, but not including a roofed over porch or terrace may occupy a front yard, 	provided the unoccupied portion of the front yard has a depth of not less than fifteen (15) feet. A 	one story bay window may project into a front yard not more than three (3) feet. Over hanging 	eaves, including gutters, may project over the minimum required side yard not more than 	eighteen (18) inches.

3.07	LOT AREA AND DIMENSION

Zoning Ordinance Amendment 2018

12

A. Contiguous Parcels When two (2) or more parcels of land, each of which lacks adequate 	area and dimension to qualify for a permitted use under the requirements of the Use District in

which 	they are located, are contiguous and are held in one ownership, they shall be used as one 	zoning lot for such use.

B. Lots or Parcels of Land of Record. Any single lot or parcel of land held in one ownership 	which was of record at the time of adoption of this amended ordinance, that does not meet the 	requirements for minimum lot width and area, may be utilized for a permitted use, provided that 	yards, courts or usable open spaces are not less than seventy five (75) percent of the minimum 	required dimensions or areas.

3.08	ACCESS TO PUBLIC STREET

Except as otherwise provided for in this amended ordinance, every residential building shall be

constructed or erected upon a lot or parcel of land, which abuts upon a public street unless a 	permanent easement of access to a public street was of record prior to the adoption of this 	amended ordinance.

3.09	NUMBER OF BUILDINGS ON A ZONING LOT

Except in the case of Planned Development, not more than one principal detached residential  	building shall be located on a zoning lot, nor shall a principal detached residential building be 	located on the same zoning lot with any other principal building.

3.10	EXISTING SPECIAL USES

Where a use is classified as a special use under this amended ordinance and exists as a 	permitted use at the date of adoption of this ordinance, it shall be considered as legal use, 	without further action of the Village Board, the Zoning Administrator or the Planning and 	Zoning Commission.

3.11	USES NOT SPECIFICALLY PERMITTED IN DISTRICTS

When a use is not specifically listed in the section devoted to "Uses Permitted" it shall be 	assumed that such uses are hereby expressly prohibited unless by a written decision of the 	Planning and Zoning Commission it is determined that said use is similar to and nor more 	objectionable than use listed.

SECTION 4.00	ESTABLISHMENT OF ZONING DISTRICTS

4.01	CLASSIFICATION OF ZONES

For purposes of this ordinance the following zones are hereby established:

1. Agricultural District (A-1)  The A-1 Agricultural District is established as a zone in which

agriculture and certain related uses are encouraged as the principal land use. The specific intent

Zoning Ordinance Amendment 2018

13

is to facilitate the proper use of lands best suited to agriculture by preventing the mixture of 	urban and rural uses which creates incompatibility and conflict, places unbalanced tax loads on 	agricultural lands to help pay for urban services and contributes to the premature termination of

agricultural pursuits. This zone is also designed to prevent the illogical placement of residential 	use in the otherwise open countryside.

2. Residential Districts.

a. One Family Dwelling District (R1-A). The R1-A zone is established to recognize 			areas presently existing as single family dwellings within the Village. These suggested 			standards recognize the need for providing adequate space for buildings, open areas and 			off street parking and reflecting the time as the traditional Village developed.

b. One Family Dwelling District (R1B). The R1-B zone is established to recognize 			areas proposed for single family residential areas. these suggest current standards 			required for providing adequate space for buildings, open areas and off street parking.

c. General Residence District (R2) The R-2 zone is established to recognize areas 			presently existing or proposed for either single family, two family or multiple family 			dwellings. These suggested standards recognize the need to properly regulated areas for 			higher density residential uses. The standards reflect the corresponding space 				requirements of higher density residential uses.

3. Business Districts

a. Local Business District (B-1)  The B-1 zone is established to permit various retail 			and related commercial uses to be located proximity to one another so as to provide 			goods and services to the local neighborhood

b. General Business District (B-2).  The B-2 zone is intended to accommodate a wider 			variety of related shopper type businesses along with personal services and other 			complementary uses.

4. Manufacturing District (M-1). The zone M-1 zone is established to protect the public 	health, safety, comfort, convenience and general welfare of the community by the proper 	allocation of land for industrial use. It is also the aim of the M-1 zone to protect and promote 	the economic base of the community by regulating manufacturing development in appropriate 		locations.

4.02	ZONING MAPS

The locations and boundaries of the districts established herein are shown upon the Zoning 	Map, which is hereby incorporated into these suggested standards. The Zoning Map, together 	with all notations, references and other information shown thereon and all amendments thereto 	shall be a part thereof and shall have the same force and effect as if the Zoning Map, together 	with all notations, references and other information shown thereon, were fully set forth and 	described herein.

Zoning Ordinance Amendment 2018

14

4.03	ABANDONMENT OF DESIGN DESIGNATED USE

The use area described on the zoning map as for Schools, Parks, Playgrounds and other public 	uses is abandoned or terminated, such area shall automatically be classified as a one family 	dwelling district subject to the regulations and restrictions applicable thereto unless and until its 	classification is changed by amendment to the zoning or ordinance and areas so described on 	the zoning map shall be considered as belonging to the R-1 one family dwelling district subject 	to the use indicated on the zoning map and their use shall comply with all restrictions to R-1 	district, excepting as necessitated by the designed use.

4.04	ZONING OF ANNEXED LAND

Prior to the annexation of any territory to the Village a plan for zoning the area to be annexed 	shall be forwarded to the Village Board by the Planning and Zoning Commission. Upon 	approval of such plan for zoning the area to be annexed, the Village Board shall direct the 	Village Planning and 	Zoning Commission to hold a public hearing in accordance with the 	regulations of the Administration Section herein.

4.05 	BOUNDARY LINES

Whenever any uncertainty exist as to the boundary of any use district as shown on the Zoning 	Map, incorporated herein, the following rules apply:

1.  Where district boundary lines are indicated as following streets, alleys, railroads or similar 		     rights of way, they shall be construed as following the center lines thereof.

2. Where district boundary lines are indicated as approximately following lot lines, such lot 	      	     lines shall be construed to such boundaries.

3. Where a lot held in one ownership and of record at the effective date of these suggested 	    	    standards is divided by a district boundary line, the entire lot shall be construed to be within 	   	    the less restricted district, provided that this construction shall not apply if it increases the 		    area of the less restricted portion of the lot by more than twenty (20) percent.

SECTION 5.00	AGRICULTURAL DISTRICT

5.01	A-1 AGRICULTURAL DISTRICTS

A. Permitted Uses

1. Agriculture. On a lot not less than (40) acres in area which is used specifically for

agricultural purpose as defined herein. Structures and land use for the pursuit of 			   agriculture are not subject to the regulations of this ordinance, except for structures 			   and uses established after the effective date of this ordinance shall conform to the 			   applicable set backs as herein established.

Zoning Ordinance Amendment 2018

15

2. Lot Size. Existing Residential Use- a subdivision, for the purpose of the sale or 			    transfer of ownership of one (1) lot, which contains an existing residential structure, 			    established on or before the date of this ordinance and the lot is not less than (1) acre 			    in area, may be approved by the Plat Officer, with or without review or approval by 			    the Planning and Zoning Commission provided that all other requirements are 				    complied with.

3. Signs. As permitted and regulated in Section 9.00

B. SPECIAL USES

1.  Agricultural implements and machinery sales, services a.nd repair.

2.  Airports or aircraft landing fields.

3.  Animal feed, storage preparation, grinding and mixing, wholesale and retail.

4.  Animal hospitals

5.  Blacksmith or welding shops.

6.  Cemeteries, including crematoriums and mausoleums provided no buildings shall be

located less than one hundred feet from lot line.

7.  Churches, temples, synagogues.

8.  Colleges and universities, including dormitories, sororities and other accessory  			     buildings and structures or trade school

9.  Commercial feeding of fish, poultry and livestock

10. Dog kennels or pounds provided that they are located not less than five hundred 			     (500) feet residential lot line.

11. Golf courses, regulation size but not including "Par 3" golf courses, commercially 			      operated driving ranges or miniature gold courses and provided that no clubhouse or 		      accessory building shall be located less than three (300) fee form a lot line.

12.  Fertilizer sales, including bulk storage and mixing.

13.  Filling holes, pits or low lands with non combustible material free from refuse and 			       food wastes.

14. Fur bearing animal farms, a minimum 1000 feet from any residential or commercial 			      zoned area.

15. Grain elevators and storage.

16. Nurseries and greenhouses, wholesale

17. Gun Clubs, if located not nearer than one thousand (1000) feet to any residence other 		      that that of the owner or lessee of the site and if not so operated as to withdraw the 			     land from its primary agricultural use.

18. Livestock depots and sale yard.

19. Milk processing and distribution, including pasteurizing and manufacture of ice 			      cream but not including the processing or manufacture of cheese.

20. Mining, loading, hauling of gravel, topsoil or other aggregate or minerals including

equipment, buildings or structures for screening, crushing, mixing, washing or for 			      storage,  Provide the Planning and Zoning Commission and approved by the Village 			      Board.

21. Outdoor amusement establishment, fairgrounds, carnivals, circuses, race track and 			      similar amusement centers and including places of assembly devoted thereto such as 		      stadiums and arenas.

Zoning Ordinance Amendment 2018

16

22. Philanthropic and eleemosynary institutions.

23. Private clubs or lodges, except those the chief activity for which is a service 				      customarily carried on as a business.

24. Private recreational areas and camps.

25. Public service sewage treatment plant.

26. Public utility and public services, including : electric substations, repeater stations, 			      micro wave towers and stations, mobile transmitting towers and stations, antenna  			      towers and other outdoor equipment essential to the operation of the exchange in the 		      interest of public convenience and necessity,  water filtration plants, water pumping 			      stations, water reservoir and similar uses.

27. Personal wireless service facilities including towers for personal wireless services 			      provided the towers are self supporting and not more than 120 feet height.

28. Railroad passenger depots.

29. Railroad right of way and track, but not including classification yards, terminal 			      facilities, or maintenance facilities.

30. Rest homes, nursing homes, hospitals and sanitariums, institutions for the aged and 			      for children for human beings only.

31. Riding academies and stables.

32. Sales yards, wholesale or retail, for agricultural products, including but not 				      necessarily limited to fruits, vegetables, flowers, plants etc.

33. Sanitary land fill provided that such lots shall be located not less than 1320 feet

(80 rods) from a zoned residential lot line and when operated or supervised by 			     Kendall County, Lasalle County or a municipality.

34. Seminaries, convents, monasteries and similar religious institutions, including 			      dormitories and other accessory uses required for operation.

35. Truck parking areas, the prime purpose of which is the loading and shipping of farm

products, livestock, poultry, fertilizer, etc.

C. 	Accessory Uses

1. Those customarily accessory to the pursuit of agricultural, provided that structures for 		    the shelter of livestock, poultry and other farm animals shall be located no less than  			    one hundred (100) feet from a front line.

2. Roadside stands for the sale of produce and poultry grown and raised on or i the 			    immediate area of the premises but not including live animals and provided that such 			    stands shall contain not more than six hundred (600) square feet of floor area. Each 			    roadside stand shall have facilities approved by the County Superintendent of 				    Highways, for vehicular ingress and egress and adequate of street parking.

D. 	Building Setback Line. Every building hereafter erected or enlarged shall provide and

maintain a setback from the public street in accordance with the following requirements.

1. Primary thoroughfares, on hundred (100) feet from right of way.

2. All other streets, seventy five (75) feet from the right of way.

Zoning Ordinance Amendment 2018

17

E. 	Side Yards. Each building hereafter erected or remodeled shall have a side yard on each 		side of each building of not less than fifty (50) feet

F. 	Maximum Floor Area Ration for special use shall be established at the time a special 			use permit is granted.

SECTION 6.00	RESIDENTIAL DISTRICTS

6.01 PURPOSE

The residential districts are established in order to protect public health and promote public 	safety, convenience, comfort, morals, prosperity and welfare. These general goals include, 		among others, the following specific purposes.

1. To protect residential areas against fire, explosion, noxious fumes, offensive noise, smoke, 	vibrations, dust, odors, heat, glare and other objectionable factors.

2. To protect residential areas to the extent possible and appropriate in each are against unduly 	heavy motor vehicle traffic, especially through traffic and alleviate congestion by promoting off 	street 	parking.

3. To protect and promote the public health and comfort by providing for ample light and air to

buildings and the windows development.

4. To promote public comfort and welfare by providing for usable open space on the same 	zoning lot with residential development.

5. To provide sufficient space in appropriate locations to meet the probable need for future 	residential expansion and to meet the need for necessary and desirable services in the vicinity of 	residences, which increase safety and amenity for residents and which do not exert 	objectionable influences.

6. To promote the best use and development of residential land in accordance with a 	comprehensive land use plan to promote stability or residential development and protect the 	character and 	desirable development and to protect the value of land and improvements and so 	strengthen the economic base.

6.02 R-1A 	Traditional Single Family Residence District

In the R-1A District the following regulations shall apply:

1. Uses Permitted Outright

a. One family detached dwellings.

b. Home occupations (see definition)

Zoning Ordinance Amendment 2018

18

c. Truck gardening and other horticultural uses where no buildings are involved and 			when no sale of products is conducted on the premises.

d. Transitional uses, principal offices of physicians, dentists, lawyers, architects, real 			estate brokers and other professional occupations, when conducted in a residential  			structure used primarily as a home and when located on lots having a side line adjoining 		a lot in a business district or on lots having as side lot line adjacent to a railroad right of 			way or directly across a street or alley from a business district provided that:

1. The lot on which the transitional uses is located does not extend more than 				seventy five (75) feet from the adjoining business or manufacturing district or 				more than one hundred twenty (120) feet in depth from the street line in cases 				where the lot does not adjoin but faces a business or manufacturing district.

2. The home occupations are conducted in conjunction with the use of a dwelling 			unit as 	a home by the occupant thereof, with not more than one (1) employee 				other than members of the immediate family and the residential character of the 				exterior of the dwelling is not changed. No retail sales shall be allowed.

e. Temporary buildings and uses for construction purposes for a period not to exceed one 		(1) year.

2. Special Uses  In an R-1A District the following uses and their accessory uses may be allowed 	by special use permit in accordance with the provisions of Section 12.00.

a. Churches

b. Convents, monasteries, rectories or parish houses to be occupied by not more than 			fifteen (15) persons.

c. Cemeteries, crematories or mausoleums.

d. Golf courses, public or private.

e. Municipal or privately owned recreation building or community center.

f. Nursery school or day nursery.

g. Off street parking areas and garages.

h. Police station or fire stations

i. Public buildings including post office, library, museum or similar structures.

j. Public or private park or playground.

Zoning Ordinance Amendment 2018

19

k. Public utility facilitates, i.e., filtration plant, water reservoir or pumping station, heat 			or power plant, transformer station and other similar facilities.

l. Schools, elementary, high and college, public or private but not including trade or 			commercial schools operated for profit.

3. Dimensional Standards In an R1-A District the following dimensional standards shall apply:

a. Lot Sizes: Every single family detached dwelling hereafter erected or structurally 			altered	shall be on a lot having an area of not less than nine thousand  (9000) square feet 		and a width at the established building line of not less than seventy five (75) feet.

b. Yard Areas:  No building shall be erected or enlarged unless the following yards are

provided and maintained in connection with such building, structure or enlargement.

1. Front Yard:  Each lot upon which a dwelling is constructed shall a have a

yard of more than thirty (30) feet.

2. Side Yard: On each lot upon which a dwelling is constructed, there shall be a 				side yard on each side equal to not less than (10) percent of the width of the lot in 			in no event not less than seven and one half (7.5) feet.

3. Rear Yard: Every lot or parcel of land upon which a building is constructed

shall have a rear yard of not less than thirty (30) feet.

4. Lot Coverage: (Maximum area) Not more than thirty five (35) percent of the

area of the zoning lot may be occupied by buildings or structures including

accessory  buildings.

5. Permitted Obstruction: The following may be located within or over

required yards:

1. Terraces

2. Awnings and canopies

3. Chimneys, not exceeding three (3) percent of the width of the yard.

4. Steps not over four (4) feet in height.

5. Arbors and trellises.

6. Fences or walls not over six (6) feet in height (rear or side yards only).

7. Breezeways and open porches (rear yards only).

Zoning Ordinance Amendment 2018

20

8. Open parking areas.

6. Dwelling Sizes: Every one (1) story dwelling hereafter erected in any"R1-A" 				One Family District shall have a total ground floor area of not less than one 				thousand (1000) square feet measured from the outside of the exterior walls, 				including utility rooms, but excluding cellars, basements, open porches, 					breezeways, garages and other spaces that are not used frequently or during 				extended periods of living, eating or sleeping purposes.

7. Height of Buildings: The maximum height of buildings permitted shall be as follows:

1. One family detached dwellings: Twenty five (25) feet and not over two and

one half (2 1/2) stories.

2. Church: Seventy five (75) feet for towers or steeples, but not more than

fifty (50) feet for the main structure.

3. Floor Area Ration: The FAR shall not exceed .40 (40%) per cent.

6.03	R-1B Single Family Residence Districts

In the R-1B District the following regulations shall apply:

1. Uses Permitted Outright: In the R-1B District the following uses and their accessory uses

are permitted outright::

a. All uses permitted in the R-1B District.

2. Special Uses:  In an R-1B District the following uses and their accessory uses may be

allowed by special use permit in accordance with the provisions of Section 12.00.

a. All the special uses in an R-1A District

3. Dimensional Standards: In an R-1 district the following dimensional standards shall

apply:

a. Lot Sizes: Lots in subdivisions approved after the effective date of this amendment

shall have an area of not less than fifteen thousand (15000) square feet and a width at

the established building line of not less than seventy five (75) feet.

b. Yard Areas:  No building shall be erected or enlarged unless the following yards are

provided and maintained in connection with such building, structure or enlargement.

c. Front Yard: Each lot upon which a dwelling is constructed shall have a front yard of

not less than thirty (30) feet.

Zoning Ordinance Amendment 2018

21

d. Side Yard: On each lot upon which a dwelling in constructed there shall be a side

yard on each side equal to not less than ten (10) percent of the width of the lot in no

event less than ten (10) feet.

e. Rear Yard: Every lot or parcel of land upon which a building is constructed shall

have a rear yard of not less than forty (40) feet.

f. Lot Coverage:  (maximum area) Not more than thirty five (35) percent of the area of

the zoning lot may be occupied by buildings or structures including accessory buildings,

driveways and other impervious surface.

g. Permitted Obstructions: The following may be located within or over required

yards:

1. Terraces

2. Awnings and canopies.

3. Chimneys not exceeding three (3) percent of the width of the yard.

4. Steps not over four (4) feet in height.

5. Arbors and trellises.

6. Fences or walls not over six (6) feet in height (rear or side yards only)

7. Breezeways and open porches (rear yards only)

8. Open parking areas.

h. Dwelling Sizes:  Every one (1) story dwelling hereafter erected in any "R-1B" One 			Family District shall have a total ground floor area of not less than one thousand seven 			hundred fifty (1750) square feet measured from the outside of the exterior walls, 				including utility rooms, but excluding cellars, basements, open porches, breezeways, 			garages and other spaces that are not used frequently or during extended periods for 			living, eating, or sleeping purposes. Every dwelling of more than one (1) story hereafter 			erected in an "R1-B"

i. Height of Buildings:  The maximum height of buildings permitted shall be as follows:

1. One family detached dwellings:  Twenty five(25) feet and not over two and

one half (2 1/2) stories

2. Church: Seventy five (75) feet for towers or steeples but not more than fifty

(50) feet for the main structure.

Zoning Ordinance Amendment 2018

22

j. Floor Area Ratio: The FAR shall not exceed 35 (35%) percent.

6.04	R-2 GENERAL RESIDENCE DISTRICT

In the R-2 District the following regulations shall apply:

1. Uses Permitted Outright: In the R-2 District the following uses and their accessory uses are

permitted outright

a. Any use permitted in the R-1A One Family Dwelling District

b. Two family dwellings.

c. One family town home dwellings (party wall type) with not more than four (4)

dwelling units in a building.

d. Multiple family dwellings and apartments.

e. Offices of physicians, dentists, lawyers, architects, real estate brokers and other

professional occupations including funeral homes when conducted in a residential

structure providing the residential character of the building is not altered extensively.

f. Boarding and lodging houses.

2. Special Uses: In an R-2 District the following uses and their accessory uses may be allowed by

special use permit in accordance with the provisions of Section 12.00.

a. Any special use permitted in the R1A One Family Dwelling District.

b. Rest homes and nursing homes.

3. Dimensional Standards: In an R-2 District the following dimensional standards shall apply:

a. Height of Buildings: No building shall hereafter be erected  or structurally altered to exceed

three (3) stories nor shall it exceed thirty five (35) feet in height, whichever is less.

b. Lot Size: No building hereafter erected or structurally altered  in this R-2 General Residence

shall have a total lot area of less than six	thousand six hundred (6600) square feet nor have a

total width of less than sixty (60) feet. Every building hereafter erected or structurally altered

as a multiple family dwelling of more than two (2) dwelling units shall provide a lot area per

dwelling unit of one thousand five hundred (1500) square feet. One family town homes

hereafter erected or structurally altered shall have a total lot area of two thousand (2000)

square feet per dwelling unit except that corner and end dwelling units of a row unit building

shall have not less than two thousand five hundred (2500) square feet of total lot area.

Zoning Ordinance Amendment 2018

23

c. Lot Coverage: Not more than forty (40) percent of the area of a lot may be covered by

buildings, structures or other impervious surfaces.

d. Floor Area Ratio: The floor area ration shall not exceed .4 (40%).

e. Yard Areas: No building or structure nor the enlargement of any building or structures shall

be hereafter erected or maintained unless the following yards and lot areas are provided and

maintained in connection with such structure enlargement.

1. Front Yard: For multiple family dwellings other than one family town house

dwellings there shall be a front yard of not less than twenty five (25) feet. For town

house dwellings, two family dwellings and one family dwellings the same regulations

shall apply as required in the R_1 District.

2. Side Yards:

a. Multiple Family Dwellings: On interior lots there shall be a side yard on each

side of the principal building or not less than ten (10) feet and combined total

of side yards of not less than twenty (20) feet for all multiple family dwellings

of not more than two (2) stories in height. On corner lots the side yard on the

intersecting street side shall be not less than twenty (20) feet, except in the

case of a reversed corner lot, where there shall be a side yard on the street side

of the corner lot of not less than fifty (50%) percent of the front yard required

in the lots in the rear of such lot. No accessory building on said reversed

corner lot shall project beyond the front yard line required on the adjacent lot

to the rear nor be located nearer than five (5) feet to the side lot line of such

adjacent lot.

b. One and Two Family Dwellings: on each lot upon which a dwelling is

constructed there shall be a side yard on each side equal to not less than five

(5%) percent of the width of the lot in no event less than five (5) feet.

c. One Family Town Home Dwellings: The same regulations as govern side

yards of multiple family dwelling except that there may not be less than fifteen

(15) feet between adjacent town home buildings.

3. Rear Yards:  For multiple family dwellings other than one family town home

dwellings there shall be a rear yard of not less than twenty five (25) feet for

interior lots nor less than twenty (20) feet for corner lots. For other type dwellings

the same rear yard regulations shall apply as required in the R-1 District.

Section 7.00	GENERAL BUSINESS DISTRICT

Zoning Ordinance Amendment 2018

24

7.01 PURPOSE

The General Business District is established to protect public health to promote public safety,

comfort, convenience and the general welfare and to protect the economic base and value of

property. The general purposes include, among others the following specific objectives.

1. To promote the most desirable use of land in accordance with a well considered plan so that

adequate space is provided in appropriate locations for the various types of business uses,

thereby protecting and strengthening the economic base.

2. To permit selected business uses in districts where adjacent to or inclusion in a residential

area has sufficient elements of service or convenience to such areas to offset the disadvantage

3. To encourage the grouping in appropriate locations of compatible business uses which will

tend to draw trade that is mutually interchangeable and so promote public convenience and

business prosperity and contribute to the alleviation of traffic and pedestrian congestion.

4. To provide for the establishment of, off street parking facilities, permitted and required, so as

to alleviate traffic congestion and so promote shopping convenience and business prosperity.

7.02 	B-1 LOCAL BUSINESS DISTRICT

In the B-1 District the following regulations shall apply:

A. Uses Permitted Outright: In the B-1 District the following uses and their accessory uses are

permitted outright:

1.   Dwelling units provided they are located above the first floor and above a permitted

business use. Dwelling units shall not be permitted on the ground floor or in the rear

business establishments on the ground floor.

2.    Air conditioning and heating sales and service.

3.    Antique shops.

4.   Art and school supply stores.

5.   Automobile service stations.

6.   Bakery shop, including the baking processing of food products, when prepared for

retail use on the premises only.

7.   Banks and financial institutions.

8.   Barber shop, beauty parlor, massage or similar service shop.

9.    Book and stationary stores.

10.  Bicycle sales and repair.

11.  Billiard and poolrooms, bowling alleys, dance halls and gymnasiums.

12.   Building materials sales, when conducted wholly within a building.

13.  Candy and ice cream shops.

14.  Camera and photographic supply shops for retail sale.

15.  Catering establishments.

Zoning Ordinance Amendment 2018

25

16.  Custom dressmaking, millinery, tailoring or shoe repair when conducted for retail

sales on the premises only.

17.  Department stores.

18.  Dry goods stores.

19.  Dry cleaning and pressing establishments, when employing facilities for the

cleaning and pressing of not more than seventeen hundred and fifty (1750)

pounds of dry goods per day and when using non flammable solvents approved

by the Fire Department.

20.  Dry goods store.

21.  Electric appliances store and repair, but not including appliances assembly or

manufacturing

22.  Employment agency.

23.  Exterminating shop.

24.  Feed and seed stores.

25.  Food and fruit stores.

26.  Frozen food stores.

27.  Furniture store and upholstery when conducted as part of the retail operations and

secondary to the main use.

28.  Garden supplies and seed stores.

29.  Gift shops.

30.  Glass cutting and glazing establishments

31.  Hardware stores.

32.  Haberdashery

33.  Household appliance repair.

34.  Jewelry stores.

35.  Laboratories, commercial (medical, dental, research, experimental and testing,

provided no production or manufacturing of products takes place.

36.  Laundromat

37.  Liquor stores and taverns (subject to local regulations).

38.  Locksmith

39.  Meat markets.

40.  News stands.

41.  Offices, business and professional, including medical clinics.

42.  Optician, optometrist.

43.  Paint and wallpaper stores.

44.  Parcel delivery station.

45.  Pet shops, kennels or animal hospitals when conducted wholly within an enclosed

building.

46.  Photograph developing and processing.

47.  Plumbing showroom, heating and roofing supply shops.

48.   Postal Stations.

49.   Printing, publishing and issuing of newspapers, periodicals, books, stationary and

other reading material.

50.  Advertising displays.

Zoning Ordinance Amendment 2018

26

51.  Awnings, Venetian blinds and window shades.

52.  Brushes and brooms.

53.  Electrical equipment appliances.

54.  Jewelry store.

55.  Medical and dental supplies.

56.  Optical goods and equipment.

57.  Products from finished materials such as plastic, bone, cloth, cork, feathers, felt,

fiber, paper, fur, glass, hair, horn, leather, precious or semi precious stone, rubber

shell or yarn.

58.  Scientific and precision instruments.

59.  Public auction rooms.

60.  Public utility collection offices.

61.  Restaurants, tearooms, cafes.

62.  Schools—music, dance business, commercial trade.

63.  Second hand stores and rummage shops.

64.  Shoe stores.

65.  Sporting goods stores.

66.  Undertaking establishments.

67.  Wearing apparel shop.

68.  Any other similar use not specifically listed herein and which has economic 				       compatibility with established uses on adjoining properties

B. Special Uses: In a B-1 District the following uses and their accessory uses may be allowed in

accordance with the provisions of Section 12.00.

1.  Auditorium, stadium, arena, armory, gymnasium and other similar places for public

events.

2.  Bus terminal, railroad passenger station, freight terminal or any other public 				     transportation terminal facilities.

3.  Clinic or medical center.

4. Cemeteries, crematories or mausoleums.

5. Clubs and fraternal organizations.

6. Garages, public for storage of private passenger automobiles and commercial vehicles 		    under one and one half (1 ½) tons capacity.

7.  Golf courses, public or private.

8.  Hospitals or sanitariums, public or private.

9.  Hotels, including dining and meeting rooms, when business uses occupy the street 			     frontage except for an entrance way to the hotel lobby.                                               			10. Meeting halls, lodge halls, fraternal organizations and clubs provided they are 			      in a basement or above the first floor and above a business use permitted in this 			      section or said uses may be located on the ground floor when permitted business 			      establishments occupy street frontage except for an entrance way to the rear use.

11.  Municipal or privately owned recreation building or community center.

12.  Nursery school or day nursery.

Zoning Ordinance Amendment 2018

27

13.  Police station or fire station.

14.  Public buildings including post office library, museum or similar structures.

15.  Public or private park or playground.

16.  Publicly or privately owned and operated fairgrounds, permanent carnivals,  “kiddie 		        parks” or other similar amusement centers.

17.  Silver plating and repair shop.

18.  Theater, indoor.

19.  Used passenger automobile sales (used car lot) on an open lot or within a building.

C. Dimensional Standards: In the B-1 District the following dimensional standards shall apply:

1.  Height of Building:  No building or structure shall be erected or structurally altered 			to exceed a height of three stories nor shall it exceed thirty five (35) feet in height. 			Parapet walls, chimneys, cooling towers, elevator bulkheads, stacks and necessary 			mechanical appurtenances may be erected over and above the maximum height of forty 			five (45) feet, provided they are constructed in accordance with all the ordinances of the 		municipality.

2.  Lot Coverage:  (Maximum area) Not more than fifty (50%) percent of the area of the 		zoning lot may be occupied by buildings or structures, including accessory buildings.

3.  Floor Aria Ratio:  The Floor Area Ratio shall not exceed 0.5 (50%)

7.03	B-2 GENERAL BUSINESS DISTRICT

The B-2 General Business District is intended to accommodate a wide variety of related 	shopper type businesses along with personal services and other complementary uses.

1. Permitted Uses:

The following uses are permitted:

a. Uses permitted in the B-1 Local Business Districts

b. Agricultural implement sales and service.

c. Auto accessory store, where there is no driveway entrance across the side walk into

the principal building.

d. Motorcycle sales and repair.

e.  Contractors offices and shop. Where no fabricating is done on premises and where all 		     storage of material is within a building or enclosed area.

f.  Food processing, packaging and distribution.

g.  Greenhouse, wholesale growers.

Zoning Ordinance Amendment 2018

28

h.  Wholesale business, excluding a building the principal use of which is for a storage 				     warehouse.

i.  Processing or assembly limited to the following, provided that space occupied in a 			     building does not exceed six thousand (6000) square feet of total floor and basement 			    space not including stairwells or elevator shafts and provided such processing or 			    assembly can be conducted without noise, vibration, odor dust or any other condition 			    which might be disturbing to occupants of adjacent buildings. When manufacturing

operations of the same or similar products demand space exceeding six thousand 			    (6000)  square feet, they shall then be located in the M-1 Manufacturing District.

j.  Any other similar use not specifically listed herein and which has economic 				    compatibility with established uses on adjoining properties.

2. SPECIAL USES: In a B-2 District the following uses and their accessory uses may be allowed in accordance with provisions of Section 12.00.

a. Uses permitted as Special Uses in a B-1 Local Business District

b. Bakeries, wholesale

c. Battery and tire service stations.

d. Automobile sales and service, but not including automobile body repair and rebuilding or 	painting of automobiles.

e. Automobile and truck (under on and one half (1 ½) ton capacity) minor motor repair and 	service shop, bu not including body repair and rebuilding or painting.

f. Automobile washing, including the use of mechanical conveyors, blowers and steam cleaning 	equipment.

g. Pattern Making

h. Personal wireless service facilities including towers for personal wireless services provided 	the towers are self supporting and not more than one hundred twenty (120) feet in height.

i. Radio and television broadcasting stations.

Dimensional Standards. In the B-2 District the following dimensional standards shall apply:

a. Height of Building: No building or structure shall be erected or structurally altered to 		    exceed a height of three (3) stories nor shall it exceed thirty five (35) feet in height. 			    Parapet walls, chimneys, cooling towers, elevator bulkheads, stacks and necessary

Zoning Ordinance Amendment 2018

29

mechanical appurtenances may be erected over and above the maximum height of 			 forty five (45) feet, provided they are constructed in accordance with all the 				 ordinances of the municipality.

b. Lot Size: No building hereafter erected or structurally altered in this B-2 General 			    Business District shall have a total lot area of less than thirty thousand (30,000) square 		    feet nor have a total lot width of less than one hundred fifty (150) feet.

1. Lot Coverage: Not more than forty (40%) percent of the area of a lot may be 				    covered by buildings or structures.

2. Floor Area Ratio:  The floor area ratio shall not exceed .50 (50%) percent.

3. Yard Areas: No building or structure nor the enlargement of any building or 				    structures shall be hereafter erected or maintained unless the following yards 				    and lot areas are provided and maintained in connection with such structure or 			    enlargement.

i. Front Yard:  Front yard in the B-2 General Business District shall be a 				minimum of twenty (20) feet. If parking is permitted in the front yard 					there shall be a minimum building setback of sixty five (65) feet of which 				a minimum of five (5) feet shall be landscaped with bushes, trees and/or 					ground cover.

ii. Side Yards: there shall be a minimum side yard of twelve (12) feet. If 					    two buildings are located on the same lot there shall be a minimum 					    separation of ten (10) feet.

iii. Rear Yard: There shall be a minimum twenty (20) foot rear yard.

8.00	M-1 MANUFACTURING DISTRICT

8.01 PURPOSE

The purpose of the industrial district is to provide for the establishment of a range of industrial and allied activities and to govern their operations in a manner that will not have a deleterious effect on the agricultural, residential and businesses in the area.

In the M-1 District the following regulations shall apply:

A. Uses Permitted Outright: In an M-1 District the following uses and their accessory 			uses are permitted outright:

1. Any use permitted in a business district but not including residences or 				apartments, except such dwelling accommodations as may be needed to house a 				care taker or watchman and their family employed on the premises.

Zoning Ordinance Amendment 2018

30

2.  Automobile and truck repair (major), painting, upholstery, reconditioning and body 			     and fender repairing when done within the confines of a structure.

3.  Batteries, manufacture and rebuilding.

4.  Bed spring and mattress manufacture.

5.  Beverage, non alcoholic, bottling and distributing.

6.  Brooms and brush manufacture.

7.  Boat building and repair.

8.  Building equipment yards, yards for building materials, lumber, coal, sand and 			      gravel yards, yards for contracting equipment, maintenance or operating equipment 			     of public agencies, or public utilities, or materials or equipment of similar kind.

9.   Bus line shops and garages.

10. Canning and preserving.

11. Canvas and canvas products manufacture.

12. Carpet manufacture

13. Cartage (local), express hauling or storage yard.

14. Cement block manufacture.

15. Ceramic products, pottery and glazed tile manufacture.

16. Chick hatcheries.

17.  Coated fabrics, except rubberized manufacture.

18.  Creameries and dairies.

19.  Drapery and bedding manufacture.

20.  Drugs and pharmaceutical products manufacture.

21.  Electric motors and generators manufacture.

22.  Glass products, from previously manufactured glass.

Zoning Ordinance Amendment 2018

31

23.  Heating appliances and sheet metal products, including stove and ranges 				       manufacture.

24. Ice cream and ice manufacture.

25. Kennel and outdoor pens and runs.

26. Machine shops and metal products manufacture, when not equipped with heavy 			     (exceeding 50 ton pressure) punch presses, drop forges, riveting and grinding 				      machines or any other equipment which may create noise, vibration, smoke, odors, 			      heat, glare or fire hazards.

27. Metal polishing and plating.

28. Motor freight terminal, private.

29. Plastic products but not including the processing of the raw material, manufacture.

30. Public utility electric substations and distribution centers, gas regulation centers, 			      underground gas holder stations.

31. Shoes and boots manufacture.

32. Storage of household goods.

33. Storage and sale of trailers, farm implements and other similar equipment on an open 		      lot.

34. Sporting and athletic equipment manufacture.

35. Storage of flammable liquids, fats or oil in tanks, each of 50, 000 gallons or less 			      capacity bu only after the location and protective measures have been approved by 			     local governing officials.

36. Tools and hardware such as hand tools, bolts, nuts, screws, cutlery, house hardware, 			      and plumbing appliances, manufacture.

37. Tool and die shop.

38. Truck terminal, including exchange and handling of freight.

39. Toys and children’s vehicles, manufacture.

40. Wire brush manufacture.

Zoning Ordinance Amendment 2018

32

41. Any other manufacturing establishment that can be operated in compliance with the 			      requirements of this section, with creating objectionable noise, odor, dust, smoke, 			     gas, fumes, or vapor and that is a use compatible with the use and occupancy of     			     adjoining properties.

B. Special Uses: In an M-1 District the following uses and their accessory uses may be allowed by a special use permit in accordance with provisions of Section 12.00

1. Any conditional uses permitted in the General Business District except clinic or 			    medical center, hospital or sanitarium, public or private.

2. Extraction of gravel, sand or other raw materials.

3. Radio and television towers commercial.

C. Dimensional Standards: In an M-1 District the following dimensional standards shall apply:

1. Height of buildings: No building or structure shall be erected or structurally altered 			 to exceed a height of four (4) stories nor shall it exceed thirty five (35) feet in height.

2. Area:  No building with its accessory buildings to be used commercial or  				manufacturing purposes shall occupy more than seventy (70%) percent of the area of a 			lot. Where the property or properties within a manufacturing district existing at the time 			of adoption of this ordinance have frontage withing limits of a block occupied by two or 		more commercial or manufacturing structures, there shall be no front yard and no side 			requirements.

3. Yard Areas:

a. Front Yards: There shall be a front yard having a depth of not less than thirty 				    (30) feet wherein there shall be no structure of any kind open storage of 			 	    materials or equipment or the parking of vehicles. Where established building 				    line is less, the established building line governs the front yard.

b. Side Yards: There shall be a minimum side yard of not less than fifteen (15) 				    feet on both sides of the building or buildings, but where the property is 	   			    adjacent to a residence district there shall be a side yard of not less than twenty 			    five (25) feet on the side nearest to the residential lots. The parking of private 				    automobiles may be permitted within the side yard areas, but not closer than 				    five (5) feet to any lot zoned for residential use.

c. Rear Yards:  A rear yard is not required except where a lot abuts upon a 				district, in which case there shall be a rear yard of not less than thirty five (35) 				feet and no storage of equipment or the parking of automobiles shall take place 				within the fifteen (15) feet closest to any residential lot or lots.

Zoning Ordinance Amendment 2018

33

D. Performance Standards: Any use established in the M-1 Manufacturing District after the effective date of this comprehensive amendment shall be so operated as to comply with the performance standards set forth hereinafter. No use lawfully established on the effective date of this comprehensive amendment shall be so altered or modified as to conflict with or further conflict with the performance standards established hereinafter for the M-1 District.

1. Noise: The sound pressure level, to be measured as described below shall not exceed the following decibel levels in the designated octave bands when adjacent to the designated types of districts:

Octave Band Cycles per Second		Residential Districts		Sound Level In Decibels B

0-75						58				           73

75-150					            54				           69

150-300					            50					65

300-600					            46					61

600-1200					            40					55

1200-2400					            33					48

2400-4800					            26					41

over 4800					            20					35

Objectionable sounds of an intermittent nature which are not easily measured shall be controlled 	as not to become a nuisance to adjacent uses.

2. Method of Measurement: Measurement is to be made at the nearest boundary of the nearest

residential area or at any other point along the boundary where the level is higher. The sound 	level shall be measured with a sound level meter and associated octave band filter as prescribed 	by the American Standards Association.

3. Odors: No overly objectionable or hazardous odor shall be emitted by any use permitted n 	this district in such  quantities as to be readily detectable by an average observer at any point on 	the boundary line of the premises or beyond.

4. Noxious Gases: Processes and operations of permitted uses, capable of dispersing gases or 	toxic particulates  in the atmosphere must be hooded or otherwise suitably enclosed. The 	omission of such toxic gases or particulate matters shall be from a stack or in such a manner 	that they create no hazard to person or property.

5. Glare and Heat: Operations producing intense light or heat shall be performed within an 	enclosed building and not be visible beyond any lot line bounding the property whereon the use 	is conducted.

6. Vibration: No uses shall be located and no equipment shall be installed in such a way as to 		produce intense, earth shaking vibrations which are noticeable at the property lines of the 	subject premises.

Zoning Ordinance Amendment 2018

34

SECTION 9.00	SIGNS

9.01	 PURPOSE AND INTENT

The provisions of this section are intended to provide for the necessary means of identification while protecting the value of adjoining properties and maintaining a safe and pleasing environment of the residents of Millington.

9.02	DEFINITIONS

The following word, terms and phases, when used in this chapter shall have the meanings ascribed to them in this section, except where the context  clearly indicates a different meaning:

1.  Canopy and awning, fixed means nay hood, canopy or awning made of cloth, other flexible material or metal with metal frames attached to a building and carried by a frame either supported or not supported from the ground or sidewalk.

2.  Integrated Center means a building, series of attached buildings or separate buildings under the single ownership or management or having contractual agreements between the separate buildings’ owners.

3.  Canopy and awnings, retractable, means any hood, canopy of awning made of cloth or other flexible material or metal with metal frames attached to a building and so erected as to permit its being retracted when not in use.

4.  Marquee means any fixed hood, constructed of metal or other incombustible material, supported solely by and extending from the building to which it is attached.

5.  Sign means a name, identification, description, display or illustration which is affixed to or painted on or represented directly or indirectly upon a building, structure, billboard or parcel of land and which directs attention to an object, product, place, activity, person, institute or organization or business. Each display surface of sign shall be considered to be a separate sign. The word “sign” does not include the following:

a.  Any sign of the United States, state or municipal corporation

b. The flag, emblem of insignia of a nation, governmental unit, school or religious group.

c. Signs located completely within an enclosed building unless affixed to a window of the 	  	    building, provided however that the provisions of this chapter regulating flashing and moving 	   signs shall apply to all such signs located completely within an enclosed building, if such 	   	   signs are visible from the public property.

d.  Paper signs affixed to the interior of the window of a building for a period of less than thirty  	     (30 	) days.

Zoning Ordinance Amendment 2018

35

e.  Identification and address of establishments on awning or marquees.

f.  Street or house number signs not exceeding one and one half (1 ½) square feet.

g.  Directional signs or instructional signs conveying directions or instructions for the safety or 		     of the public (vehicular or pedestrian) or necessary for the operation of the premises and not 	     	     exceeding three (3) square feet in area:

h.  A barber pole or similar symbol or device used to identify and display in connection with a 	   	     barber shop.

6. Sign advertising means a sign which directs attention to a business, commodity, service or entertainment conducted, sold or offered upon premises other than where such sign is located or to which it is affixed.

7. Sign, animated means any sign that uses movement or change of lighting either natural or artificial, to depict action or create a special effect or scene.

8. Sign, architect contractor means a sign representing a business or firm hired to make capital improvements to r financing of a structure or property on a parcel of land upon which the sign is being displayed.

9. Sign business means a sign which directs attention to a business or profession conducted or to a commodity, service or entertainment conducted, sold or offered upon the premises where such sign is located or to which it is affixed.

10. Sign, changeable copy/reader board means a sign or portion thereof, with characters, letters or illustrations that can be changed or rearranged with out altering the face or the surface of the sign. A sign of which the message changes more than eight times per day will be considered an animated sign and not a changeable copy sign for purposes of this ordinance. A scrolling light message portion of a sign must comply with the message change limit. A sign on which the only copy that changes is an electronic or mechanical indication of time or temperature shall be considered a “time and temperature” portion of a sign and not a changeable copy sign for purposes of this ordinance. All messages for the changeable copy/reader board signs shall be limited to the business to which the sign is attached and/or public and/or public service announcements.

11. Sign, flashing means any illuminated sign on which the artificial light is not maintained stationary or constant in intensity and color at all times when such sign is in use. For the purpose of this chapter,, any moving illuminated sign except scrolling light messages on a changeable copy/reader board sign shall be considered a flashing sign.

12. Sign, gross surface area of, means the entire area covered by the sign consisting of the areas comprised within a single continuous perimeter enclosing the extreme limits of it.

13. But such perimeter shall not include any structural elements lying outside the display area of sign.

Zoning Ordinance Amendment 2018

36

14. Sign, ground base or monument means a sign supported by uprights, columns or braces placed in the ground and not attached to any building. The base of all ground base or monument signs shall be no more than two (2) feet above grade and not to exceed six (6) feet above grade in height.

15. Sign, historical plaque means a commemorative plaque by recognized federal, state or local historical agency depicting and identifying a buildings’ contribution to the field of architecture, or a building or sites contribution to the life of an individual or as being representative of a historical event

or era of development in the village. Such plaque shall not exceed four (4) square feet in area and shall be affixed flat against a building or if a vacant site, shall be permanently mounted on a pole or other suitable support.

16.  Sign, pole means a sign mounted or otherwise attached, to a steel column or columns or other suitable material placed in the ground, the bottom of which is no less than eight (8) feet above grade. Pole signs shall conform to the restrictions in Table 9-1. Also known as a pylon sign.

17. Sign, political means a sign representing an individual or party running for an elected office for which the individual or party has duly filed for elected office or is pursuing a write in campaign. Signs concerning a referendum or an issue of retention shall also be considered political signs.

18. Sign real estate is a sign which advertises property of buildings for sale, rent or lease by the owner of the property or a property licensed agent in the sale of such property.

19. Sign, temporary means a sign banner, bunting, pennant, valance or advertising display to be displayed for a period not to exceed forty five (45) days.

20. Sign, wall means a sign attached, applied to, painted or placed flat against the exterior front, rear or side wall of any building, with exception in section 9.05.2

21. Sign, window means signs, regardless of material, which are on the inside of the window.

9.03	ADMINISTRATION

It shall be the responsibility of the Zoning Administrator or his appointee to administer this chapter.

9.04	VIOLATIONS AND ENFORCEMENT

It shall be unlawful to display any sign in violation of the provision of this chapter. Each day of such violation shall constitute a separate offense  under this chapter. Any person who owns or manages a building or premises and any occupant thereof who displays a sign shall comply with the provisions of this chapter. The Zoning Administrator is hereby designated and authorized to enforce this chapter.

9.05 	PERMITTED AND NON PERMITTED SIGNS

1. The following types of signs only are permitted signs within the standards set out in this chapter:

a. Ground base or monument signs.

Zoning Ordinance Amendment 2018

37

b. Wall signs.

c. Pole signs, in business zoning districts only, subject to the restrictions in Table 9-1.

d. Temporary signs

e. Marquees

f. Fixed canopies and awnings.

g. Retractable canopies and awnings.

h. Window signs.

i. Historical signs.

j. Changeable copy and reader board signs.

2. Non Permitted Signs.

a. Sign on fences or on walls which are not a part of a building or structure.

b. Flashing signs, which are illuminated signs (whether stationary, revolving or rotating) which 	contain any flashing lights or lights creating an illusion of movement, except those signs which 	display time and temperature information only, or a scrolling light message subject to the 	limitations contained in section 9.02 Difinitions.

c. Moving signs, which are signs which flutter, undulate, swing, rotate, oscillate or otherwise 	move by natural or artificial means. Pennants and banners are specifically included within the 	definition of moving signs, with an exception sited in section 9.11.

d. Advertising signs except temporary subdivision advertising signs as provided in section 9.10.

e. Signs painted on building walls which face or are visible from a public street.

f. Signs in direct line of vision of any traffic control signal shall not have illumination of red, 	green or amber color.

g. Roof signs.

h. Billboards and poster boards.

i. Signs which contain statements, words or pictures of obscene, indecent, or immoral character 	and which offend public morals and decency.

j. Any sign not included as a permitted sign

9.06	SIGNS ON RESIDENTIAL PROPERTIES

No sign shall be displayed on buildings or premises, or that portion thereof used for residential purposes, regardless of the zoning district in which it is located, or on any vehicles parked on such property for more than twenty four (24) hours where such vehicle is readily visible to the general public, except for the following permitted signs.

1. One nameplate sign not exceeding one hundred forty four (144) square inches in area, which 	may be combined with a street or house sign.

2. Non commercial signs behind or affixed to windows or doors.

3. In the case of apartment houses for more than two families, there shall be permitted, in 	addition to the foregoing and in addition to any other signs permitted by reason of any 	commercial use of the first floor, one identification sign not exceeding five (5) square feet in 	area indicating the name of the building and/or the ownership of management of such building.

4. Temporary signs permitted under section 9.09

Zoning Ordinance Amendment 2018

38

9.07	SIGNS OF CERTAIN CHARITABLE, EDUCATIONAL AND SIMILAR 	ORGANIZATIONS.

1. No sign shall be displayed on the building or premises of a private educational, philanthropic, 	 or charitable institution or organization or any private club, except for temporary signs 	permitted under section 9.09 and except for the following permitted signs:

a.  Identification signs, identifying the name and/or nature of the institution or 				organization.

b. Bulletin board signs, being structures of a permanent nature but having changeable 			words or figures.

2. The total area of all signs on premises of each such institution or organization shall not 	exceed fifty (50) square feet in area and no sign shall exceed thirty (30) square feet in area.

9.08	BUSINESS SIGNS

Business signs of any type not prohibited by section 9.05 may be displayed on any property used for business of manufacturing purposes, subject to the following limitations and regulations:

1. Where a sign is illuminated by light reflected upon it, direct rays of light shall not beam upon 	any part of an existing residential building nor into a residential district or into a street. A sign 	located in direct line of vision of a traffic signal shall not have red, green or amber illumination.

2. The total permitted gross surface area of all signs for a single building shall not exceed two 	and half (2 ½)  times in square feet the total building frontage expressed in linear feet, up to a 	maximum of five hundred (500) square feet for a building with one frontage, eight hundred 	(800) square feet for a building with two (2) frontages and one thousand (1000) square feet for a 	building with three or more frontages. If a single building is designed for occupancy by more 	than one business use and is so used, the formula to be applied in ascertaining the permitted 	gross surface area for each individual establishment shall be based on the formula set out in this 	subsection. The owner of the building shall then be responsible for negotiating with all tenants 	in the building, the amount of signage allowable for each business.

3. No wall sign shall project more than twenty four (24) inches into a public right of way and 	any such sign shall be limited in height to the height of the building or twenty (20) feet above 	grade at the base of the building, whichever is lower.

4. Any permanently installed sign entirely separate from any structure, located entirely upon 	private property and not projecting into a public right of way and otherwise in conformance 	with all requirements of this chapter shall conform to Table 9.1

5. Signs which are not embodied as part of the soft or pliable material of an awning or canopy 	which project from a building in excess of six (6)  inches or over a public right of way shall 	have the lowest portion no less than nine (9) feet above the grade, immediately below the sign.

Zoning Ordinance Amendment 2018

39

6. A unified shopping center or integrated center in single ownership or control may erect only one pole sign to server the shops therein in addition to the individual signs allowed by this chapter. Such signs shall conform to the standards in Table 9. 1.

9.09	TEMPORARY SIGNS

In addition to signs otherwise permitted by this chapter, the following temporary signs shall be permitted, so long as such temporary signs are otherwise in conformity with this chapter:

1. Real estate signs (non illuminated) advertising the sale or lease of the premises on which they are maintained, not exceeding ten (10) square feet in area and not in excess of one (1) sign per lot or premises, except that on corner lots one (1) such sign may face the street. Such signs shall be removed immediately upon sale or lease of premises. Said signs must be located on private property.

2. Architect contractor signs.  One sign of architect and/or contractor not exceeding ten (10) square feet in area may be displayed on a premises located in any residential district established by the Village Zoning Ordinance, as amended and one such sign not exceeding twenty (20) square feet on premises located in any business or manufacturing district established by the Village Zoning Ordinance where new construction or remodeling is in progress, to be removed immediately upon completion of such work. In an area zoned for commercial or industrial use where the property on which construction or remodeling is in progress,  one contractors or architect’s sign may be erected on each street frontage said property abuts. Such signs may not be located closer than eight (8) feet from the property line or one half (½)  the distance between the property and the building lines, whichever is less.

3. Decorations displayed in connection with civic, patriotic or religious holidays.

4. Flags, emblems and signs of political, civic, philanthropic or educational organizations temporarily displayed for non commercial purposes. Said signs must be located on private property unless placed by federal, state or municipal government.

5. Political signs shall be permitted in any zoned district as long as such signs do not obstruct traffic or create a hazardous condition. The total surface area of the total of all political signs on a zoning lot shall not exceed one hundred (100) square feet. With seven (7) days following an election, political signs shall be removed. In the event signs are still displayed seven (7) days following an election the Village shall remove said signs. Political signs shall not be permitted on any right of way.

6. Signs are permitted which advertise for sale agricultural produce grown on the same premises whereon such signs are placed on personal property owned by the resident of said premises, provided such signs shall not be larger than ten (10) square feet in area, that no such sign shall advertise produce grown elsewhere or personal property purchased for the purpose of resale on said premises and that no more than two (2) such signs shall be allowed on any premises nor allowed to remain in place longer than sixty (60) days, except as anything herein prohibited shall be permitted by another section of the Ordinance.

Zoning Ordinance Amendment 2018

40

9.10	TEMPORARY SUBDIVISION ADVERTISING SIGNS

The Zoning Administrator, when requested in writing, may mitigate the provisions of this chapter regarding advertising signs on a temporary basis only in accordance with the following provisions for the purpose of encouraging rapid and competitive sale of new subdivision development within the village.

1. Temporary means an indefinite length of time which shall terminate upon sale of eighty 	(80%) percent of all available parcels included in the platted subdivision.

2. Arterial route means all routes indicated as arterial routes in the Village of Millingtons’ 	Comprehensive Land Use Plan and all subsequent amendments.

3. Onsite allowance. One sign for each direct arterial route frontage which may be advertising 	as well as directional. The surface area cannot exceed one hundred (100) square feet and twelve 	(12) feet in height above grade. Such signs are subject to fees as set forth in section 9.20.3.

4. Off site allowance. Available to subdivisions which lack direct arterial route frontage. One 	sign shall be used for any one subdivision for any one arterial route in the immediate vicinity of 	logical traffic service to the subdivision. Only private property fronting such arterial route may 	be so used, must be directional only and the surface area cannot exceed one hundred (100) 	square feet and twelve (12) feet in height above grade. Such signs are subject to fees as set forth 	in section 9.20.3.

5. Public right of way. One sign shall be used for any one subdivision for any one arterial route. 	Only arterial routes in the immediate vicinity and of logical traffic service to the subdivision 	may be so used. Such signs must be directional only and the surface area cannot exceed two (2) 	feet in length and two (2) feet in height. The placement of such signs on right of ways not 	controlled exclusively by the village shall be at risk of the subdivision. The total number of such 	signs shall be limited to three (3) per subdivision.

6. Authorization. All temporary subdivision sign location requests shall be reviewed by the 	Zoning Administrator.

7. Removal. All permitted subdivision signs shall be removed when required by this section.

9.11	PENNANTS OR BANNERS

The Zoning Administrator, when requested in writing may mitigate the provisions of this 	chapter prohibiting pennants and banners, but on a temporary basis only in accordance with the 	following provisions:

1. Definition Temporary means a period of time not in excess of forty five (45) days.

Zoning Ordinance Amendment 2018

41

2. Grand Openings, special sales. The use of pennants and banners may be allowed by the 	Village Clerk in connection with grand openings of commercial establishments and in support 	of special sales or events or other matters of similar importance.

3. Authorization. Authorization for the use of temporary pennants and banners shall be 	authorized by the Village Clerk and shall state the number of days such use is to be permitted. 	No such authorization shall be given more than twice in a calendar year for a specific location.

4. Removal. All such pennants and banners shall be removed at the expiration of such 	authorization.

9.12	CONSTRUCTION REQUIREMENTS

1. Date of erection, name on permit and voltage to be on sign. Every sign hereafter erected shall 	have painted in a conspicuous place thereon, in legible letters, the date of erection, the name on 	the permit and the voltage of any electrical apparatus used in connection therewith.

2. Wind pressure and dead load requirements. Any sign shall be designed and constructed to 	withstand a wind pressure of not less than thirty (30) pounds per square foot of net surface area 	and shall be constructed to receive dead loads as required in the building code and other 	ordinances of the Village.

3. Fireproof construction. No sign, the majority of the display area of which is within four (4) 	feet of any building or which is greater than eighteen (18) square feet in area shall be 	constructed at any location within the Village fire limits unless the face is constructed of 	noncombustible material.

4.  Obstruction to doors, windows and fire escapes No sign, retractable canopy or awning shall 	be erected, relocated or maintained so as to prevent free ingress to or egress from any window 	or fire escape. No sign, retractable canopy or awning of any kind shall be attached to a 	standpipe or fire escape.

5. Sign not to constitute traffic hazards.  In order to obtain and secure reasonable traffic safety, it 	shall be unlawful for any person to erect or maintain any sign, retractable canopy or awning, in 	such a manner as to obstruct free and clear vision or as to distract the attention of the driver of 	any vehicle by reason of the position, shape, color or intensity thereof. Pursuant to the 	foregoing, no sign, retractable canopy or awning shall be erected or maintained in such manner 	as to likely to interfere with, obstruct the view of or be confused with any authorized traffic 	sign, signal or devise. Accordingly no sign, retractable canopy or awning shall make use of the 	words “stop”, “go”, “look”, “slow”, “danger”, or any other similar word, phrase, symbol or 	character, or employ any red, yellow, orange, green or other lamp or light in such a manner as to 	interfere with, mislead or confuse traffic.

6. All monument and pole signs shall have a landscape area at the base of the sign equal to a minimum of hundred (100%) percent of the total surface area of the sign.

Zoning Ordinance Amendment 2018

42

9.13	INSPECTIONS

The building inspector shall make inspections of signs as he deems necessary and for this purpose has the right during business hours to enter upon the premises where a sign is maintained. If it is found that a sign is out of repair, unsafe, or nonconforming to this chapter, the owner shall be notified in writing of the condition and if the owner does not remedy the sign within thirty (30) days from the time of the serving of the notice, the building inspector shall act in accordance with section 9.17.

9.14	NUISANCES

Any sign displayed in violation of this chapter is hereby deemed a public nuisance. Any sign, retractable canopy or awning which is immediately dangerous to persons or property in its existing condition or which is declared to be a hazard by the village is hereby declared to be a nuisance and may be abated as such.

9.15	MAINTENANCE

All signs, retractable canopy or awnings shall be kept and maintained in a safe, neat and orderly condition and appearance, and shall be repainted or otherwise maintained periodically by the permit holder  to prevent corrosion or deterioration caused by weather or age and to keep the same in safe, neat and orderly condition and appearance.

9.16	REMOVAL OF CERTAIN SIGNS

Any sign, retractable canopy or awing now or hereafter existing, which advertises a business no longer being conducted, or a product no longer being sold, shall be taken down and removed by the owner, agent or person having the beneficial use of the building, structure or premises upon which such sign may be found, within thirty (30) days after receipt of written notice by the village to so act. Upon failure to comply with such notice within the time specified in such order, the building inspector is hereby authorized to cause removal of such sign. Any expense incurred shall be paid by the owner, agent or person having beneficial use of the building, structure or premises to which such sign is attached.

9.17	 UNSAFE AND UNLAWFUL

If the building inspector shall find that any sign, retractable canopy or awning is unsafe or insecure or is a menace to the public, has been constructed or erected, or is being maintained in violation of the provisions of this chapter he shall give written notice of such findings to the permit holder of the same. If the permit holder fails to remove or alter the sign, retractable canopy or awning so as to bring it into compliance with standards established by this chapter within thirty (30) days after receipt of such notice, such sign, retractable canopy or awning may be removed by the village at the expense of the permit holder. If upon reasonable and diligent search the permit holder cannot be found so that the notice described in this section cannot be given, then the village may remove the sign, retractable canopy or awning or, in the alternative, may send written notice of the building inspectors finding to the owner of the building or structure of premises. If the owner then fails to remove or alter the sign,

Zoning Ordinance Amendment 2018

43

retractable canopy or awning so as to bring it into compliance with the standards established by this chapter within thirty (30) days after receipt of such notice, such sign, retractable canopy or awning may be removed by the village at the expense of the owner. The village may cause any sign, retractable canopy or awning which the building inspector finds to be an immediate peril to persons or property to be removed summarily and without notice.

9.18	EXEMPTIONS AND ZONING RESTRICTIONS

The provisions of this section regulating the location, placement, gross surface area, projection, height limitation and number of signs or other advertising structures shall be subject to and may be further restricted by any applicable provision of the present or hereafter adopted zoning ordinance of the village including provisions therein relating to nonconforming structures and uses.

9.19	BUILDING AND ELECTRICAL CODES

All signs erected or constructed shall conform to all technical and structural requirements to the village building and electrical codes.

9.20	PERMITS AND FEES

1. A permit shall be obtained from the building inspector or written application thereof, by any 	person seeking to construct, erect or maintain any sign over three (3) square feet in area., except 	for temporary nonstructural signs. Such application shall be accompanied by plans and 	specifications which show the size, character, material, location, bracing, anchorage, support or 	means of attachment and other structural characteristics thereof and of its frame and mounting. 	Prior to the issuance of a permit, the application plans and specifications shall be approved by 	the building inspector.

2. No permit shall be issued if the building inspector shall determine that the sign will constitute 	a hazard to traffic or the public by reason of obstruction of view, distraction or endangering the 	safety of persons using public property.  Any permit required elsewhere in this Code for 	electrical work in connection with any sign must be obtained in addition to the sign permit 	provided for in this section.

3. The fee to be charged for each permit issued pursuant to this chapter shall be twenty five 	($25) 	dollars plus twenty (.20) cents per square foot of the gross surface area of a sign. The fee 	for any re inspection of the sign shall be thirty ($30) dollars.

4. Upon certification that a requested sign is in conformance with all the requirements and 	restrictions of this chapter, the sign is located within a business or manufacturing/industrial 	zoning district and all fees have been paid, the building inspector shall issue a permit for the 	erection or placement of the requested sign.

Zoning Ordinance Amendment 2018

43

9.21	VARIATIONS

Upon an application or permit to construct, alter or maintain any sign which does not conform to the regulations of this chapter, upon a finding by the Village Board that a particular hardship does exist, the Village Board may grant a variation in the application of the regulations of this chapter but in harmony with the general purposes of intent of this chapter. In no instance shall a variation greater than ten (10%) percent above the allowance permitted in section 9.08.2 or table 9-1 be granted by the Village Board unless such sign is determined to have artistic, historic, or architectural significance.

9.22	LEGAL NONCONFORMING SIGNS

Signs which were erected on or before the enactment of this ordinance and no longer conform shall be subject to the following provisions:

1. The size, shape, context, wording and appearance of such sign shall not be changed or altered 	unless necessary to comply with this chapter, as amended.

2. A nonconforming sign which is destroyed or damaged by fire or other casualty to the extent 	that the cost of restoration will exceed fifty (50%) percent of the construction cost of a 	comparable new sign shall not be restored unless the sign conforms to all requirements of this 	chapter.

3. If such damage or construction is less than fifty (50) percent of the construction cost of a 	comparable new sign, all repairs shall be completed within one hundred eighty (180) days from 	the date of the partial destruction.

4. If such damage or destruction is less than fifty (50) percent of the construction cost of a 	comparable new sign and repair construction is not completed with one hundred eighty (180) 	day for the date of partial destruction, then the damaged nonconforming sign shall not be 	restored unless the sign is made to conform to all regulations of this chapter.

5. Such signs shall be properly maintained as required by this chapter, but authorized 	maintenance shall not include the right to change or alter the size, shape, context, wording or 	appearance of such sign.

6. No sign which projects more than twenty four (24) inches into a public right of way shall be 	approved or erected.

9.23	GRIEVANCE PROCEDURE

Should a petitioner be aggrieved by a decision of the Village Clerk in his enforcement of this chapter, the Planning and Zoning Commission shall hear the petitioners request for relief.

Zoning Ordinance Amendment 2018

44

Table 9.1

POLE SIGNS

Size of building or 			Maximum height (ft.)		Maximum gross surfaced

integrated center (sq. ft.)								area (sq. ft.)

0 to 35,000			       Not allowed				Not allowed

35,000 to 100,000				18					       150

100,001 to 500,000				23					       200

500,001 +				28					       250

SECTION 10.00	OFF STREET PARKING AND LOADING

10.01 PURPOSE

The purpose of this section is to alleviate or prevent congestion of the public streets and so promote the safety and welfare of the public, by establishing minimum requirements for the off street parking and loading and unloading of motor vehicles in accordance with the use to which property is put.

10.02 	GENERAL PROVISIONS—PARKING AND LOADING

1. Procedure. An application for a building permit for a new or enlarged building, structure or use, shall include therewith a plot plan, drawn to scale and fully dimensional, showing any parking or loading facilities to be provided in compliance with the requirements of these suggested standards.

2. Extent of control. The off street parking and loading requirements of these suggested standards shall apply as follows:

a.  All buildings and structure erected or enlarged and land uses initialed after the effective date 	of these suggested standards shall provide accessory off street parking or loading facilities as 	required hereinafter for the use thereof, except that a building or structure for which a building 	permit has been issued prior to the effective date of these suggested standards shall not be 	required to furnish parking or loading  facilities if construction is begun thereon with six (6) 	months of the effective date of the building permit and diligently prosecuted to completions.

b. When a building or structure erected or enlarged prior to or after the effective date of these 	suggested standards shall undergo a decrease in number of dwelling units, gross floor area, 	seating capacity, number of employees, or other unit of measurement specified hereinafter for 	required, parking or loading facilities and further, when said decrease would result in a 	requirement for fewer total parking or loading facilities may be reduced accordingly provided 	that existing parking and loading facilities shall be so decreased only when the facilities 	remaining would at least equal or exceed the parking or loading requirements resulting from 	application of the provisions of these suggested standards to the entire building or structure as 	modified.

Zoning Ordinance Amendment 2018

45

c. When a building or structure shall undergo any increase in number of dwelling units, gross 	floor area, seating capacity, or other unit of measurement  specified hereinafter for required 	parking or loading facilities  and further, when said increase would result in requirement for 	additional total parking or loading spaces, through application of provisions fo these suggested 	standards, parking and loading facilities may be increased accordingly, provided that existing 	parking or loading facilities shall be so increased that the facilities would at least equal or 	exceed the parking or loading requirements resulting from application of these provisions to the 	entire building and structure as modified.

3. Existing Parking and Loading Spaces. Accessory off street parking and loading spaces in existence on the effective date of these suggested standards may not be reduced in number unless already the requirements of this section for equivalent new construction, in which even, said spaces shall not be reduced below the number required herein for such equivalent new construction.

4. Permissive Parking and Loading Spaces. Nothing in this section shall prevent the establishment of the off street automobile parking or loading facilities to serve any existing use of land or buildings, subject to full compliance with the provisions of this section, except that off street parking areas accessory to existing multiple family structures cannot be located off the premises containing the principal use, unless on a lot adjacent thereto, without special authorization by the Village Board of Trustees.

5. Damage or Destruction. Any building, structure or use which is in existence and is a nonconforming use on the effective date of these suggested standards and which subsequently shall be damaged or destroyed by fire, collapse, explosion or other cause may be reconstructed, reestablished or repaired with or without off street parking or loading facilities, except that parking or loading facilities equivalent to any maintained at the time of such damage or destruction shall be restored or continued in operation. However, it shall not be necessary to restore or maintain parking or loading facilities in excess of those required herein for equivalent new construction.

6. Schedule of Requirements

a. Tables for Required Parking and Loading. Requirements governing the number and location 	of off street parking and off street loading facilities in relation to the use of property are 	established hereinafter in this section. The parking and loading requirements for any use not 	specified herein shall be the same as for a similar specified use, determined by Subsection 	10.06.

b. Floor Area. The term “floor area” as used in the case of office, merchandising or service 	types of use shall mean the gross floor area of a building or structure used or intended to be 	used for service to the public as customers, patrons, clients, patients or tenants, including areas 	occupied by fixtures and equipment used for display or sale of merchandise. “Floor Area” for 	the purpose of this section shall not include any area used for:

1. Storage accessory to the principal use of a building.

2. Incidental repairs.

3. Processing or packaging of merchandise.

Zoning Ordinance Amendment 2018

46

4. Show windows or offices incidental to the management or maintenance of a store or 			building.

5. Restrooms.

6. Utilities.

7. Dressing, fitting or alteration rooms.

10.03	ADDITIONAL REGULATIONS—PARKING

1. Use of Parking Facilities. Off street parking facilities accessory to residential use and developed in any residential district in accordance with the requirements of this section shall be used solely for the parking of passenger automobiles owned by occupants of the dwelling structures to which such facilities are accessory or by guest of said occupants. Under no circumstances shall required parking facilities accessory to residential structures for the storage of commercial vehicles or for the parking of automobiles belonging to the employees, owners, tenants, visitors or customers of business or manufacturing establishments, except as permitted in subsection five (5) below.

2. Joint Parking Facilities. Off street parking facilities for different buildings, structures or used or fore mixed uses, may be provided collectively in any zoning district in which separate parking facilities for each constituent use would be permitted, provided that the total number of spaces so located together shall not be less the sum of the separate requirements for each use.

3. Control of Off Site Facilities. When required accessory of site parking facilities are provided

elsewhere than on the same lot on which the principal use served is located, they shall be in the same possession, either by deed or long term lease, as property occupied by such principal use and the owner shall be bound by covenants filed of record in the Office of the Recorder of Deeds, requiring the owner and his or her heirs and assigns to maintain the required number of parking spaces during the existence of said principal use.

4. Surfacing. All open off street parking areas shall be hard surfaced or shall be surfaced with a dustless material.

5. Screening and Landscaping. All open off street parking areas for six or more cars shall be effectively screened by a wall, solid fence or hedge shall be at least four (4) feet but not more than seven (7) feet in height and shall be maintained in good condition.

6.  Lighting. Any lighting used to illuminate an of street parking area shall be so arranged as to reflect the light away from adjoining properties.

7. “Special Use” Public Parking Areas. Any automobile parking area developed for transient trad and not accessory to specific principal uses or groups of uses for which parking is required by these suggested standards, shall be treated as a “special use” as defined in 12.00 by the Village Board.

Zoning Ordinance Amendment 2018

47

10.04	ADDITIONAL REGULATIONS—LOADING

1. Design

a. Loading Berth- Description. An off street loading berth shall be a hard surface area of land, 	open or enclosed, other than a street or public way, used principally for the standing, loading or 	unloading of motor trucks, tractors and trailers so as to avoid undue interference with the public 	use of streets and alleys. A required loading space shall not be less than ten (ten) feet in width, 	forty five (45) feet in length and fourteen (14) feet in height, exclusive of access aisles and 	maneuvering space, except as otherwise specifically dimensioned hereafter.

b. Location. No permitted or required loading berth shall be closer than fifty (50) feet to any 	property in residential district unless completely enclosed by building walls, or a uniformly 	painted solid fence or wall or any combination thereof not less than six (6) feet in height. No 	permitted or required loading berth shall be located within twenty five (25) feet of the nearest 	point of intersection of any two (2) streets. Loading berths open to the sky may be located in 	any required yards.

c.  Measurements of Berth. When determination of the number of required off street loading 	berths results in a requirement of a fractional berth, any fraction up to and including one half 	(½) shall be disregarded and fractions over one half (½) shall be interpreted as one loading 	berth.

d. Surfacing. All open off street loading berths shall be improved with a hard surface or shall be 	surfaced with dustless material.

10.05	LOCATION OF PARKING AREAS

1. Extent of Control. Off street automobile parking facilities shall be located as hereinafter specified, where a distance is specified, such as distance shall be walking distance measured from the nearest point of the parking area to the nearest entrance of building that said parking area is required to serve:

a. For one and two family dwellings, on the same lot with the building they are required to 	serve.

b. For three and four family dwellings not over two stories in height, on the same lot or parcel 	of land as the building they are required to serve. For the purpose of these suggested standards, 	a group of such   uses constructed and maintained under single ownership or management shall 	be assumed to be one single lot or parcel of land.

c. For apartment houses containing four or more dwelling units, on the same lot or parcel of 	land as the building they are required to serve or on a separate lot or parcel of land not more 	than three hundred (300) feet from the nearest point of the parking area, provided the separate 	lot or parcel intended for the parking facilities is located in the same district as the principal 	permitted use or in a less restricted district.

Zoning Ordinance Amendment 2018

48

10.06	SCHEDULE OF PARKING REQUIREMENTS

1.  For one and two family dwellings, one parking space for each family dwelling unit.

2. For three and four family dwellings, on parking space for each family dwelling unit.

3. For hotels and clubs, one parking space for each six (6) guests or sleeping rooms and suites, plus one parking space for each three hundred (300) square feet of floor area devoted to supplementary parking generative activities such as bars, ballrooms, dining rooms, night club facilities and the like.

4. For motels, parking space for each guest or sleeping room or suite, plus one additional space for the owner, manager and employee on the premises.

5. For lodging, rooming and boarding houses, one parking space for each four guests, plus one additional space for the owner or manager if resident on the premises.

6.  For private clubs or lodges (without sleeping rooms) parking spaces in number to twenty five (25) percent of the total membership.

7. For hospitals, one parking space for each three hospital beds, plus one parking space for each fifteen hundred (1500) square feet of gross floor area in residents’ quarters, plus one additional space for each staff or visiting doctor.

8. For sanitariums, convalescent homes, nursing homes or homes for the aged, one space for each four (4) patient beds, plus one additional space for each staff or visiting doctor.

9. For medical or dental clinics, three parking spaces per doctor engaged at the clinic.

10. For mortuaries or funeral parlors, five (5) parking spaces for each room used as a chapel or parlor, plus one space for each funeral vehicle maintained on the premises, plus one space for each family residing on the premises.

11. For bowling alleys, two parking spaces for each alley plus such additional spaces as may be determined by the Zoning Administrator  for affiliated uses such as bars, restaurants and the like.

12. For convention halls, dance halls, skating rinks, assembly halls, exhibition halls or other places of assembly, one parking space for each one hundred (100) square feet of floor area used for assembly.

13. Stadium, sports arena, auditorium and gymnasium (other than incidental to a school), one parking space for each eight seats.

14. For theaters:

a. Indoor-- one parking space for each ten seats up to five hundred (500) plus one parking space 	for each five (5) seats above five hundred (500).

b. Outdoor-- Reserved space equal to ten (10) percent of the capacity.

Zoning Ordinance Amendment 2018

49

15. For church, Sunday school, high school, college and university auditoriums, one parking space for twelve (12) seats provided in said building or structures.

16. For airports, railroad passenger stations, bus depots or other passenger terminal facilities (special use) such as parking space as the Village Board, subject to recommendation of the Planning and Zoning Commission, shall deem to be adequate for employees, passengers, spectators, visitors and others.

17. For banks, business or professional offices or public administration buildings, on parking space fore each four hundred (400) squared feet of floor area.

18. For mixed use-- the parking facilities required shall be the sum of the requirements for the various individual uses, computed separately in accordance with this section, parking facilities for one use shall not be considered as providing the required parking facilities for any other use in the same building on the same lot.

19. For retail stores and service shops, on parking space for each three hundred (300) square feet of retail floor area.

20. For establishments handling the sale and consumption of alcoholic beverages, food or refreshments on the premises, one parking space for each three hundred (300) square feet of floor space.

21. For furniture and appliance stores, motor vehicle sales, wholesale stores, household equipment or furniture repair shops or machinery sales, one parking space for each six hundred (600) square feet of floor space.

22. For manufacturing and industrial uses, research and testing laboratories, laundry and dry cleaning plants, printing, binding, publishing, and issuing of newspapers, periodicals, books, and other reading matter, telephone exchanges, warehouses, storage buildings, engraving shops, assembly of materials and products, and other similar uses, one parking space for each four employees, based upon the maximum number of persons to be employed at any one work period during the day or night, plus such additional parking facilities as shall be required for all vehicles used in the conduct of the enterprise.

10.07	OFF STREET LOADING AND UNLOADING FACILITIES

In all districts where property uses include the loading and unloading of materials or merchandise from vehicles, of street loading and unloading facilities shall be provided in accordance with the following requirements:

1.  For hospitals or sanitariums containing forty thousand (40,000) to one hundred thousand (100,000) square feet of gross floor area, one off street loading and unloading space, plus one additional such space for each additional one hundred thousand (100,000) square feet of gross floor area for fraction thereof in excess of one hundred (100,000) square feet.

2.  For building containing bowling alleys, taverns, restaurants, or any retail shops and having ten thousand (10,000) or more square feet of gross floor area, one off street loading and unloading space.

Zoning Ordinance Amendment 2018

50

3. For banks, business or professional offices or public administration buildings containing twenty (20,000) square feet or more of gross floor space, one off street loading and unloading space.

4. For buildings containing furniture and appliance stores, motor vehicles sales, wholesale stores, household equipment or machinery sales and having eight thousand (8000) to twenty five thousand (25,00) square feet of gross floor area, one off street loading and unloading space.

5. For buildings containing manufacturing uses, research and testing laboratories, laundry and dry cleaning establishments, printing, binding, publishing and issuing of newspapers, periodicals, books and other reading matter, warehouse and storage facilities, engraving shops, assembly of materials and products, processing and distribution of materials and products and other similar uses. Having more than eight thousand (8000) square feet of gross floor area, exclusive of the basement area, and less than forty thousand (40,000)  square feet of gross floor area, one off street loading and unloading space, plus one additional such space for each additional sixty thousand (60,000) square feet of gross floor area in excess of forty thousand (40,000) square feet.

SECTION 11.00	NONCONFORMING BUILDINGS AN USES

11.01	CONTINUANCE OF USE

1. Any lawfully established use of a building or land on the effective date of this ordinance or of amendments thereto, that does not conform to the use regulations for the district in which it is located, shall be deemed to be a legal nonconforming use and may be continued, except as otherwise provided herein

2. Any legal, nonconforming building or structure may be continued in use provided there is no physical change other than necessary maintenance and repair, except otherwise permitted herein.

3. Any building for which a permit has been lawfully granted prior to the effective date of the ordinance or of amendments thereto may be completed in accordance with the approved plans, provided construction is started within ninety (90) days and diligently prosecuted to completion. Such building shall thereafter be deemed a lawfully established building.

11.02	DISCONTINUANCE OF USE

1. Whenever any part of a building, structure or land occupied by a nonconforming use is changed to or replaced by a use conforming to the provisions of this ordinance, such premises shall not thereafter be used or occupied by a nonconforming use ,even though the building may have been originally designed and constructed for the prior nonconforming use.

2. When a nonconforming use of a building or structure or part thereof has been discontinued for a period of twelve (12) consecutive months or wherever there is evident a clear intent on the part of the owner to abandon a nonconforming use, such use shall not after being discontinued or abandoned, be reestablished and the use of the premises thereafter shall be in conformity with the regulations of the district.

Zoning Ordinance Amendment 2018

51

3. Where no enclosed building is involved, discontinuance of a nonconforming use for a period of six (6) months shall constitute abandonment and shall not thereafter be used in nonconforming manner.

4. A nonconforming use not authorized by the provisions of the zoning ordinance in effect at the time the amended ordinance becomes effective, shall be discontinued and nor reestablished, except when the provisions of the amended ordinance find the use to be conforming to the district in which it is then located.

11.03	CHANGE OF USE

A nonconforming use of a building or structure, or part thereof may be changed or a use of the same or of a more restricted character, but may not hereafter be changed to any less restricted use. No part of a building, structure or land occupied by a nonconforming use which is changed to or replaced by a use conforming to the provisions to of this ordinance shall thereafter be used or occupied for nonconforming use.

11.04	TERMINATION AND REMOVAL OF NONCONFORMING USES, BUILDINGS AND STRUCTURES

The period of time during which the following nonconforming uses of buildings, structures or land may continue or remain shall be limited from the effective date of this ordinance or amendment hereto which causes the use to be nonconforming. Every such nonconforming use shall be completely removed from the premises at the expiration of the period of time specified below:

1. Any nonconforming use of building or structure having an assessed valuation not in excess of five hundred (500) dollars on the effective date of this ordinance shall be removed after two (2) years.

2. Any nonconforming use of land where no enclosed building is involved, ore where the only buildings employed are accessory or incidental to such use or where such use is maintained in connection with a conforming building shall be removed after a period of two (2) years

3. All nonconforming signs, billboards and outdoor advertising structures shall be removed after a period of three (3) years.

11.05	REPAIRS AND ALTERATIONS

1. Normal maintenance of a building or other structures containing a nonconforming use is permitted, including necessary nonstructural repairs and incidental alterations which do not extend or intensify the nonconforming use.

2. No structural alteration shall be made in a building or other structure containing a nonconforming use, except in the following situations:

a. When the alteration is required by law.

b. When the alteration will actually result in eliminating the nonconforming use.

Zoning Ordinance Amendment 2018

52

c. When a building in a residential district containing residential nonconforming uses may be 	altered in any way to improve livability, provided no structural alterations shall be made which 	increase the number of dwelling units or the bulk of the building.

11.06	DAMAGE AND DESTRUCTION

If a building or other structure containing a nonconforming use is damaged or destroyed by any means to the extent of fifty (50) percent or more of its replacement value at that time, the building or other structure can be rebuilt or used thereafter only for a conforming use and in compliance with the provisions of the district in which it is located. In the event the damage or destruction is less than fifty (50) percent of it replacement value, based upon prevailing costs, the may then be restored to it original condition and the occupancy or use of such building may be continued which existed at the time of such partial destruction.

In event allowing restoration or repair, such restoration of the building or other structure must be started within a period of six (6) months form the date of damage or destruction and diligently prosecuted to completion and completion shall be within one (1) year from the date of damage, if not completed with one (1) year the use shall be abandoned as specified in Section 11.02

11.07	ADDITIONS AND ENLARGEMENTS

1.  A nonconforming building may be enlarged or extended only if the entire building is thereafter devoted to a conforming use and is made to conform to all regulations of the district in which it is located.

2. No building partially occupied by a nonconforming use shall be altered in such a way as to permit the enlargement or expansion of the space occupied by such nonconforming use

3. No nonconforming use may be enlarged or extended in such a way as to occupy any required usable open space, or any land beyond the boundaries of the zoning lot as it existed on the effective date of ordinance or to displace any conforming use in the same building or on the same parcel.

11.08	USE DISTRICT CHANGES

Whatever the boundaries of a use district shall be changed so as to transfer an area from on use district to another us district of a different classification, the foregoing provisions shall also apply to any nonconforming uses existing therein.

12.00	SPECIAL USES

12.01 PURPOSE

The development and execution of a zoning ordinance is based upon the division of the Village into districts, with which districts the use of land and buildings and the bulk and the location of buildings and structures in relation to the land are substantially uniform. It is recognized, however, that there are

Zoning Ordinance Amendment 2018

53

uses which because of their unique characteristics, cannot be properly classified in any particular district or districts without consideration, in each case, of the impact of those uses on neighboring land and of the public need for the particular use at the particular location. Such special uses fall into two categories:

1. Uses publicly operated or traditionally affected with a public interest.

2. Uses entirely private in character bu of such and unusual nature that their operation may give 	rise to unique problems with respect to their impact upon neighboring property or public 	facilities.

A. Initiation of Special Uses.  Any person owning or having an interest in the subject property may file application to use such land for one or more of the Special Uses provided for in this ordinance in the zoning district in which the land is situated.

B. Application for Special Use.  An application for a Special Use of expansion of a Special Use shall be filed with the Village Clerk and shall be accompanied by such plans or date as prescribed by the Planning and Zoning Commission from time to time.

C. Hearing on Application. Upon receipt of the application referred to above, the Planning and Zoning Commission shall hold at least one public hearing. At least fifteen (15) day in advance of such hearing, but not more than thirty (30) days, notice of the time place and purpose of such hearing shall be published in a newspaper of general circulation in the Village of Millington. In addition to the above

the applicant of Special Use shall within the same period serve written notice, either in person or by registered mail, return receipt requested, on the owners, as recorded in the office of the recorder of deeds of all adjoining  property owners and such property owners holding land within two hundred fifty (250) feet in every direction of the lot upon which the Special Use is requested. The number of feet occupied  by all public roads, streets, alleys and other public ways shall be excluded in computing the two hundred fifty (250 feet requirement. The notice herein required shall contain a description of the location for which the Special Use requested, a brief statement of the nature of the requested Special Use, the name and address of the legal and beneficial owner of the property for which the Special Use is requested and the time, date and place of the hearing. If after a bona fide effort to determine such address the applicant for Special Use, the owner of the property on which the notice is served cannot be found at his or her last known address or the mailed notice is returned because the owner cannot be found at the last known address the notice is requirements of this subsection shall be deemed satisfied. The applicant shall furnish to the Planning and Zoning Commission proof of providing  the above notice. The Planning and Zoning Commission shall not hear the application for Special Use until all notice requirements are satisfied.

D. Authorization. For each application for a special use, the Planning and Zoning Commission shall report to the Village Board its findings and recommendations, including the stipulations of additional conditions and guarantees that such conditions will be complied with when they are deemed necessary for the protection of the public interest. The Village Board may grant or deny any applications for a

Zoning Ordinance Amendment 2018

54

Special Use, provided, however, that in the event of written protest against any proposed special use, signed and acknowledged by the owners of twenty (20) percent of those owners required to receive notice of the hearing, such special use shall not be granted except by the favorable vote of two thirds (2/3) of all members of the Village Board.

E Standards. No Special Use be recommended by the Planning and Zoning Commission or granted by the Village Board absent the following findings:

1. That the establishment, maintenance or operation of the Special Use will not be unreasonably detrimental to or endanger the public health, safety, morals, comfort or 	general welfare.

2. That the special use will not be injurious to the use and enjoyment of other property in the immediate vicinity for the purpose already permitted nor  substantially diminish and impair property values within the neighborhood.

3. That the establishment of the special use will not impede the normal and orderly development and improvement of surrounding property for uses permitted in the district.

4. That adequate utilities, access roads, drainage or other necessary facilities have been or are being provided.

5. That adequate measures have been or will be taken to provide ingress and egress so designed as to minimize traffic congestion in the public streets.

6. That the special use shall in all other respects conform to the applicable regulations of the district in which it is located.

12.02 Special Standards for: PERSONAL WIRELESS SERVICE FACILITIES AND COMMERCIAL RADIO OR COMMERCIAL TELEVISION TOWERS

In addition to the above mentioned standards for conditional uses, the following requirements shall be applicable when consideration of a request for a telecommunications facility including a tower or when consideration of a request for commercial radio or commercial television tower.

1. The applicant proposing to construct an antenna is required to demonstrate, using technological evidence:

A. The location is to satisfy its function in the providers grid system.

B. That the applicant has mapped the location area and contacted the owners of all structures in

the area, not less than a one (1) mile radius of the site proposed, asked permission to install

the antenna on those structures and was denied for reason other than economically reason

concerns. The fees and cost required in order to share and existing antenna shall be presumed

unreasonable if such costs exceed the costs of a new antenna facility.

C. That the height requested is the minimum height with a necessary allowance for he potential

co-location by other potential uses.

Zoning Ordinance Amendment 2018

55

D. That the proposed antenna is designed to accommodate antennas for more than one (1) user,

unless the applicant demonstrate why such design is not feasible for economic, technical or

physical reasons.

E.  That the proposed antenna complies with applicable setbacks and where possible the fall zone

of the antenna lies within the applicable setbacks. In no event shall a fall zone of tower include

a residential structure.

F. That the proposed antenna and related facility complies with all F.C.C. and F.A.A. standards

and applicable standards of any other governmental agency and the applicable standards for

towers that are published by the Electronic Industries Association (“EIA”).

G. That the maintenance and improvement plan exists t assure ongoing compliance with

applicable standards including compliance with amendments to standards made more

stringent.

H. That the proposed antenna and related equipment will not cause localized interference with the

reception or transmission of any other communications signals including, but not limited to,

public safety signals, television and radio broadcast signals and telephone usage.

I. At time of application applicant must provide all applications, licenses, and/or leases filed with

private parties and/or governmental regulators including bu not limited to the F. C. C pertaining

to the proposal site.

J. That required lease and license agreements have been secured with private parties and/or

governmental regulators including but not limited to the F.C.C.

K.  The Planning and Zoning Commission or Village Board may request the services of a third

party expert to review technological evidence presented by the applicant and the applicant

shall be liable for the costs of such third party expert. The Planning and Zoning Commission

may secure the services of an independent safety consultant to conduct periodic inspections

and the special use holder shall be liable for the costs of such Inspections.

2. The applicant must demonstrate that the antenna or tower facilities will not have an adverse affect on nearby property values. Where the total equalized assessed value of improved property exceeds one million (1,000,000) dollars within one (1) mile of either a proposed antenna or tower exceeds one hundred (100) feet in height or a proposed antenna or tower involving more than one (1) antenna or tower on the same ¼ mile, the antenna shall be presumed to have a significant adverse affect on nearby property values.

3. The applicant must demonstrate that reasonable steps will be taken to mitigate the adverse visual affect of the antenna. This shall include minimal tower lighting, consideration of tower paint or finishing, architectural  and aesthetic compatibility to adjoining locality and structures, screening and vegetation.

Zoning Ordinance Amendment 2018

56

4. The applicant must demonstrate that the antenna is protected for security purposes. Security shall include visual buffering, fencing, anti-climbing devices, physical and electronic security.

5. In the event the use of any tower or facility has been discontinued for a period of sixty (60) consecutive days, the facility shall be deemed to be abandoned. Determination of the site of abandonment shall be made by the Planning and Zoning Commission, which shall have the right to request documentation and/or affidavits from the tower owner or operator regarding the issue of tower usage. Upon such abandonment, the owner or operator of tower shall have an additional sixty (60) days within which to reactivate the use of the tower or transfer the tower to another owner or operator who make actual use of the tower or dismantle and remove said facilities. A bond or other security shall insure funds are available to dismantle an abandoned facility.

6. Additional notice of a special use hearing invoking proposed antenna which exceeds one hundred (100) fee in height or two (2) or more towers one fourth (¼) mile shall be provided to all land owners with one (1) mile. Notice shall be provided either in person or by registered mail, return receipt requested.

12.03	CONDITIONS

A grant of Special Use under the terms of this article may be made subject to such conditions as are proper and necessary for the protection of the public welfare and use by the applicant or subsequent owner or occupant of the property, of the privilege granted by the Special Use, may be continued only as such conditions are complied with.

SECTION 13.00	VARIATIONS

13.01	GRANTING

Granting from the terms and regulations of the zoning ordinance may be granted, as authorized by statute, in specific cases where an undue hardship exists. Such variations shall be granted by the Board of the Village, by resolution or ordinance, following a public hearing before the Planning and Zoning Commission. The grant of variation shall incorporate the findings of the Planning and Zoning Commission.

13.02	APPLICATIONS

Applicants for the grant of a variation shall be made in writing in triplicate and shall contain all the facts necessary to a consideration of the proposal and shall be accompanied by a fee of three hundred ($300) dollars plus attorney’s fees if needed.

13.03 	HEARING

No variation shall be granted excepting after a public hearing before the Planning and Zoning Commission notice of which must be published at least once not less than fifteen (15) or more than thirty (30) days before the hearing, as required by law. In addition to the above, the applicant for

Zoning Ordinance Amendment 2018

57

variance shall within the same time period serve written notice, either in person or by registered mail, return receipt requested, on the owners, as recorded in the office of the recorder of deeds of all adjoining property owners and such property owners holding land within two hundred fifty  (250) feet in every direction of the lot upon which the variance is requested. The number of feet occupied by all public roads, streets, alleys and other public ways shall be excluded in computing the two hundred fifty (250) feet requirement. The notice herein required shall contain a description of the location for which the variance is requested, a brief statement of the nature of the requested variance, the name and address of the legal and beneficial owner of the property for with the variation is requested and the time, date and place of hearing. If after bona fide effort to determine such address by the applicant for variance =, the owner of the property on which the notice is served cannot be found at his or her last known address, the notice of requirements of this subsection shall be deemed satisfied. The applicant shall furnish to the Planning and Zoning Commission proof of providing the above notice. The Planning and Zoning Commission shall not hear the application for until all notice requirements are satisfied.

13.04	REPORT ON VARIATIONS

After holding a public hearing on a requested variation the Planning and Zoning Commission shall make its report, together with its findings of fact to the Mayor and Board promptly.

The Planning and Zoning Commission shall also report to the Village Board additional conditions and guarantees that are deemed necessary for the protection of the public interest. The Village Board may grant or deny any application for a variance, provided, however that in the event of written protest against any proposed variance signed and acknowledged by the owners of twenty (20) percent of those owners required to receive notice of the hearing, such variance shall not be granted except by the favorable vote of two thirds (2/3) of all members of the Village Board.

13.05 	STANDARDS

No variance shall be recommended by the Planning and Zoning Commission or granted by the Villages Board absent the following findings:

1. The property in question cannot yield a reasonable return if permitted to be used only under the conditions allowed by the regulations in that zone.

2. The plight of the owner is due to unique circumstances.

3. The variation, if granted will not alter the essential character of the locality.

The Planning and Zoning Commission in its recommendation to the Village Board may also wish to consider the following issues:

1. The particular physical surrounding, shape or topographical condition of the specific property involved would result in a practical difficulty or particular hardship upon the owner as distinguished from a mere inconvenience, if the strict letter of the regulation were carried out.

Zoning Ordinance Amendment 2018

58

2. The conditions upon which the petition for a variation is based are unique and would generally not be applicable to other property within the same zoning classification.

3. The purpose of the variation is not based exclusively upon a desire to realize economic gain.

4. The alleged difficulty or hardship has not been created by any person presently having an interest in the property.

5. The granting of the variation will not be materially detrimental to the public welfare or injurious to the other property or improvements in the neighborhood in which the property is located.

6. The proposed variation will not impair an adequate supply of light an air to the adjacent property.

13.06 	CONDITIONS

A grant of variation under the terms of this article may be made subject to such conditions as are proper and necessary for the protection of the public welfare and used by the applicant or subsequent owner or occupant of the property, of the privilege granted by the variat52n, may be continued only so long as such conditions are complied with.

13.07	TIME LIMIT ON VARIATIONS

No order for a variance permitting the erection or alteration of a building shall be valid for a period of longer than six (6) months unless such use is established with such period, provided however, that such use permitted is depending upon the erection or alteration of a building, such order shall continue in force and effect if a building permit for said erection or alteration is obtained within such period and

such erection or alteration is started and proceeds to completion in accordance with the terms of such permit.

No order for a variance permitting a use of a building or premises shall be valid for a period longer than six (6) months, unless such use is established within such period, provided however, that where such use permitted is dependent upon the erection or alteration of a building, such order shall continue in force and affect if a building permit for said erection or alteration is obtained with such period and such erection or alteration is started and proceeds to completion in accordance with the terms of such permit.

14.00 	ADMINISTRATIVE PROVISIONS

14.01	ENFORCEMENT OFFICER

The building inspector of the Village, or such other officer as may be designated by the Mayor with the approval of the Board, shall be the zoning enforcement officer of the Village.

14.02	CREATION AND MEMBERSHIP

The Planning and Zoning Commission of the Village of Millington as heretofore created and established under the provisions of Zoning Ordinance of the Village of Millington as amended is

Zoning Ordinance Amendment 2018

60

hereby reconstituted and reestablished as the Planning and Zoning Commission under the provisions of this comprehensive amendment. The said Board shall consist of seven (7) members appointed by the Mayor by and with the consent of the Board. The term shall be five (5) years, provided that the members of the first Board shall server respectively  for the following terms (or until their respective successors are appointed and qualified) One for one year, one for two years, one for three years, one for four years, one for five years, one for six years and one for seven years and one five years each of those following the first appointment. One of the members of said board shall be designated by the Mayor, with consent of the Board, as Chairman of said Commission and shall hold this office as Chairman until successor is appointed.

14.03	MEETINGS

Meetings of the Planning and Zoning Commission shall be held at the call of the Chairman or any two (2) members or at such times as such Commission may determine. All hearings conducted by said Commission shall be open to the public. The Commission shall keep minutes of its proceedings showing the vote of each member upon each question or is absent or failing to vote, indicating such fact and shall also keep records of it hearings and other official actions. Findings of fact shall be included in the minutes of each case of a requested variation or special permit and the reason for granting or denying such application shall be specified. Every rule or regulation, every amendment or repeal thereof and every order, requirement, decision or determination of the Commission shall be filed immediately in the office of the Commission and shall be a public record. The Commission shall adopt its own rules of procedure not in conflict with this Ordinance or with the Illinois Statutes in such case made and provided and may select or appoint such officers as it deems necessary.

14.04	APPEALS

Any person aggrieved by any decision of the zoning enforcement officer involving the interpretation of the zoning ordinance may appeal, as provided by statute to the Planning and Zoning Commission. Upon such appeal being made, the Planning and Zoning Commission shall hold a hearing and shall affirm, overrule or modify the decision of the zoning enforcement officer as the facts may indicate.

14.05 	 HEARING ON APPEAL

The Planning and Zoning Commission shall keep an accurate record of all proceedings and of all evidence presented to it on appeals from the zoning enforcement officer and shall furnish the same to the attorney for the Village in the event an administrative review action is taken to have its decision reviewed by the courts as provided by law.

14.06	NOTE REQUIRED ON APPEALS

The occurring vote of four (4) members of the Planning and Zoning Commission shall be necessary for the Planning and Zoning Commission to reverse any decision by the zoning enforcement officer.

Zoning Ordinance Amendment 2018

61

14.07	FEES

Petitions to appeal from a decision of the zoning enforcement officer, petitions for variation of special use and petitions for a change in an amendment to the zoning ordinance, must be accompanied by a fee of fifty (50) dollars and no action shall be taken on any such petition until the fee has been paid unless the Board waives requirement of the payment of the fees.

14.08	AMENDMENTS

A public hearing on any proposal to amend the zoning ordinance as required by statute, shall be held before the Planning and Zoning Commission of the Village unless the Board by resolution or ordinance directs otherwise. The Commission or other body conducting  such public hearing shall make its recommendation promptly  after the termination of its hearing on the proposed amendment.

Three copies of each petition to amend the zoning ordinance shall be filed, starting the purpose of the proposed  amendment and describing the property affected thereby.

14.09	SEPARABILITY

Each section, clause and provision of this ordinance shall be considered as separable and the invalidity of one or more shall not have any affect upon the validity of other sections, clauses or provisions of these ordinance.

14.10	PENALTY

Any person, firm or corporation violating any provision of this ordinance shall be fined not less than twenty five (25) dollars nor more than five hundred (500) dollars for each offense and a separate offense shall be deemed committed on each day during or on which a violation occurs or continues.

Zoning Ordinance Amendment 2018

62