# Pool Contamination Response Protocol

☣️ Fecal, Vomit, or Biohazard Event Procedures

# I. Immediate Response

- ☐ Clear the pool immediately and evacuate all swimmers
- ☐ Identify the type of contamination (formed stool, diarrhea, vomit, blood)
- ☐ Close the affected pool and post signage
- ☐ Notify supervisor and document the time of incident
- ☐ Remove visible contamination using a net/skimmer — DO NOT vacuum

# II. Disinfection Procedure

**Formed Stool:**
- ☐ Maintain free chlorine at 2 ppm or higher
- ☐ Maintain pH between 7.2–7.5
- ☐ Ensure proper circulation for 30 minutes before reopening

**Diarrhea or Vomit:**
- ☐ Raise free chlorine to 20 ppm
- ☐ Maintain pH 7.2–7.5
- ☐ Circulate water for **13 hours** minimum
- ☐ Backwash filter after disinfection period ends
- ☐ Record time disinfection started and completed

# III. Cleanup & Documentation

- ☐ Dispose of contaminated material properly (biohazard bag)
- ☐ Clean and disinfect pool deck area where incident occurred
- ☐ Document all chemical adjustments
- ☐ Complete incident report and submit to facility director
- ☐ Log time pool was reopened
- ☐ Notify maintenance if additional sanitation is required

# IV. Communication

- ☐ Inform staff of situation and response timeline
- ☐ Notify patrons if necessary (template email available)
- ☐ Update signage when pool is reopened