.

PRODUCT DATA

TC-8081

A SERIES OF PHOSPHATE ESTERS SUPPLIED AS THE FREE ACID. WITH MODERATE  TO   HIGH FOAMING PROPERTIES.  NEUTRALIZATION WITH AN ALKYL OR  ALKANOLAMINE WILL ALTER ITS’ SOLUBILITY.  GENERALLY THESE PRODUCTS HAVE  EXCELLENT COUPLING PROPERTIES AND HIGH ELECTROLYTE TOLERANCES.

.

TYPICAL PROPERTIES

APPEARANCE					CLEAR AMBER LIQUID

POUR POINT					< 40  F

FLASH POINT, TCC  F				> 300 F

pH, (1% SOLUTION)				1.5 - 2.5

SPECIFIC GRAVITY, 72  F			1.1

DENSITY, LBS/GAL  72  F			9.7

VISCOSITY, 72  F

SOLUBILITY, 10%

WATER				SOLUBLE

ISOPROPANOL			SOLUBLE

KEROSENE			             INSOLUBLE

XYLENE				SOLUBLE

CAUSTIC SODA 50%

ACID NO.  (MG. KOH/G)

pH 5.5	           			160 - 190

pH 9.5	           			310 - 360

APPLICATION

THE OUTSTANDING OIL AND GREASE REMOVAL CHARACTERISTICS OF THESE   COMPOUNDS MAKE THEM EXCELLENT CANDIDATES FOR USE IN HIGHLY ALKALINE     CLEANER, LIQUID OR DRY BLENDED.

6710 Cadillac Street, Houston, Texas 77021

713-747-3134