47-53 Chevy Truck

Firewall Insulator

This Firewall Insulator was reproduced for the year and model of the vehicle you specified in your order.  However, there may be some variation between models in any given year of manufacturer.  Compare the new Firewall Insulator to your original or to the vehicle’s firewall for fit and alignment.  If you find that this insulator does not fit your vehicle, do not alter it in any way.  Contact us immediately and we will solve the problem.

Installation: Remove all electrical/mechanical controls form the vehicle metal firewall.  Remove mounting fasteners carefully so that they can be cleaned and reused and remove the old insulator pad.  Clean and paint the firewall as required.  Position the new Firewall Insulator against the firewall and using a small Phillips head screwdriver locate a mounting hole near the top/center of the firewall and insert a long nail.  Work toward the sides and bottom inserting nails as you go.  When you are satisfied with the fit and alignment, remove a nail and replace with a fastener.  You may find the assistance of a friend helpful.  Once all of the mounting fasteners are in place re-install the electrical/mechanical controls, etc.  Do not over tighten mounting screws—“snug them.”  It’s better to tighten them little by little over time as the materials relax and compress.

Modifications:  Many of our Firewall Insulators are shipped with “factory knockouts” scribed on the surface.  Additional holes can be easily installed with an electrical hand drill and metal drill bits.  Experiment with drilling and cutting on piece of plastic enclosed in this envelope.  After you have determined the exact location of the holes to be installed, place the Firewall Insulator on a block of wood on your workbench and pull back the felt insulation material.  Slowly drill your hole and clean the edges with a small bladed knife or piece of rolled sand paper.

Care:  This Firewall Insulator is made of ABS plastic material and needs little or no care.  It can be cleaned like any other upholstery material with a damp cloth or vinyl conditioner.  Do not clean with solvents, as this may damage the surface luster.

www.roddoors.com

806 H S DIVISION STREET

WAUNAKEE, WI 53597

PH 530-828-9426

FX 530-896-1518