Date:	July 30, 2026						Bid Stage:

QUALIFICATIONS:

PROJECT:  NV Oakey Maternal Child

Net Square feet: 12,176 SFT

Scale: 3/16" = 1'-0" (Scale Verified by 3’ Door)

Scope Of work:  Used As a Medical Clinic and Office.

Addendum and RFI

Take-off based on plans and specs dated 09/16/2020

MATERIAL VERIFICATION LINKS:

Carpet: (Broadloom. CPT, Underlayment, Padding)

C: CPT-1: Shaw Contract, Array, Shiny Pebble, 94761, Modular 24" X 24", Eco Solution Q Nylon, Brick Ashlar.

https://www.shawcontract.com/en-us/product/details/5t295?colorNumber=94761

C: WM-1: Mannington, Ruffian Ii 3802, Ebony Earth 1506, 24" X 24", Infinity Re Modular, Monolithic.

https://www.manningtoncommercial.com/products/carpet/entryway-systems/ruffian-ii/?color=c3adb87d-0ed6-4f28-84ed-d2971d066c87

Resilient: (VCT, SDT, RT, LVT, Underlayment)

R: LVT-1: Mannington, Amtico, Vintage Teak Arow7600, 36" X 6" X .096" Thick.

https://www.manningtoncommercial.com/products/hard-surface/amtico-signature/amtico-wood/?color=cd484c1e-4684-4829-8f3d-9406774d841f

R: VCT-1: Armstrong, Imperial Texture Standard Excelon, 51858 Sandrift White,(12"X12").

https://www.armstrongflooring.com/commercial/en-us/products/vinyl-composition-tile/std-excelon-imp-texture/item/51858.html

R: RF-1: Mannington, Lino, Alfalfa Etw100, Modeled Color To Match. (12’ Wide Roll)

https://www.manningtoncommercial.com/products/hard-surface/heterogeneous/lino/?color=bc6f8079-75b4-4346-ba61-86a09125cab9

NOTE: Use Quantum Guard Pen At Seams - Heat Weld All Seams. 12' Wide Roll.

R: RF-2: Mannington, Lino, Sepia Etw101, Modeled Color To Match. (12’ Wide Roll)

https://www.carpet-wholesale.com/item-335989/vinyl/mannington-commercial-vinyl-flooring/entwined-collection/lino-12'/sepia.php

NOTE: Use Quantum Guard Pen At Seams - Heat Weld All Seams. 12' Wide Roll.

R: RF-3: MANNINGTON, LINO, STONE ETW103, MODELED COLOR TO MATCH. (12’ Wide Roll)

https://www.manningtoncommercial.com/products/hard-surface/heterogeneous/lino/?color=9eb84ee5-a8fd-4c53-918a-a521761943fc

NOTE: Use Quantum Guard Pen At Seams - Heat Weld All Seams. 12' Wide Roll

Tile:

T: CT-1: Daltile, Affinity, Beige Af02, 12" X 12", Custom Building Products Grout (Epoxy), #172 Urban Putt, 1/16" Grout Joint.

https://www.daltile.com/product/affinity?shape=Square&color=Beige&size=12%20X%2012

T: CWT-1: Daltile, Color Wheel Collection, Urban Putty 0161 - Semi-Gloss Finish, 4" X 12", Custom Building Products Grout (Epoxy), #172 Urban Putty, 1/16" Grout Joint

https://www.daltile.com/product/color-wheel-collection-linear?color=Urban-Putty&shape=Rectangle&size=4%20X%2012

Paint:

P: PT-1: SHERWIN WILLIAMS - PROMAR 200 LATEX, SW 6140 MODERATE WHITE, EGG SHELL.

https://www.sherwin-williams.com/homeowners/color/find-and-explore-colors/paint-colors-by-family/SW6140-moderate-white

Location: General Wall Paint.

P: PT-2 SHERWIN WILLIAMS - PROMAR 200 LATE, 6141 SOFTER TAN, EGG SHELL.

https://www.sherwin-williams.com/homeowners/color/find-and-explore-colors/paint-colors-by-family/SW6141-softer-tan

Location: Accent Wall Paint.

P: PT-3: SHERWIN WILLIAMS - PROMAR 200 LATEX TO MATCH GLIDDEN PROFESSIONAL, A1936 GRAVEL PIT, EGG SHELL.

https://www.sherwin-williams.com/painting-contractors/products/promar-200-hp-zero-voc-interior-latex

Location: Accent Wall Paint.

P: PT-4: SHERWIN WILLIAMS - PROMAR 200 LATEX TO MATCH GLIDDEN PROFESSIONAL, A1914 DUSTY MILLER, EGG SHELL.

https://www.sherwin-williams.com/homeowners/products/promar-200-interior-latex-eg-shel-finish-pl-9520017

Location: Accent Wall Paint.

P: PT-6: SHERWIN WILLIAMS - PROMAR 200 LATEX, 7103 WHITETAIL, FLAT.

https://www.sherwin-williams.com/homeowners/color/find-and-explore-colors/paint-colors-by-family/SW7103-whitetail

Location: Ceiling Soffit

P: PT-7: SHERWIN WILLIAMS - PROMAR 200 LATEX, SW 7006 EXTRA WHITE, EGG SHELL.

https://www.sherwin-williams.com/homeowners/products/SW7006-extra-white

Location: Wall Paint.

P: DF-1: SHERWIN WILLIAMS - SOLO 100% LATEX, 6141 SOFTER TAN, SEMI-GLOSS.

https://www.sherwin-williams.com/homeowners/color/find-and-explore-colors/paint-colors-by-family/SW6141-softer-tan

Location: Wall Paint.

BASES:

B: VB-1 (Cove Base): Burke, 4" High Vinyl Traditional Coved Base, #597 Mocha,

4" High, 1/8" Thick, Continuous Roll

B: VB-1 (Straight): Burke, 4" High Vinyl Traditional Straight Base, #597 Mocha,

TRANSITIONS:

T/A/A603: VT-01 :( Vinyl TO Carpet): Mannington, Equalizer Transition

T/D/A603: LVT to Vinyl Flooring: Butt Together Flooring Flush Transition

T/D1/A603: RESILIENT TO TILE: Schluter - Schiene Ae100

VT-01: (RESILIENT TO WALK-OFF MAT) :Mannington, Equalizer Transition

MISCELLANEOUS:

Threshold (Pemko 2727a) Set In Sealant Bed.

Conc-1 Sealed Concrete. Refer to section 09 91 23 part 2

Notes:

1 For Base Refer A801 General Notes Point L.

2 For Wall Tile of Waiting Toilet Refer Elevation C2, D2, E2, F2 of Sheet A401.

3 For wall Tile of Staff Toilet 145 and 146 Refer Elevations of Sheet A402.

4 For wall Tile of Waiting Toilet 103 Refer Elevations of Sheet A403.

5 For wall Tile of Lab Toilet 106 Refer Elevations of Sheet A403.

6 For wall Tile of OB Lab Toilet 126 Refer Elevations of Sheet A403.

7 For wall Tile of Changing Room 143 Refer Elevations of Sheet A411.

8 For Wall Tile of Ultrasound Toilet Refer Elevation of Sheet A412.