1.  Lifts must be installed on a concrete floor with a minimum thickness of 4”, a minimum compression strength of 3000 PSI and reinforced with steel mesh or bar as per commercial practice. If concrete pads are poured in place of a standard slab, it must be 4’ X 4’ X 1’ thick. Concrete must be aged 30 days prior to the installation of the lift(s). Concrete should be in good structural condition without cracks, visible damage, powdering or deterioration. To check concrete thickness, drill a hole in the installation area and measure the concrete depth.

- CONDITION OF CONCRETE					Good _____ Fair  _____ Poor  _____

- CRACKS OR SEAMS							Yes _____ No _____

(Columns must be 6” from any seam or crack)

- IS THERE A SLOPE								Yes _____ No _____

(Standard shims only accommodate a ½” slope - full contact shims are extra)

- DEPTH OF CONCRETE 	__________________

(Minimum 4” @ 3000 PSI)

- CEILING OBSTRUCTIONS							Yes _____ No _____

EXPLAIN__________________________________________________________________________________

- IS THERE ELECTRICITY?							Yes _____ No _____

- ARE THERE ANY OBSTRUCTIONS FOR UNLOADING?		Yes _____ No _____

2.  If the lift is to be installed on a second floor over a basement, the site must be evaluated and approved by a qualified structural engineer to determine if the structural integrity of the building is suitable for this type of installation.

- Lift is being installed on a second floor or over a basement.			Yes _____ No _____

- Evaluation and approval by structural engineer is attached.			Yes______No _____

PAGE 1 OF 2

3.  The lift is intended for indoor installation only. Is the site indoors?			Yes _____ No _____

4.  Installation area must allow sufficient clearances around lift.  Remember: overhead doors, tracks, heaters, gas, water pipes, floor drains, and in-ground piping may create interference with the lift and/or vehicles.

WILL INSTALLATION AREA ACCOMODATE LIFT?					Yes _____ No _____

Ceiling Height	_______ ft. _______ in.

Bay Width 	_______ ft. _______ in.

Bay Length	_______ ft. _______ in.

Verify that vehicle approach positioning has been considered.                                             Yes _____ No _____

Model # ________________	Brand  _______________________

IMPORTANT NOTICE:

It is the responsibility of the purchaser to inform the installers of any wiring and or plumbing under concrete prior to installation.  Lone Star Equipment and Tool Solutions and its affiliates assume NO LIABILITY for damage to plumbing and or electricity or wiring under concrete.  Installation does not include electrical or plumbing.

Concrete floor thickness for lift installation must be a minimum of 4”.  Concrete must have minimum compression strength of 3000 PSI and reinforced with steel mesh or bar as per commercial practice.  Concrete pads must be a minimum of 4’ x 4’ x 1’ thick.  Concrete pads must be reinforced with steel per commercial practice when on asphalt or dirt base.

Failure by purchaser to provide the recommended mounting surface could result in unsatisfactory lift performance, property damage and or personal injury.

The undersigned customer states and warrants that the site evaluation has been completed, that the above information is accurate, and the site meets all site evaluation criteria.  Customer agrees to all terms and conditions of the lift install.  Customer agrees that Lone Star Equipment and Tool Solutions and its affiliates will not be held liable for damages and will not withhold any monies due.  Lone Star Equipment and Tool Solutions agree to work to find any and every remedy, option and solution available.  Customer is aware that this will be at customer’s expense.

###### Distributor:  LONE STAR EQUIPMENT AND TOOL SOLUTIONS

Office: 	(210) 332–9268

Customer Name:	  ______________________________________________

###### Customer Signature:  ______________________________________________

###### Business Name:     	  ______________________________________________

###### Installation Address:  ______________________________________________

###### ______________________________________________

PAGE 2 OF 2