Material Safety Data Sheet

Identity:  SPRA-MULCH™ FRM

Fiber Reinforced Matrix

Product: Wood fiber mulch with a binder and poly fiber mixture.

Date Prepared: 07/27/2008

# SECTION I

Manufacturer/Supplier:	Fiber Marketing International, Inc.

11111 East Trent

Spokane, WA  99206

Tel:	509-927-4071	or	800-426-6002

Fax:  509-927-2330		Email: fibermarketing@comcast.net

# SECTION II - HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

Common Name:	Wood mulch	CAS #:	None

OSHA PEL-STEL:	10mg/m3	ACGIH TLV:	5 mg/m3 (respirable)

Polysaccharide & Crosslinked Binder..……….10%		CAS #................................................None

Poly Fiber  ……………..……………………………..  5%		CAS # …………………………………  25038-59-9

Basic Green 4 and Acetic Acid	<1%	CAS#.......................................... 569-64-2

# SECTION III - PHYSICAL AND CHEMICAL CHARACTERISTICS

Boiling Point (F)	Not Applicable	Solubility in Water	Insoluble, Dispersible

Vapor Density	Not Applicable	Specific Gravity (H2O=1)	0.025-0.075

Vapor Pressure (mm Hg)	Not Applicable	Appearance & Odor	Finely divided material

Melting Point	Not Applicable		Green in color

Slight wood odor

# SECTION IV – FIRE AND EXPLOSIVE HAZARD DATA

Flash Point	>550F

Flammability Limits LEL	Not Applicable	See Unusual Fire and Explosion Hazards below.

UEL	Not Applicable

Extinguishing Media		Water. CAUTION: Wet product will be very slippery.

Special Fire Fighting Procedures	None

# Unusual Fire and Explosion Hazards	A concentration of 40,000 mg/m3 is often used as the

# lower explosive limit for wood dusts in the presence of

# a flame or spark. Limit depends on moisture content

# and particle diameter.

# SECTION V – REACTIVITY DATA

Stability	Stable

Conditions to Avoid:	Material is organic, therefore combustible.

Avoid extreme heat and flame and contact with oxidizing agents.

Hazardous Decomposition:	May produce carbon monoxide,

carbon dioxide and aldehydes.

Hazardous Polymerization	Will not occur

Material Safety Data Sheet: SPRA-MULCH™ FRM (Cont.)

# SECTION VI – HEALTH HAZARD DATA

Material is primarily a Nuisance Dust

Routes of Entry:	Inhalation, skin contact, eye contact

EFFECT OF ACUTE EXPOSURE:

Overexposure:	May cause irritation to eyes and respiratory system.

Persons with respiratory problems should avoid breathing

dust.

Inhalation:	Can cause irritation to mucous membrane and

upper respiratory system.  Remove to fresh air.

Ingestion:	May cause vomiting or diarrhea.

Eyes:	In case of eye contact, flush thoroughly with clean water.

Skin:	In case of broken skin, wear gloves.

Wash dust from skin with soap and water.

Carcinogenicity Listings:

NTP:	not listed	IARC Monographs	not listed	OSHA:	not listed

# SECTION VII – PRECAUTIONS FOR SAFE HANDLING AND USE

Do Not store near heat or flame source. A sealed container will protect product.

Spill:	Sweep up excess material. Wet product will be very slippery.

Disposal:	Dispose in landfill or normal refuse in accordance

with Federal, State and local regulations

# SECTION VIII – CONTROL MEASURES

Respiratory Protection:	Use NIOSH approved respiratory masks.

Eye Protection:	Use goggles or eye glasses

Hand protection:	If sensitive, wear gloves

Other Protective clothing:	None

Ventilation:	normal ventilation

Work/Hygiene Practices:	Follow good hygiene practice.

Information presented herein has been compiled from sources considered dependable and is accurate and reliable to the best of our knowledge and belief, but it is not guaranteed to be so.  Nothing herein is to be construed as recommending any practice or any product in violation of any patents or in violation of any laws or regulations.  It is the user’s responsibility to determine the suitability of any material for a specific purpose and adopt necessary safety precautions.  We make no warranty as to results to be obtained in using any material and, since conditions of use are not under our control, we must necessarily disclaim all liability with respect to the use of any material supplied by us.