Sterile Processing Department: Log review

| Log | Question | Answers | Findings |
| --- | --- | --- | --- |
| Autoclave Steam Sterilizers | Daily Biological Indicator test | Bowie Dick test done before the sterilizer is used for the day |  |
|  | Individual load testing | Biological indicator test done on each load sterilized |  |
|  | Biological Indicator validation test | Test done to ensure the BI used in individual loads are valid |  |
|  | Leak test | Test done weekly |  |
|  | DART test | Test to be done daily before Bowie Dick test |  |
|  | Chamber cleaning | Performed bi-annually |  |
| Low temperature sterilizers (STERRAD, VPro) | Individual load testing | Biological indicator test |  |
|  | Biological Indicator validation test | Test done to ensure the BI used in individual loads are valid |  |
|  | Leak test (Not done in STERRAD) | Test done weekly |  |
| High level disinfectors | Individual load testing | Test strip indicators done on each load sterilized |  |
|  | Test strip validation | Test strips are checked for validation for use |  |
| Instrument / Cart washers | Washing process indicator test | Done on each washer daily |  |
| Ultrasonic washers | Cleaning indicator daily test | Test done daily to ensure cleaning efficacy |  |
| Decontamination sink; Enzymatic Dosing system (AccusInQ) | Daily testing | To include detergent dose and water temperature |  |
| Endoscope flushing system (Scope Buddy) | Daily testing performed? | Flow validation test is to be done each day before use |  |
| Eyewash station log | Weekly testing performed | Testing to be charted weekly | Current tracked via paper on specific eyewash station, by Facilities |
| Drying cabinet | No daily testing expectation; cleaning schedule should be in place | MIFUs identify weekly cleaning expectation on most drying cabinets |  |
| Room temperature & humidity logs | Daily testing | Charted daily | Current tracked electronically by Facilities |
| Positive / Negative air pressure logs | Daily testing | Charted daily | Current tracked electronically by Facilities |