# DIVISION XII

# HAZARDOUS MATERIALS

INCIDENTS

## STANDARD OPERATING

GUIDELINES

### TABLE OF CONTENTS

SUBJECT								    SECTION

Definitions									1

General									2

First Responder Responsibilities					        	3

Size Up									4

Plan of Action									5

Incident Command								6

Team Trailer Notification and Response					7

Passport/Accountability							8

Haz. Mat. Branch Officer – Responsibilities – H.M.B.O.			9

Haz. Mat. Safety Officer - Responsibilities – H.M.S.O			10

Science Information Officer – Responsibilities				11

Material Identification and Information					12

Decontamination Group Officer – Responsibilities				13

Decontamination Group – Responsibilities					14

Decontamination Procedures							15

Contaminated Victims							16

Mass Decontamination: Setting up the Emergency Decon Corridor		16.10

Haz. Mat. Medical Group – Officer Responsibilities	- H.M.M.O.		17

Medical Team Responsibilities 						18

Entry Group Officer – Responsibilities					19

TABLE OF CONTENTS CONTINUED

SUBJECT								  SECTION

Entry and Back – up Team Responsibilities					20

Communications								21

Site Clean up									22

Post Incident Critique								23

Documentation								24

Reimbursements								25

Team Membership Requirements						26

Team Training Requirements							27

### TABLE OF CONTENTS CONTINUED

SUBJECT								   Appendix

Minimum Equipment List for First Responsers				A

Hazardous Materials Organizational Chart					B

Incident Command Chart							C

Incident Planning Chart							D

Haz. Mat. Group Chart							E

Safety Group Chart								F

Product Data Form								G

Decontamination Group Chart						H

Template – Gross Decon							I

Template – Dry Decon							J

Template – Dilution Decon							K

Entry Data Form								L

Rehab Form									M

Entry Team Officer Chart							N

Training Attendence Form							O

Level A & B Encapsulated Donning Procedure				P

Level B Non-encapsulated Donning Procedure				Q

Division 12 State-Wide Box Card						R

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  1

Standard Operating Guidelines				        		Page:  1 of 2

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:   DEFINITIONS

Contaminant.  A hazardous material that physically remains on or in people, animals, the environment, or equipment, thereby creating a continuous risk of direct injury or a risk of exposure outside the hot zone.

Contamination.  The process of transferring a hazardous material from its source to people, animals, environment, or equipment, which may act as a carrier.

Control Zones.  The designation of areas at a hazardous materials incident based upon safety and the degree of hazard.

Hot Zone  (Restricted Zone).  The area immediately surrounding  the incident,

extending far enough to prevent adverse effects from hazardous materials releases to personnel outside the zone.

Warm Zone  (Limited Access Zone).  The area where personnel and equipment decontamination and hot zone support take place.  It includes control points for the access corridor and thus assist in reducing the spread of contamination.

Cold Zone  (Support Zone).  The area that contains the command post and other 	support functions deemed necessary to control the incident.

Decontamination (Contamination Reduction).  The physical and/or chemical process of reducing and preventing the spread of contamination from persons and equipment used at a hazardous materials incident.

Hazard/Hazardous.  Capable of posing an unreasonable risk to air, water, soil quality and to plants or wildlife.

Hazardous Material.   A material that poses an unreasonable risk to the health and safety of operating or emergency personnel, the public, and/or the environment if it is not properly controlled during, handling, storage, manufacturing, processing, packaging, use, disposal, or transportation.

Haz Mat Safety Officer (H.M.S.O.).  The individual assigned to oversee safety in Haz Mat Division.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  1

Standard Operating Guidelines				        		Page:  2 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:   DEFINITIONS

Hazardous Incident Team (TEAM).  Specially trained members of the participating Fire Departments who have met the requirements as specified in Sections 14 and 15 of these Standard Operating Guidelines, and have been approved by their Fire Chief to participate in hazardous materials incidents in specialized functions.

Hazardous Material Branch.  That function within the overall incident command system that deals with the mitigation of a hazardous materials incident.  It is directed by the Haz Mat Branch Officer (H.M.B.O.) and principally the technical aspects of the incident.

Haz Maz Branch Officer (H.M.B.O.).  The person responsible for the management of the hazardous material division.

Incident Commander (IC).  The person responsible for all decisions relating to the management of the incident.  The incident commander is in charge of the incident site.  This is equivalent to the on-scene incident commander as defined by 29 CFR 1910.120.

Incident Command System (ICS).   An organized system of roles, responsibilities, and Standard Operating Guidelines used to manage and direct emergency operations.

Operations.  Directs, coordinates, and controls all tactical activities and functions necessary to carry out the objectives of Command.  Reports directly to the fire department incident commander.

Monitoring Equipment.  Instruments and devices used to identify and quantify contaminants.

Reconnaissance (Recon).  A preliminary survey of the incident by personnel to identify and evaluate the specific mechanics of the incident.

Division.  A geographical or specific function assigned  at a incident, to which the incident commander assigns a person to reduce the span of control.

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 2

Standard Operating Guidelines				          		Page:  1 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  GENERAL

Purpose:  The MABAS Division 12 Hazardous Incident Team and the Regional Hazardous Incident Response Team have established these Standard Operating Guidelines as a basis for operating at hazardous materials incidents, either singularly or jointly.  Members from either or both teams may respond to a request for assistance from a stricken fire department or fire protection district when that jurisdiction cannot field the necessary resources to handle the emergency.  Response by either or both teams shall not relieve the stricken jurisdiction of incident command.

2.01	Under certain circumstances involving military transport, military aircraft, the     Department of Defense, the Department of Energy or other federal agencies, incident command may be relinquished by the stricken jurisdiction and passed to federal authorities.

2.02	It shall be the responsibility of each participating fire department and fire protection 	district to meet the required minimums for equipment for first responder use.  The 	required minimum list of equipment is in Appendix A.  It shall also be the responsibility              of each participating fire department and fire protection district to meet the minimum                   training requirements for first responders as specified in the OSHA and NFPA Standards.

  - Emergency response t hazardous materials incidents must be a joint and cooperative         effort. Member organizations, local, county, state and federal agencies, private industry

and any other resources that are deemed necessary must cooperate ti attain athe goal of the safe resolution to an emergency response.

2.04	All participating municipalities, cities, villages, and fire protection districts shall have a spiller pay ordinance.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  2

Standard Operating Guidelines				          		Page:  2  of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  GENERAL

2.05	The fire departments and fire protection districts currently participating in the MABAS Division 12 Hazardous Incident Team, the Regional Hazardous Incident Response Team, or both, and have approved the Standard Operating Guidelines are:

Addison Fire Protection District

Batavia Fire Department

Bensenville Fire Department

Bloomingdale Fire Protection District

Carol Stream Fire Protection District

Elmhurst Fire Department

Glen Ellyn Volunteer Fire Department

Glenside Fire Protection District

Hanover Park Fire Protection District

Itasca Fire Protection District

Lombard Fire Department

Oak Brook Fire Department

Oak Brook Terrace Fire Protection District

Roselle Fire Protection District

Villa Park Fire Department

Warrenville Fire Protection District

West Chicago Fire Protection District

Wheaton Fire Department

Winfield Fire Protection District

Wood Dale Fire Protection District

York Center Fire Protection District

2.06	These Standard Operating Guidelines are approved as of :  M/D/Y

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  3

Standard Operating Guidelines				         		Page:  1 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  FIRST RESPONDER RESPONSIBILITIES

Purpose:  The overall objective during any hazardous material incident is to stabilize the incident to protect health, property, and the environment.  The involvement of the fire department having jurisdiction continues until the conclusion of the incident.  Extreme caution and attention must be the first consideration of all responding personnel.  To the extent of its manpower, equipment, and level of training, the stricken department should, prior to the notification and arrival of the TEAM, follow these procedures:

3.01	Response to any reported hazardous material incident such as a leak, spill, fire, or potential container failure shall be made with a defensive posture.

3.02  Responding companies should approach from an upwind and uphill direction.  Alternate cross wind routes should be considered if an upwind approach is not possible.

3.03	Stop and assess the incident from a distance upwind and upgrade.  Avoid driving into vapor clouds or smoke.

3.04	Treat all containers as hazardous materials until proven otherwise.

3.05	Avoid the use of water until the material or mixture is determined not to be

water reactive and/or to avoid increased runoff.  Any water runoff that may be contaminated should be contained for later disposal.

3.06   Decide if a close recon can be done safely by the fire department personnel on the scene. 	 If so:

3.06.01 	Commit two men to size up the incident.

3.06.02 	Wear full protective clothing including SCBA.

3.06.03 	Approach from upwind and upgrade.

3.06.04 	Avoid contact with the hazardous material (s) and/or vapor(s).

3.06.05 	Monitor the atmosphere, assess the problem quickly, and return.

3.06.06 	When it is possible to perform an immediate rescue, it should be accomplished with an awareness of the risk and with a minimum number of personnel.

3.06.07	In situations where a rescue is required, the protection and safety of rescuers and

victims is paramount.  In such cases all means and methods to accomplish this should be considered.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  3

Standard Operating Guidelines				        		Page:  2 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:   FIRST RESPONDER RESPONSIBILITIES

3.07	Identify and determine the hazards of the material(s) involved, and the guidelines for the safe handling of it, through reference sources.

3.08	Decide on a plan of action.

3.09	If any area is beyond the capabilities of the stricken department, the TEAM should be notified.

3.10  Prior to the arrival of the TEAM, command should be established and the site should be isolated and secured to prevent the spread of the hazardous material(s) or the accidental contamination of personnel.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  4

Standard Operating Guidelines				          		Page:  1 of  1

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:   SIZE UP

Purpose:  To provide a systematic evaluation  of  the critical factors that lead to the development of a plan of action.  For developing a plan of action the following factors should be considered.

4.01	Type of incident:

4.01.01	Fire

4.01.02	Leak

4.01.03	Release

4.01.04	Spill

4.02	Harmful nature of the material(s).

4.03	Type and condition of the container(s).

4.04	Conditions:

4.04.01	Location

4.04.02	Time

4.04.03	Weather

4.05	Exposures:

4.05.01	Life

4.05.02	Environment

4.05.03	Fire Department Equipment

4.05.04	Property

4.06	Resources

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 5

Standard Operating Guidelines				         		Page:  1 of  1

Effective Date:   June 1, 1994

Revised:  March 11, 2008

Section Title:  PLAN OF ACTION

Purpose:  After the size up, a plan of action should be established be it offensive, defensive, or a combination of both.  The safety of all response personnel must be considered in this plan.

- Operations

5.01	Evaluation of the scene and size up

5.02	Secure the scene

5.03	Perform rescue & gross decon

5.04	Contain, divert, dike

5.05	Protect exposures

  - Call for additional

5.1	Technician

5.1.1	Evaluation of the scene and size up

5.1.2	Monitoring

    - Confinement, contatinment

    - Rescue endangered persons that were not assessable without chemical protective clothing.

    - Prevent container failure.

    - Extinguish ignited materials.

    - Request additional resourses.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  6

Standard Operating Guidelines				          		Page:  1 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  INCIDENT COMMAND

Purpose:  OSHA 1910.120 requires that any hazardous incident operations be conducted with an Incident Command System in place.

6.01  The Incident Commander shall be from the stricken community in which the incident has

occurred.

6.02	The Incident Commander shall make a size up of the incident.

6.03	Through the use of resources and references the Incident Commander shall determine and

establish Hot, Warm, and Cold Zones.

6.04	A Command Post shall be established and shall be:

6.04.01	located in the Cold Zone, uphill, and upwind.

6.04.02	announced, and identified by a green light and/or an orange flag.

6.04.03	the location for representatives of support agencies to report to.

6.05	The Incident Commander shall be identified by a vest.

6.06	After the size-up the Incident Commander shall develop and initiate a plan of action.

6.07	The plan of action shall be reviewed, revised and updated as needed.

6.08	The Incident Commander shall assign a person to be in charge of the Hazardous Material

Division.  This person shall meet all competencies for the responder at the level of operations

being performed.

6.09  The Incident Commander shall assign a person as a Hazardous Material Safety Officer                           (H.M.S.O.). This person shall meet all competencies for the responder at the level of operations

being performed.

6.10  Dependant on the complexity and/or the size of the incident, the incident commander

should consider assigning a Staging Group and a Public Information Officer.

6.11	The Incident Commander shall assign two (2) ALS ambulances, each staffed with two (2) paramedics,  to the Hazardous Material Division.  One ambulance shall remain on the scene at all times.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  6

Standard Operating Guidelines				          		Page:  2 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  INCIDENT COMMAND

6.12	The Incident Commander shall be the individual that releases the team.

6.13	The Incident Commander is responsible to cause a post incident critique.

6.14	Appendices C and D are to be used for documentation.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  7

Standard Operating Guidelines				          		Page:	1 of  2

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  TEAM TRAILER  NOTIFICATION & RESPONSE

Purpose: To provide a systematic and organized response of departments, personnel, and equipment to the scene of a hazardous materials incident.

7.01	Requests for additional resources shall be initiated by the Incident Commander through the

department's dispatch center in accordance with the department's hazardous materials box card.

7.02	The Incident Commander from the stricken jurisdiction shall determine the response needed:

7.02.01	Response of a TEAM is not required. (The stricken jurisdiction has sufficient

manpower and equipment to mitigate the incident).

7.02.02	Response of a TEAM trailer is required for additional equipment.

7.02.03	Response of a TEAM is required. (This response includes both a trailer & manpower).

7.03	The dispatch center will then notify the appropriate MABAS dispatch center of their request.  	The following information from the stricken jurisdiction must be given when the alarm is requested:

7.03.01	The name of the requesting department.

7.03.02	The mutual aid box alarm requested.

7.03.03	The level of the alarm requested.

7.03.04	The location of the incident.

7.03.05	The command post location.

7.03.06	The staging area and direction of approach, if needed.

7.04	It shall be the responsibility of all requested department dispatch centers, upon receipt of the 	box alarm, to notify the MABAS dispatch center that the alarm has been received.

  - It shall be the responsibility of the responding departments to notify the MABAS dispatch center of the equipment and the number of personnel responding.

7.06	When TEAM members respond to an incident, they shall respond in a fire department vehicle and bring the following equipment, if available:

7.06.01	Personal protective clothing (structural firefighting clothing).

7.06.02	Air pack and extra bottle (1 hour preferred).

    - Portable radio with fire ground channels

    - Passports

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  7

Standard Operating Guidelines				        		Page:  2 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:   TEAM TRAILER  NOTIFICATION & RESPONSE

7.07	Response of  TEAM trailers will be governed by the following guidelines:

7.07.01	The trailers will respond, obeying all traffic regulations, without emergency lights or sirens.  Once near the scene, the driver may turn the emergency lights on to aid in vehicle identification and provide safety to the personnel and vehicles around the unit.

7.07.02	The driver of the trailer should report to staging.

7.07.03	The driver will be in charge of distribution and documentation of all equipment that is used from the trailer.

7.07.04	The driver shall supply power to the trailer.

7.07.05	The driver shall remain on the scene until the trailer is released by the Incident Commander or replaced by relief personnel in the event of an extended incident.

  - Upon arrival all personnel shall report to the staging area.  Passports shall be given to the

Staging	Officer.  Responding units shall turn off their emergency lights while in staging.

7.09 Statewide Haz-Mat Response: The MABAS Division 12 Haz Mat Team is a Level A Team. If the StateWide plan is activated, all departments involved will follow the MABAS Statewide protocol. Equipment and personnel requirements depend upon a multitude of variables inclucing but not limited to; location of the incident, anticipated deployment period, support services required, on scene resources, and resources required to accompany our team, i.e. Decon trailer and other equipment/vehicles needed. Point of departure Addison Fire Station #3.

7.09	State Wide Response will be governed by the following guidelines:

7.09.01	Red Center will request the team through DuComm.  The team will consist of a minimum of ten (10) Technicians, one of which will be a coordinator.

7.09.02	DuComm will notify the primary departments listed on the box card (Appendix R).  Departments requested shall call DuComm with an availability status with in one half hour.  If a primary department is not available they will contact the next department on the alternate list.

7.09.03	Responding personnel shall be prepared to be self-sufficient for a minimum of one (1) week, including but not limited to bedding, toiletries, uniforms and identification.

    - All responding personnel and equipment shall assemble within two (2) hours of notification at Addison Station #3.

    - All non-essential personal and departmental vehicles shall be removed from the Addison parking area within twenty-four (24) hours.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  8

Standard Operating Guidelines				        		Page:  1 of  1

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:   IDENTIFICATION/ACCOUNTABILITY OF TEAM MEMBERS

Purpose:  To work in conjunction of the Incident Command System.  To identify individual members of a team and their assignment.  To account for the assignments of teams and units at an emergency incident.

8.01	The fire departments shall use the Passport Accountability System as adopted by the

MABAS Executive Board.

8.02	Members of the Haz Mat Team shall be issued two Passport System name tags.  The tags

shall show the rank, name, and department (using the four letter abbreviation) in accordance with MABAS guidelines.

8.03	TEAM members who are technicians shall be issued red name tags.

8.04	TEAM members who are trained at the first responder operations level shall be issued

green name tags.

8.05	The name tags shall be used in accordance with MABAS guidelines and the Passport

Accountability System.

8.06	TEAM members who do not have their name tags at the incident shall not be allowed to

participate.

HAZARDOUS MATERIALS INCIDENT		    	 		Section Number:  9

Standard Operating Guidelines				         		 Page:	1 of  1

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  HAZ MAT BRANCH OFFICER (H.M.B.O.) - RESPONSIBILITIES

Purpose:  The H.M.B.O. principally deals with technical aspects and mitigation of the hazardous material incident.

9.01	The H.M.B.O. shall  report to the Incident Commander or Operations Chief depending on the size of the incident.

9.02	The H.M.B.O. shall be a Hazardous Materials Technician.

9.03  The H.M.B.O. shall be identified with a vest.

9.04	The H.M.B.O. division/group shall be responsible for all hazardous materials operations.

9.05	The H.M.B.O. shall establish the following divisions/groups based on needs:

9.05.01	Resource division/group

9.05.02	Decon division/group

9.05.03	Entry Team division/group

    - Haz Mat Medical division/group

    - Science/Information division/group

    - Monitoring division/group

    - Safety division/group

9.06	The H.M.B.O. shall brief the division/group of the plan of action.

9.07	The H.M.B.O. shall assist the Incident Commander with

re-evaluation of the control zones.

9.08	The H.M.B.O. shall assign personnel to monitor vapors and gases.

9.09	The H.M.B.O. shall determine the need for additional resources and notify Incident Command.

9.10	The H.M.B.O. shall continually monitor the hazardous materials operations.

9.11   Appendix E is to be used for documentation.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  10

Standard Operating Guidelines				          		Page:  1 of  2

Effective Date:   June 1, 1994

Revised: March 11, 2008

Section Title:  HAZ MAT SAFETY OFFICER (H.M.S.O.) - RESPONSIBILITIES

Purpose:  To provide for the safety of all personnel and civilians during a hazardous materials incident.

10.01	The H.M.S.O. shall report to the HAZ MAT BRANCH OFFICER (H.M.B.O.).

10.02	The H.M.S.O. shall be a Hazardous Materials Technician.

10.03	The H.M.S.O. shall be identified with a vest.

10.04	The H.M.S.O. is responsible for site safety and completing the written Site

Safety Plan. (The Site Safety Plan to be used. (Appendices B thru N)

10.05	The H.M.S.O. shall ensure that a  pre-entry briefing is conducted for

entry and decon personnel.

10.06	The H.M.S.O. shall ensure that an Entry Data Sheet is kept on each Team

Member entering the Hot Zone and in Decon.

10.07	The H.M.S.O. shall ensure that copies of the Entry Data Sheets are

forwarded to the departments of each team member as applicable.

  - Appendix F is to be used for documentation.

  - On scene emergencies:

10.09.1  In the event of injury ofr imminent danger the H.M.S.O. shall, at his discretion, halt                 all operations.

10.09.2 The I.C. and H.M.B.O. shall be notified immediately of the actions taken and the                      reason for operations being halted.

10.09.3 The H.M.S.O. shall give the I.C. and the H.M.B.O. recommendations for resuming or 	  terminating operations.

10.09.4  Non-threatening safety concerns may be pointed out to the H.M.B.O. and the

Applicable division commanders to be corrected immediately without cessation of operations. If the hazards are not corrected oprerations shall be stopped until all safety hazards have been corrected.

HAZARDOUS MATERIALS INCIDENTS				Section Number:  10

Standard Operating Guidelines						Page:  2  of  2

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  HAZ MAT SAFETY OFFICER – (H.M.S.O.) – RESPONSIBILITIES.

Purpose: To provide for the safety of all personnel and civilians during a hazardous materials incident.

10.09.5 The Haz Mat division may require more than one Safety officer if the incident covers               a large area, is technically complex or if several simultaneous Level A entries are                        taking place. The H.M.B.O. will assign additional safety officers as the situatin                        dictates.

10.09.6  The following personnel may halt operatins if safety concerns arise:

10.09.6.1         Incident Commander (I.C.)

10.09.6.2         H.M.B.O.

10.09.6.3         H.M.S.O.

10.09.6.4        Entry Team Members

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 11

Standard Operating Guidelines				         		 Page:	1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title: SCIENCE INFORMATION OFFICER- RESPONSIBILITIES

Purpose:  The Science Information Officer shall be responsible for researching chemical data for consultation with the H.M.B.O. He shall also make this data available to the

H.M.S.O., Haz Mat Medical Officer (H.M.M.O.), Decon,  and  Entry Officer.

11.01	The Science Information Officer shall report to the H.M.B.O.

11.02	The Science Information Officer shall be a Hazardous Materials Technician.

11.03	The Science Information Officer shall be identified by a vest.

11.04	The Science Information Officer shall direct the resource personnel in the identification of the material(s) involved.

11.05	The Science Information Officer should make contact with CHEMTREC, the product 		manufacturer, the shipper and consignee, or the fixed facility personnel as needed.

11.06	The Science Information Officer shall obtain additional resources and materials as requested by the Hazardous Materials Officer and/or Incident Commander.

11.07	Appendix G is to be used for documentation.

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 12

Standard Operating Guidelines				         		Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:	MATERIAL IDENTIFICATION AND INFORMATION

Purpose:  To obtain proper information on the hazards of the chemical(s) involved.

12.01	A minimum of three reference sources shall be used to research all chemical

information.

12.02	A minimum of two team members shall check all pertinent data in each reference source

to eliminate errors.

12.03	Material Safety Data Sheets shall be checked against at least two different reference

sources.

12.04	Computer information shall be checked against at least two different reference sources.

12.05	Information received from any chemical manufacturer, shipper, or user shall be checked

against at least two different reference sources.

12.06	Identification of materials in mobile vehicles shall begin by identifying the container shapes                and types.  Efforts should be made to observe the Department of Transportation placards                     displayed on the exterior of the vehicle.  Observation of the placards and the containers should             be made with binoculars from as far a distance as possible.

12.07	If readily available, the shipping papers will provide the most accurate material

information.

12.10	When appropriate, notify CHEMTREC to obtain their assistance in identification and

notification of the shipper, manufacturer, consignee, and carrier.

12.11	Identification of material at fixed sites should include the use of knowledgeable facility

personnel.  Facility personnel should make available the Material Safety Data Sheets to the Science Information Officer if possible.  Personnel should be observant of the NFPA 704 Identification System to help identify possible hazards.

12.12	Appendix  G is to be used.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  13

Standard Operating Guidelines				         	 	Page:  1 of  2

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  DECONTAMINATION GROUP - OFFICER RESPONSIBILITIES

Purpose:   In order to protect first responders, TEAM members, other emergency responders and civilians, a process must be in place to minimize the potentially harmful effect presented by hazardous materials.  Decontamination is the process that will achieve this objective.

13.01	The Decon Officer shall report to he H.M.B.O.

13.02 	The Decon Officer shall be a Hazardous Materials Technician.

13.03	The Decon Officer shall be identified by a vest.

13.04	Decontamination shall be established and be functional prior to any personnel entering the identified Hot Zone.

13.05	The Decon Officer should have assigned 4-6 personnel for decon operations.

13.06	The Decon Officer shall be responsible for the supervision of decon procedures.

13.07	The decon corridor shall be located in the Warm Zone, upwind and

upgrade.

13.08	The entrance and egress points of the decon corridor shall be clearly marked.

13.09	The Decon Officer, through the Science Information Officer, shall identify the material(s) involved and the method and extent of decontamination required.

13.10	The normal decon solution shall be soap and water, unless otherwise identified by the Science Information Officer.

13.11	The Decon Officer shall confer with the H.M.S.O. on the appropriateness of the decon procedures to be instituted.

13.12	Decontamination personnel shall wear protective clothing equal to, or one level below, the level worn by the Entry Team, as outlined by the EPA.  Use of fire suppression protective clothing should be avoided.

13.13	The Decon Officer shall ensure that water and solution runoff is controlled to prevent

environmental pollution.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  13

Standard Operating Guidelines				         	 	Page:  2 of  2

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  DECONTAMINATION GROUP- OFFICER RESPONSIBILITIES

13.14	All contaminated items shall be isolated and secured.

  - Appendices  H, I, J, K, L are to be used for reference & documentaion.

  - For a mass decon set up see section 16.10

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 14

Standard Operating Guidelines				         		Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  DECONTAMINATION GROUP RESPONSIBILITIES

Purpose:  To provide for the safety of decontamination personnel and provide proper decontamination for the entry and decon personnel.

14.01	The decontamination group shall fall under the control of the Decontamination Officer.

14.02	Decontamination group members shall meet all the competencies for the responder at the

level of decon operations being performed.

14.03	Decontamination group members shall receive a pre-entry check of vitals by the medical

evaluation team prior to donning chemical protective clothing.

14.04	Decontamination group members shall hydrate prior to donning chemical protective

clothing.

14.05	Decontamination group members shall place personal belongings in a plastic bag then

give the bag to the Decontamination Officer.

14.06	Decontamination group members shall use SCBA of equal capacity or be on supplied air

systems.

14.07	Decontamination group members shall wear protective clothing equal to, or one level below,                the level worn by the entry team, as outlined by the EPA.  Use of fire suppression clothing                   should be avoided.

14.08	The decontamination group shall follow the decon procedures as outlined by Section 15

and the Decontamination Officer.

14.09	Contamination reduction of the decontamination team members shall be determined on

their level of involvement in the decontamination corridor.

  - The decontamination team shall proceed to Rehab for medical evaluation.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  15

Standard Operating Guidelines				          		Page:  1 of  2

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:	DECONTAMINATION PROCEDURES

Purpose:  To set forth procedures and guidelines for the reduction of contamination on victims, personnel, and equipment.  As the level, scope, and involvement of each hazardous materials incident may be different, personnel should adapt their decon procedures to meet these needs.

15.01	The type of decon chosen may be:

15.01.01	Gross decon for contaminated victims.

15.01.02	Isolation/disposal or dry decon to minimize water runoff.

15.01.03	Two pool dilution decon for a major hazard (Appendix K).

15.02	Gross decon is for the quick removal of contaminates from a limited # of victim, firefighters               or team members so life saving medical care can be given.  Procedures for gross decon are:

15.02.01	Place a non - permeable covering on the ground to serve as the decon area.

15.02.02	Restrict access to essential personnel only.

15.02.03	Remove all contaminated clothing and personal property from the victim.

15.02.04	Place clothing and personal property in proper containers.

15.02.05	Brush all dry contaminants from the victims body.  Avoid the use of water on 			dry contaminants.

15.02.06	Flush all liquid contamination from the victims body.  Attention should be

given to the eyes, facial and skin fold areas.

15.02.07	Transport the victim to a medical facility for treatment.

15.02.08	The clothing and personal articles are to remain in the decon area for further

decontamination or disposal.

15.02.09	Template - Appendix I

15.03	Isolation/disposal or dry decon may be used where the hazard is light, control of water

runoff is not possible, or the use of water is contraindicated for the material involved. The procedures for dry decon are:

15.03.01	Place a non - permeable covering on the ground to serve as the decon area.

15.03.02	Restrict access to essential personnel only.

15.03.03	Decon personnel are to be assigned as "dirty man" or "clean man".

15.03.03.01	The "dirty man" shall touch the outside of the protective clothing only.

15.03.03.02	The "clean man" shall touch the inside of the protective clothing only.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  15 Standard Operating Guidelines	                                                                      Page:  2 of  2

Effective Date:  June 1, 1994

Revised:  March 11, 2008

Section Title:  DECONTAMINATION PROCEDURES

15.03.04	Personnel entering  decon shall have their protective clothing opened by the "dirty man".  The clothing shall be removed in such a way that it will be turned inside out during removal.

15.03.05	The "clean man" shall assist in the removal of the protective clothing, touching only the inside of the protective clothing, or the person doffing the suit.

15.03.06	The procedure shall continue until the suit is at the feet of the person doffing it and he can step free of the suit.

15.03.07	The person doffing the suit shall be assisted by the "clean man" away from the suit for removal of the SCBA.

15.03.08	The "dirty man" shall place the suit in the proper container for further

decontamination or disposal.

15.03.09	Template - Appendix J

15.04	The dilution decon method  shall be used in all incidents the Decon Officer considers appropriate.  Dilution decon may include any or all of the following steps, as determined by the decon Officer considering the severity of the hazard:

15.04.01	Place Dike Tubing, hose or raised barrier on the ground in a rectangular

dimension then place a non - permeable covering on the ground to serve as the decon area. A salvage cover should be used on rough surfaces such as gravel.

15.04.02	Personnel entering the decon corridor shall deposit their tools in the

appropriate area or container.

15.04.03	Personnel shall step in the for rinse, wash and rinse sequence.

15.04.04	Personnel shall step from the first pool into the second pool for further

contamination reduction.

15.04.05	After the dilution of the contaminate is completed personnel shall proceed through the decon process for suit doffing.

15.04.06	Using the "clean man"/"dirty man" procedure, decon personnel shall assist in doffing the protective clothing.

15.04.07	When personnel are clear of the hazard, remove the SCBA.

15.04.08	Removal of the personal clothing and personal showering should be done with the consideration of the privacy of the personnel as well as the severity of the contamination.

15.04.09	Personnel completing the decon process shall proceed to Rehab for medical evaluation.

    - Template - Appendix K

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 16

Standard Operating Guidelines	  					Page:  1 of  4

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  CONTAMINATED VICTIMS

Purpose:  A victim who is poisoned or chemically contaminated must be handled properly.  The safety of response personnel and the optimum care of the victim should be the end result.

16.01	Before handling a victim three determinations should be made:

16.01.01  That the victim is poisoned or contaminated.

16.01.02  That the victim is alive or has a chance of surviving.

16.01.03  That the product is identified specifically or generally.

16.02	Response personnel involved in the rescue attempt shall wear proper protective

equipment. The victim should be moved from the area of contamination to the decontamination area.  The victim may walk to the decon on his own or with assistance.  If the victim has to be carried, a metal backboard or stokes basket should be used when available.

16.03	Victims who have been exposed to contaminants should be given, at minimum, a gross field decontamination.  Clothing should be removed and placed in plastic bags for proper disposal.  If possible, this should be done in the decontamination area.

16.04	Medical personnel shall wear protective clothing to prevent or limit contamination during the care of the victim.

16.05	The treatment area shall be located just inside the warm zone to prevent or limit the spread of contamination.  The victim should be protected from hot and cold conditions.  Disposable clothing or blankets should be furnished to the victim.

16.06	All bodily fluids shall be considered contaminated. When applicable, bodily fluids should

be collected and brought to the hospital for laboratory examination and proper disposal.

16.07	When communicating with the receiving hospital, medical personnel shall advise of the

contamination and the possible need for further decontamination.  The hospital should give directions for routing the patient to an area in the hospital where further decontamination can take place while limiting the exposure.

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 16

Standard Operating Guidelines	  					Page:  2 of  4

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  CONTAMINATED VICTIMS

16.08 	In the event of radiological contamination, the only field decontamination of the victim should be the removal of clothing and the brushing of contaminants from the skin.   Clothing shall be isolated for proper disposal.  The use of water  shall be avoided.  Take care to prevent internal contamination of the victim.  Further external decontamination is best accomplished at a medical facility.

16.09	In the event a TEAM member is contaminated and transported to a medical facility, the

H.M.M.O. shall notify the H.M.B.O.  The H.M.B.O., in turn, shall notify Command.  The Incident Commander shall be responsible for contacting the team member's department to ensure that the baseline medical record is sent to the receiving hospital.

16.10  	           MASS DECONTAMINATION: SETTING UP THE EMERGENCY                                       DECON CORRIDOR

    - Evacuate victims upwind, upgrade, and upstream of the hazard.

    - Isolate and extablish control of the victims. Decon should be immediate.

    - Communicate with victims in order to perpare for decontamination. Segregate symptomatic victims from the asymptomatic victims and male from female. Take into consideration families, small children, the  elderly, hadicapped persons and uncooperative persons.

    - Position the first arriving engine with the pump panel facing a hydrant. Start to decontaminate as soon as possible. Speed is of the utmost importance if lives are at risk. Consider placing the deck gun or fog line in use before additional units arrive.

    - Have the second arriving engine position 12 to 16 feet apart from the first engine facing the opposite direction. The pump panels will be on the aspect and all windows are closed. The basic mass decon coorridor is between the two engines.

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 16

Standard Operating Guidelines	  					Page:  3 of  4

Effective Date:  June 1, 1994

Revised: March 11, 2008

- Section Title:  CONTAMINATED VICTIMS

    - MASS DECONTAMINATION: SETTING UP THE EMERGENCY DECON CORRIDOR

- The first engine will provide a water supply to the second. Fog nozzles are                                     attached to the 2 ½” discharges opposite the pump panels on both engines.

    - Pump pressures are set at 30 psi and fog nozzles are adjusted on wide fog. The fog patterns should meet in the center.

    - Direct symptomatic victims to remove clothing down to their undergarments. If    available provide clear plastic bags for clothing and valuables

    - Have exposed victims walk through the fog streams for a 1- to 3- minuite wash.

    - Provide modesty wear for cover if available.

    - Direct victims to safe refuge or medical triage or treatment areas.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  16

- Standard Operating Guidelines						Page:  4 of  4

- Effective Date:  June 1, 1994

- Revised: March 11, 2008

Section Tittle:	CONTAMINATED VICTIMS

    - MASS DECON: ADDITIONAL TACTICAL CONSIDERATIONS

- As additional resources arrive, basic mass decon can be expaned adding another                    	engine company. The additional corridor may separate male and female victims or

- symtomatic from contaminated victims. Further triage can remove victims who                             were in the area but are not contaminated. These individuals should be segregated                         and held for further evaluation.

    - Approximately 80 percent of the contamination is removed with clothing. If the    contaminant is radiological or biological, the victim should be wet down before    removing the clothes. Wet chothing prevents reaerosolization as clothing is           removed.

    - Soap and water is recommended for decontaminating warfare agents. In some       cases, bleach and water will cause more harm to the victim. Mustard agents is an example; bleach and water causes further tissue damage. When capabilities and resourses allow, employ soap and water as the best decontaminate of choice.

    - In an actual biological attack, response to the agent may come hours or days later. Most likely this response will occur remote from the point of contamination and will not result in a mass decon incident. However, there have been many occasions where a group believes they have been contaminated but show no signs of exposure. In this case, psychological decon may be performed. Victims who initially  are not symptomatic or contaminated can be decontaminated at another location. Use of a fixed facility or the decon trailer offers privacy and peace of mind.

    - When possible, attempt to capture runoff. Realistically, however, when confronted with hundreds of patients, runoff is a secondary concern. However, uncontrolled runoff may result. Environmental officials and municipal authourities must be notified to assist with runoff control and proper disposal of contaminated materials.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  17

Standard Operating Guidelines	  				Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:	HAZ MAT MEDICAL GROUP - OFFICER RESPONSIBILITIES                           (H.M.M.O.)

Purpose: To ensure that the physical and mental condition of all fire department personnel operating on the scene of a hazardous materials incident do not deteriorate to the point which affects their own or any other person's safety.

17.01	The HAZMAT Medical Officer (H.M.M.O.) shall report to the H.M.B.O.(Haz Mat Branch Officer).

  - The (H.M.M.O.)  shall be a Paramedic and First Responder Operational

.

17.03	The (H.M.M.O.) shall be identified with a vest.

17.04	The (H.M.M.O.) shall ensure that two ambulances have been assigned to the Hazardous Materials Incident by the Incident Commander.

17.05	The (H.M.M.O.) shall determine the need for additional personnel and resources.

17.06	The (H.M.M.O.) shall obtain information from the Science Information Officer about any immediate and long term health effects to contaminated personnel.  All unidentified hazardous materials are to be considered deadly until proven otherwise.

17.07	If additional information is needed about the chemical, a resource/associate hospital shall be contacted for assistance.

17.08	The (H.M.M.O.) shall ensure that vital signs are taken and documented on the following personnel.

17.08.01	Entry Team - before and after entering the Hot Zone.

17.08.02	Back-Up Entry Team - before and after entering the Hot Zone.

17.08.03	Decon Team - before and after decon.

17.09	The (H.M.M.O.) shall forward the completed Entry Data Sheets and rehab sheets for all entry and decon personnel to the H.M.S.O. (Haz Mat Safety Officer).  If a medical condition exists or an exposure has occurred,  an EMS run report shall be used.

17.10	Medical attention shall be given to personnel exhibiting signs and symptoms from exposure to the chemical(s) and/or the environment, physical exertion, or any other medical condition.

17.11 	Appendices L and M are to be used.

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 18

Standard Operating Guidelines				        		Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:   MEDICAL EVALUATION TEAM RESPONSIBILITIES

Purpose:  To minimize the health risks to entry team, back-up team, and decon team personnel.

18.01	The medical evaluation team shall report to the Haz Mat Medical Officer (H.M.M.O.).

18.02	Using an Entry Team Data Sheet, the evaluation team shall document pre-entry vital

signs on entry team, back-up team and decon personnel.  These vitals shall be taken and documented for each entry.

18.03	The donning of chemical protective clothing shall be denied to any person with any of

the following:

18.03.01	Temperature >100.6F

18.03.02	BP > 150/90

18.03.03	Respirations > 24

18.03.04	Pulse >110

18.04	The evaluation team shall obtain information from the (H.M.M.O.) as to

the health hazards  that can be expected when a person becomes contaminated from the hazardous material(s) being dealt with.

18.05	The evaluation team shall advise the entry team, back-up team, and decon team

personnel of the expected signs and symptoms from exposure to the product(s).

18.06	The evaluation team shall ensure that the entry team, back-up team, and decon team

personnel are hydrated prior to donning their protective clothing.

18.07	Appendix L to be used.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  19

Standard Operating Guidelines				          		Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  ENTRY TEAM GROUP- OFFICER RESPONSIBILITIES

Purpose:  Operations during the mitigation of a Hazardous Material Incident should focus on the entry team and their safety.

19.01	The Entry Team Officer shall report to the Haz Mat Branch Officer (H.M.B.O.).

19.02	The Entry Team Officer shall be a Hazardous Materials Technician.

19.03	The Entry Team Officer shall be identified by a vest.

19.04 	The Entry Team Officer shall assign a minimum of two (2) personnel for

an entry team. Three (3) preferred if manning is available

  - The Entry Team Officer shall assign a minimum of two (2) personnel for a back-up team.

Three (3) preferred if manning is available.

19.06 	The entry and back-up team shall be approved by the Medical group prior to entry.

19.07	The Entry Team Officer shall supply one (1) support person per entry team member.

19.08	The Entry Team Officer shall supply the proper protective equipment for each entry team

member based on the information received from the Science Information Officer.

19.09	The Entry Team Officer shall brief the entry team and back-up team prior to entry.

19.10	The Entry Team Officer shall monitor air pressure during entry and decon operations of

entry team and back-up team members.

19.11	The Entry Team Officer shall communicate with the entry team during  Hot Zone

operations and continually monitor their status.

19.12	The Entry Team Officer shall debrief the entry team after decon to evaluate progress and

additional needs to mitigate the incident.

  - Appendices L, N, P and/or Q to be used by the Entry Officer.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  20

Standard Operating Guidelines				         		 Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  ENTRY AND BACK - UP TEAM RESPONSIBILITIES

Purpose:  To identify the responsibilities of the entry and back up team.

20.01	The entry and back up teams shall fall under the control of the Entry Team Officer.

20.02	Entry and back up team members shall be Hazardous Materials Technicians.

20.03	Entry and back up team members shall receive a pre-entry check of vitals by Medical

group personnel prior to donning chemical protective clothing.

20.04	Entry and back up team members shall hydrate prior to donning chemical protective

clothing.

20.05	Entry and back-up team personnel shall place personal belongings in a plastic bag then

give the bag to the Entry Officer.

20.06	Entry and back up team members shall wear suits providing equal protection.

20.07	Entry and back up team members shall use SCBA of equal capacity.

20.08	Entry team and back up team members shall be thoroughly briefed prior to entry into the

Hot Zone.

20.09	The back up team shall be prepared to enter the Hot Zone to rescue or assist the entry

team.

20.10	To prevent heat stress and use of air the back up team shall be partially suited with

chemical protective clothing and off air while on standby.

20.11	Entry and back up teams exiting the Hot Zone shall proceed to the Decon division for

proper decontamination.

20.12	The entry team shall be debriefed after exiting decon by the Entry Team Officer.

20.13	Entry and back up teams exiting decon shall proceed to Rehab for medical evaluation.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  21

Standard Operating Guidelines				         		 Page:	1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  COMMUNICATIONS

Purpose:  In all complex and protracted fire department operations, communications play a critical role.  With this in mind the following guidelines should be utilized for communications within the framework of the Incident Command System.

21.01	The radio channels should  be utilized in the following manner:

    - IFERN (154.265) used by Incident Commander, Staging and responding companies.

    - The use of radio frequencies 154.280 (WHITE fireground) and 153.803 (RED                 fireground) will be announced by the Incident Commander.

    - Either 154.280 (WHITE fireground) or 153.830 (RED fireground) should be                  designated for use by the Entry Team Officer and Entry Team Personnel during “Hot      Zone” operations.

    - Other fireground channels that may be used: BLUE (154.295), BLACK (154.2725),       GOLD (153.8375), GRAY (154.2875)

21.02	The pre-entry briefing should identify hand signals that will be used in the event that radio

communications between the Entry Team and the Entry Team Officer fail.

21.03	During Hot Zone operations all radio communications should be limited to essential

messages only.

21.04	All communications directed to the entry team shall be made by the Entry Team Officer.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  22

Standard Operating Guidelines				         		 Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  SITE CLEAN-UP

Purpose:  Cleanup of the spilled product, incident site, and associated locations shall be the responsibility of the Spiller.

22.01	A management representative of the Spiller's organization shall be informed that his

company must arrange to have the site cleaned up and that the expenses incurred shall be the responsibility of the Spiller.

22.02	If representatives of the Command Post determine that, in the interest of Public Safety,

cleanup of the incident site should not be delayed, then the Incident Commander should request that the appropriate agency,  such as the Illinois Environmental Protection Agency,  notify a suitable professional cleanup contractor.   Failure to have the appropriate agency  contact a cleanup contractor may result in the fire department bearing the cost of the contractor.

22.03	The replacement cost of tools, equipment, and protective clothing deemed non-reusable

by the fire department or the TEAM following the incident shall be the responsibility of the Spiller.   A representative of either the Spiller or the cleanup contractor should review the inventory and issue a receipt for the equipment to a fire department command representative.

22.04	The fire department may choose to maintain a representative on the scene of the

incident until such time that cleanup is completed.  The final decision on the adequacy and extent  of the clean up should rest with the appropriate agency.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  23

Standard Operating Guidelines				         		Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:	POST INCIDENT CRITIQUE

Purpose:  A comprehensive analysis is an important part of hazardous materials management in order to evaluate present operations, and to prepare for further occurrences.

23.01	The Incident Commander shall cause, and if at all possible host, a critique and evaluation

of any hazardous materials incident as soon as is practical following the termination of the incident.  Representatives from all participating agencies and organizations should be invited to attend.

23.02	The critique process should seek to renew and improve on the scene operation and

should not function to determine fault, blame, or liability for the incident.

HAZARDOUS MATERIALS INCIDENT		    			Section Number:  24

Standard Operating Guidelines				        		  Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:  DOCUMENTATION

Purpose:  To provide records and reports for legal, fact finding, training, and data collection.

24.01	All information concerning the hazardous materials incident is important and should be

documented thoroughly and  completely.

24.02	Documentation of a Hazardous Materials Incident should include the following:

24.02.01	Illinois NFIRS report, including Haz Mat & Casualty Forms.

24.02.02	A copy of the dispatchers notes and logs

24.02.03	A list of agencies and their representatives involved.

24.02.04	Copies of shipping papers, MSDS, FAX information, and CHEMTREC

information.

24.02.05	Appropriate eyewitness statements.

24.02.06	Incident Command and division work sheets.

24.02.07	Any other available investigative reports.

HAZARDOUS MATERIALS INCIDENT		    			Section Number: 25

Standard Operating Guidelines				        		Page:  1 of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:   REIMBURSEMENT FOR EQUIPMENT AND SUPPLIES

Purpose:  To ensure that the TEAM trailers and/or TEAM Departments are reimbursed, in a timely manner, for non-reusable equipment and supplies used at a hazardous materials incident.

25.01	Departments requesting the TEAM and/or TEAM trailer(s) shall be responsible for the

cost of replacing non-reusable equipment and supplies used at a hazardous materials incident.

25.02	The coordinators of the TEAM Departments shall be responsible for purchasing and

replacing their own non-reusable equipment and supplies.

25.03	The coordinators of the department housing the TEAM Trailers shall be responsible for

purchasing and replacing non -reusable equipment and supplies used from the trailers.

25.04	A detailed list of the non - reusable equipment and supplies, along with the associated

cost, shall be forwarded to the Chief of the stricken department within ten business days of the incident by the coordinators of TEAM Departments as applicable.  Accompanying the list shall be a request for reimbursement of the total cost including shipping charges.

25.05	Stricken Departments should reimburse TEAM Departments within thirty business days of the

receipt of the detailed list of non-reusable equipment and supplies, along with the associated cost from the TEAM Department. The chief of the stricken department shall inform the Chiefs of the TEAM Departments in the event of any delay.

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  26

Standard Operating Guidelines				         		 Page:  1  of  1

Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:	TEAM MEMBERSHIP REQUIREMENT

Purpose:  To have a standard level among all TEAM members.

26.01	Members of Fire Departments and Fire Protection Districts who desire to become

members of the TEAM must have the following minimum requirements:

26.01.01 	 OSFM Certified First Responder.

26.01.02	Approval from the employees Fire Chief.

26.02	In order to remain active as a member of the Team, members must fulfill the following.

26.02.01 	Meet the minimum annual training requirements.

26.02.02 	Become familiar with all aspects of the TEAM operation.

26.03	TEAM members should be aware of the opportunity to receive specialized training from

outside the fire department and TEAM setting and should prepare to  share that information with other team members.

26.04	The following lists the roles of personnel generally required at a hazardous materials

incident and their minimum certification levels:

26.04.01	Haz Mat Branch Officer – H.M.B.O. (Technician)

26.04.02 	Haz Mat Safety Officer – H.M.S.O. (Technician)

26.04.03	Decontamination Officer (Technician)

26.04.04	Haz Mat Entry Team Officer (Technician)

26.04.05	Haz Mat Science Information Officer (Technician)

26.04.06	Haz Mat Medical Officer – H.M.M. O. (First Responder & Paramedic)

26.04.07	Decon Team (Technician/First Responder)

26.04.08	Entry team (Technician)

26.04.09	Resources (First Responder)

    - Trailer Supply Officer (First Responder)

HAZARDOUS MATERIALS INCIDENT		    		Section Number:  27

Standard Operating Guidelines				         		 Page:  1  of  1               Effective Date:  June 1, 1994

Revised: March 11, 2008

Section Title:   TEAM TRAINING REQUIREMENTS

Purpose:  Training is essential to maintain the high level of competency necessary for a                           Hazardous Material Incident Team.  This section sets the requirements to meet these training needs.

- 27.01   Five TEAM drills are required per calendar year.  One must be a full scale evolution.

- 27.02	 Credit for a drill will be considered for:

27.02.01	Involvement in an actual hazardous materials incident.

27.02.02	Attendance at a course, school, conference, class and symposium.

27.02.03	A request for credit should be submitted to the TEAM member's                        department coordinator with a brief description of the incident or class and         the level of  involvement.  The coordinator will approve/disapprove

and inform the individual.

  - A member may use 27.02.01 or 27.02.02 one time per year.

  - Each TEAM Hazardous Materials Technician shall suit qualify on an annual basis.

  - TEAM members failing to meet the training requirements will be placed on inactive

status with the TEAM, not to exceed twelve (12) months, until the training requirements                are met.

  - Notification to the chief of the department shall be made in writing by the

departments TEAM Coordinator of the TEAM Member's inactive status.

  - The individual placed on inactive status shall surrender the Haz Mat Passport name tags to    the department's coordinator.

  - TEAM members that meet the training requirements will be placed back on the TEAM.

  - The Chief of the department will certify on an annual basis that hazardous materials              responders    have shown competency at the level in which they operate. This list of TEAM   members shall be   forwarded to the TEAM Coordinator by February 1st each year.

  - Normal training sessions will be on the third Monday 1830-2130, Tuesday 0900-1200, and   Saturday 0900-1200.  This does not include full scale evolutions or guest speakers where

- other arrangements have been made.

  - Hosting Departments shall provide a notice/reminder to each participating department

two weeks prior to the date of the class.  The notice shall include the location, time and                  subject	area two weeks prior to the date of the class.

  - Appendix O  is to be used.

APPENDIX A

MINIMUM EQUIPMENT LIST FOR FIRST RESPONDERS

Reference Books

- DOT Emergency Response Guidebook

- NFPA Fire Protection Guide for Hazardous Materials

- Hazardous Materials Reference Material of Department’s Choice

Protective Equipment (per Responder)

1set	      Full set of protective clothing including helmet, coat, gloves, pants and

boots per individual.

- Self Contained Breathing Apparatus

- Hearing Protection (ear plugs)

Monitoring Devices

- Binoculars (minimum magnification 7 x 35)

- Electochemical Sensor (3 or 4 range)

1 pkg.	      pH paper

Spill Control

- Spade Shovel

- Street Broom

- Roll of Visqueen

- Sandbags (empty)

50#	      Absorbant Material

Leak Control

Asst.	       Plugs and Patches

Asst.	       Plugging Compound

Asst. 	       Handtools

2 rolls	       Duct Tape

Foam and Foam Appliances

20 gal.*	        AFF/ATC 3% or 6%

- 95 GPM Foam Eductor

- Foam Aspirating Nozzle (95 GPM minimum)

Miscellaneous

1 roll	         Barricade Tape

- *This is a minimum quanity. Recommended enough to properly proportion the tank capacity.

APPENDIX C

INCIDENT COMMAND

DATE__________ LOCATION_______________________________________INCIDENT #__________

INCIDENT COMMANDER________________________ OWNER__________________ TX__________

OCCUPANT__________________________________________________TX______________________

RESPONSIBLE PARTY_________________________________________TX______________________

CHEMICAL(S) INVOLVED______________________________________________________________

NFPA 704: HEALTH________ FLAMABILITY________ REACTIVITY__________ OTHER ________

| # TIME DISPATCHED |  | PRODUCT IDENTIFICATION |  |
| --- | --- | --- | --- |
| # ON SCENE |  | PRODUCT CONFINED/CONTAINED |  |
| COMMAND ESTABLISHED |  | # DIBRIEFING CONDUCTED |  |
| SITE SAFETY PLAN ESTABLISHED |  | COMMAND TERMINATED |  |
| # EVACUATION INITIATED |  | POST INCIDENT CRITIQUE |  |
| ZONES ESTABLISHED |  | IC RECORDS COMPLETED |  |
| # EVACUATION COMPLETED |  |  |  |

## ASSIGNMENTS

| # HAZMAT GROUP |  | # SAFETY GROUP |  |
| --- | --- | --- | --- |
| Officer: |  | Officer: |  |
| # MEDICAL GROUP |  | Officer: |  |
| Officer: |  | # ENTRY GROUP |  |
| SCIENCE INFORMATION GROUP |  | Officer: |  |
|  |  | # DECON GROUP |  |
| STAGING GROUP |  | Officer: |  |
| Officer: |  | # PUBLIC INFORMAION |  |
|  |  | Officer: |  |

LAW ENFORCEMANT TASKS

| EVATUATION STARTED |  | TRAFFIC CONTROL INITIATED |  | CROWD CONTROL PROVIDED |  |
| --- | --- | --- | --- | --- | --- |
| EVACUATION COMPLETED |  | TRAFFIC CONTROL ENDED |  |  |  |

APPENDIX D

INCIDENT PLANNING DIAGRAM

LOCATION:___________________________________________________________________________

INCLUDE: HOT, WARM, AND COLD ZONES

APPENDIX E

HAZMAT GROUP

LOCATION:_____________________________________________________ DATE:________________

HAZMAT OFFICER:____________________________________________________________________

CHEMICAL (S) INVOLVED:_____________________________________________________________

HAZARD CLASS:____________ ID#________________ NFPA: H____F____R____OTHER_________

| Report to IC |  | Overall Hazard Analysis |  |
| --- | --- | --- | --- |
| Don Vest |  | 1st Entry into Hot Zone |  |
| Establish Zones |  | Recon Complete |  |
| Decon Group Established |  | Determine Need for Additional Resources |  |
| Science Information Group Established |  | Re-evaluate conditions |  |
| Entry Grouup Established |  | Product Confined/Contained |  |
| Medical Group Established |  | Last Entry into |  |

## AMOUNT OF PRODUCT

| Quantity: Released: Remaining: |  |
| --- | --- |
| Quantity: Released: Remaining: |  |

## CONTROL TECHNIQUES A-Attempted C-Completed

| Diversion | Diking | Retention | Plugging | Patches |
| --- | --- | --- | --- | --- |
| Pressure Reduction | Vapor Suppression | Neutralizing | Diposal | Other Methods |

## AGENCIES NOTIFIED

| AGENCY | TIME NOTIFIED | TELEPHONE | REPRESENTATIVE’S NAME | ETA | SCENE |
| --- | --- | --- | --- | --- | --- |

### APPENDIX F

SAFETY GROUP

LOCATION:_________________________________________________DATE:____________________

SAFETY OFFICER:___________________________________________INCIDENT #_______________

| Don Vest |  | Back-up Team Established |  |
| --- | --- | --- | --- |
| Report to IC |  | Suit Support Team Established |  |
| Hazmat Group Established |  | Briefing of Entry and Decon Teams |  |
| Science Information Group Established |  | Recon Established |  |
| Evacuation Started |  | Product Identified |  |
| Evacuation Completed |  | Product Control: Confined/Contain |  |
| Security of Zones Established |  | Decontamination |  |
| Security of Site Established |  | Debriefing |  |
| Decon Group Established |  | Recovery Initiated |  |
| Decon PPE Established |  | Disposal/Decontmination of Contaminated Equipment Completed |  |
| Medical Group Established |  | Disposal/Decontmination of Contaminated Clothing Completed |  |
| Entry Group Established |  | Post Incident Critique Established |  |
| Entry Group PPE Established |  | Records Received |  |
| Entry Team (s) Established |  | Termination |  |

## RECORDS

| # RECORDS | RECEIVED | FROM | COMPLETED | BY |
| --- | --- | --- | --- | --- |
| IC Records |  |  |  |  |
| HAZMAT Records |  |  |  |  |
| Entry Team Records |  |  |  |  |
| Decon Records |  |  |  |  |
| Medical Records |  |  |  |  |
| Science Information Records |  |  |  |  |

### APPENDIX G

### PRODUCT DATA SHEETS

## DULICATE THIS SHEET AS NEEDED, ONE )1) SHEET PER PRODUCT

NUMBER EACH SHEET.   SHEET _______ OF _______

| SPECIFIC NAME |  |  | RQ |  |
| --- | --- | --- | --- | --- |
| CHEMICAL NAME |  |  | PRODUCT AMOUNT |  |
| HAZARD CLASS ID # |  |  | TYPE OF CONTAINER |  |
| CAS# |  |  | TYPE OF INCIDENT: SPILL LEAK FIRE REACTION: |  |
| STCC# |  |  | NFAP 704: H F R Other |  |
| REFERENCE SOURCES | 1. | 2. |  | 3. |
| Flash point |  |  |  |  |
| Flammable Range |  |  |  |  |
| Ignition Temerature |  |  |  |  |
| Boiling Point |  |  |  |  |
| Specific Gravity |  |  |  |  |
| Vapor Pressure |  |  |  |  |
| PH |  |  |  |  |
| TLV |  |  |  |  |
| STEL |  |  |  |  |
| Ceiling Limit |  |  |  |  |
| IDLH |  |  |  |  |
| Odor Threshold |  |  |  |  |
| LC or LD 50 100 |  |  |  |  |
| Min. Evacuation Distance |  |  |  |  |
| Reacts with |  |  |  |  |
| Solubility |  |  |  |  |
| Exposure Routes |  |  |  |  |
| Signs & Symptoms |  |  |  |  |
| First Aid |  |  |  |  |
| PPE Hot Zone |  |  |  |  |
| PPE Warm Zone |  |  |  |  |
| Pollution Potential |  |  |  |  |
| Extinguishing Agent (s) |  |  |  |  |

### APPENDIX H

### DECONTAMINATION GROUP

DATE______________________ LOCATION________________________________________________

DECONTAMINATION OFFICER__________________________________________________________

| Report to HAZMAT Officer |  | 4 – 6 Personnel Assigned |  |
| --- | --- | --- | --- |
| Don Vest |  | Decon Group Data Sheet Completed |  |
| Determine Decon Needed |  | Decon Established |  |
| Establish Decon Area |  | HAZMAT Officer Notified |  |
| PPE Selected |  | Safety Officer Notified |  |

## EQUIPMENT

| Water Supply |  | Hose and Adapters |  |
| --- | --- | --- | --- |
| Diking Material |  | Plastic Bags |  |
| Template |  | Spray Tanks |  |
| Visqueen |  | Soap |  |
| Entry Markers |  | Brushes |  |
| Pools |  | Suit Doffing Seats |  |
| Shower Wands |  | Absorbant Pads |  |

## DISPOSABLE CLOTHING

| Gowns |  | Coveralls |  | Shoes |  |
| --- | --- | --- | --- | --- | --- |

### APPENDIX I

### TEMMPLATE FOR GROS DECON

| # SCBA # CONTAMINATED CLOTHING CONTAMINATED EQUIPMENT | ## EXIT CRITICAL TREATMENT AREA (IF NEEDED) # SCBA REMOVAL (IF SAFE) ## GROSS DECON AREA (Diked Area) CLOTHING REMOVAL. ENTER | # CLEAN AREA |
| --- | --- | --- |

### APPENDIX J

TEMPLATE FOR DRY – ISOLATION/DISPOSAL DECON

20’

| ##### SCBA 40’ # CONTAMINATED # CLOTHING # CONTAMINATED EQUIPMENT | ### EXIT ##### SCBA REMOVAL # SUIT REMOVAL. ENTER | ##### CLEAN AREA |
| --- | --- | --- |

### APPENDIX K

TEMPLATE FOR DILUTION DECON

20’

| # SCBA 40’ CONTAMINATED CLOTHING CONTAMINATED EQUIPMENT | ## EXIT PERSONAL SHOWER (OFF SITE – IF POSSIBLE) # SCBA REMOVAL POOL 2. POOL 1.. ## ENTER | # CLEAN AREA |
| --- | --- | --- |

### APPENDIX L

ENTRY TEAM DATA SHEET #______

SHEET ______ OF ______

ENTRY SHALL BE DENIED TO ANY PERSON WITH ANY OF THE FOLLOWING: TEMPERATURE >100.6 F RESPIRATIONS > 24 BLOOD PRESSURE > 150/90 PULSE >110

## MEDICAL GROUP DATA

| NAME:__________________________________ DEPARTMENT:___________________ID#_____ # PRE ASSIGNMENT Vital signs: Temp:_____R_____P____BP____/____ APPROVED: YES NO Initials_____________ REASSIGNED: Staging_________ Other________ | NAME:__________________________________ DEPARTMENT:___________________ID#_____ # PRE ASSIGNMENT Vital signs: Temp:_____R_____P____BP____/____ APPROVED: YES NO Initials_____________ REASSIGNED: Staging_________ Other________ |
| --- | --- |

## ENTRY GROUP DATA

| Suit#_________Level________Type____________ Suit support (name)__________________________ Yes No Suit inspected pior to entry Yes No Briefed Time on Air:_______________ PSI_______________________ Time of entry into hot zone____________________ Time left in hot zone_________________________ Yes No Routed to Decon Division |  | Suit#_________Level________Type____________ Suit support (name)__________________________ Yes No Suit inspected pior to entry Yes No Briefed Time on Air:_______________ PSI_______________________ Time of entry into hot zone____________________ Time left in hot zone_________________________ Yes No Routed to Decon Division |
| --- | --- | --- |
| Notified decon to go on air. Yes_________ | Notified decon to go on air. Yes_________ |  |

## DECONTAMINATION GROUP DATA

| Time entered decon___________________________ Time on air bottle #1__________________________ Time off air bottle #1__________________________ Time on air bottle #2__________________________ Time off air bottle #2__________________________ Time decon completed_________________________ Yes No Full decon completed Yes No Routed to medical group | Time entered decon___________________________ Time on air bottle #1__________________________ Time off air bottle #1__________________________ Time on air bottle #2__________________________ Time off air bottle #2__________________________ Time decon completed_________________________ Yes No Full decon completed Yes No Routed to medical group |
| --- | --- |

## MEDICAL GROUP DATA

| Post assignment Time____________ Vital Signs: Temp._____R_____P_____BP___/____ Additional monitoring time_____________________ Approved Yes No Initials__________________ Reassigned Rehab_________RX’d_______________ Transported______________Other_______________ | Post assignment Time____________ Vital Signs: Temp._____R_____P_____BP___/____ Additional monitoring time_____________________ Approved Yes No Initials__________________ Reassigned Rehab_________RX’d_______________ Transported______________Other_______________ |
| --- | --- |

### APPENDIX M

APPENDIX N

ENTRYGROUP

| ###### Report to HAZMAT Officer |  | Briefing |  |
| --- | --- | --- | --- |
| Don Appropriate Vest |  | Radio Checks |  |
| Assign Entry Team (Min. 2) |  | Air Supply Monitored |  |
| Assign Back Up Team (=to entry) |  | Working Time Monitored |  |
| Entry Data sheet Rec’vd |  | Status Checks |  |
| Assign support person per member |  | Debrief Entry Upon exiting |  |
| Determine PPE |  | Brief HAZMAT Group Officer |  |
| Decon Set Up |  | Entry to Rehab |  |

RECON INFORMATION

Container Information

| # CONTAINER #________ Type of container: Carboy__________Box/Bag____________ Drum: Plastic____________Steel________ Cylinder________________ Tank Car: Non-Pressure___Pressure___Hopper___ | # CONTAINER #________ Type of container: Carboy__________Box/Bag____________ Drum: Plastic____________Steel________ Cylinder________________ Tank Car: Non-Pressure___Pressure___Hopper___ |
| --- | --- |
| Identification Markings: Color____________________________________ Label/Information__________________________ Placard___________________________________ Container Information_______________________ Other Markings____________________________ | Identification Markings: Color___________________________________ Label/Information_________________________ Placard__________________________________ Container Information______________________ Other Markings___________________________ |
| # Damage to Container Corroded_______Tear_______Punctured_________ Shear__________Dent_______Gouge___________ Score__________Other_______________________ | # Damage to Container Corroded_______Tear_______Punctured_______ Shear__________Dent_______Gouge_________ Score__________Other_____________________ |
| Area of Damage: Shell_______Valve(s)_________ Piping______Other___________ | Area of Damage: Shell_______Valve(s)_______ Piping______Other_________ |
| Exposures: Sewers_________________________ Other Containers_________________ Property________________________ | Exposures: Sewers_______________________ Other Containers_______________ Property______________________ |
| Other Findings: | Other Findings: |

### APPENDIX O

Training & Attendance Sheet

| Date: | Location: |
| --- | --- |
| Time: | Subject: |
| Practical Hrs: | Goals & Objectives |
| Lecture Hrs. | Instructor: |
|  | Instructor: |

| Name (Print) | Signature | Department |
| --- | --- | --- |

### Appendix P

#### Level A and Level B Encapsulating

# Donning Procedure Checklist

________  1. 	All components of the suit ensemble must be gathered and staged directly next to

the member being dressed.

________  2. 	All components of the suit ensemble, including the zipper, must be inspected for  defects for cuts, tears, abrasions, etc.

________  3. 	Establish entry vitals and register the results on the entry record.

IF ANY MEMBER DOES NOT MEET THE VITAL SIGNS REQUIREMENTS LISTED ON THE ENRTY RECORD, THE MEMBER SHALL NOT BE DRESSED IN CHEMICAL PROTECTIVE CLOTHING.

________  4. 	A final briefing from Safety, Entry, or Decon Division is conducted prior to entry.

The entry team game plan and final check of equipment needed to execute the

plan shall be reviewed.

________  5. 	Review and perform emergency hand signals with all entry team members.

________  6. 	Remove personal items.

________  7. 	Remove the member’s shoes.

________  8. 	Assist the member in sitting. (Remind entrants to conserve energy.)

________  9. 	Place feet in the suit, one at a time.

________  10. Place feet in over boots, one at a time.

________  11. Assist member in standing.

________  12. Don SCBA harness.

________  13. Register tank pressure on the member’s entry record.

________  14. Perform radio check.

________  15. Don SCBA face piece and perform positive and negative pressure check.

#### Level A and B Encalpsulating

Donning Procedure Checklist (continued)

________  16. Don protective head gear and perform bend-over test.

________  17. Put on the inner gloves.

- Nitrile (Blue) exam glove

- Silver Shield / Ansell

- Nitrile (Blue) exam glove

________  18. Work suit up and around SCBA and place arms into suit, one at a time.

________  19. Register in-suit time on the member’s entry record.

________  20. Place on outer gloves and banding. (Kevlar gloves)

________  21. Assist member to go on air.

________  22. Register the on-air time on the member’s entry record.

________  23. Encapsulate member.

________  24. Suit Support personnel shall perform a final visual inspection of the suit, checking     		for any damage that may have been caused by donning the suit.

Entry records shall now be passed to the entry officer.

Team members are now ready to enter the Hot Zone.

### Appendix Q

##### Level B Non-Encapsulating

Donning Procedure Checklist

________ 1. All components of the Level B suit ensemble must be gathered and staged

directly next to the member being dressed.

________ 2. All components, including the zipper, of the Level B ensemble shall be

inspected for defects, tears, cuts, abrasions, etc.

________ 3. Establish entry vitals and register the results on the member’s entry

record.

IF THE MEMBER DOES NOT MEET THE VITAL SIGN REQUIREMENTS LISTED ON THE ENTRY RECORD THE MEMBER WILL NOT BE ALLOWED TO DON CHEMICAL PROTECTIVE CLOTHING.

________ 4. Final briefing is conducted by Safety, Entry or Decon Officer.  The briefing

shall review the plan for the member being dressed and all equipment used

to execute the plan shall be checked.

________ 5. Review and have each member perform the emergency hand signals.

________ 6. Have member remove personal items.

________ 7. Have member remove shoes.

________ 8. Assist member in sitting. (Remind member to conserve energy.)

________ 9. Place member’s feet in the suit, one at a time.

________ 10. Place member’s feet in the over boots, one at a time.

________ 11. Don SCBA face piece and perform positive and negative pressure check.

________ 12. Put on gloves:

- Nitrile (Blue) exam glove

- Silver Shield / Ansell glove

- Nitrile (Blue) exam glove

- Green Nitrile glove

________ 13. Assist member in standing.

#### Level B Non-Encapsulating

Donning Procedure (continued)

________ 14. Work suit up and around member and place arms in the suit, one at a time.

________ 15. Place banding over the glove / sleeve closure.

________ 16. Zip the suit up

________ 17. Record the in-suit time on the member’s entry record

________ 18. Put on the member’s hood.

________ 19. Assist member in donning SCBA harness / tank.

________ 20. Assist member in donning protective head gear.

________ 21. Have member perform bend-over test.

________ 22. Perform radio check.

________ 23. Assist member to go on air.

________ 24. Record the on-air time in the member’s entry record.

________ 25. Suit Support personnel shall perform a final visual inspection of the suit, checking for any                                damage that may have been caused during donning.

Entry Records are now passed to the Entry or Decon Officer.

The members are ready to perform their assigned task.

### Appendix R

| MABAS Division 12 #### State Wide HazMat Response Team (Includes Responses to non MABAS XII Communities) | #### HazMat |
| --- | --- |

| P R I M A R Y | HazMat Coordinator | Technicians (Two per Department) | HazMat Trailer* | Other |
| --- | --- | --- | --- | --- |
|  | Primary Equipment |  |  |  |
|  | Addison | Addison Carol Stream Lombard Itasca Wood Dale | Addison |  |
|  | Staging Area: Addison Station #3 @ 100 N. Rohlwing Road (IL 53) Responding Units have TWO (2) hours to assemble in staging area |  |  |  |

| A L T E R N A T E S | HazMat Coordinator | Technicians (Two per Department) | HazMat Trailer | Other |
| --- | --- | --- | --- | --- |
|  | Alternate Equipment |  |  |  |
|  | Hanover Park | Bloomingdale Elmhurst Glenside Roselle Wheaton | Hanover Park |  |
|  | Staging Area: Addison Station #3 @ 100 N. Rohlwing Road (IL 53) Responding Units have TWO (2) hours to assemble in staging area |  |  |  |

Special Instructions

- For Responses to MABAS Division IX (Chicago) also include: - 1 – engine, 1 – truck or squad, 1 – ALS ambulance and 1 – chief officer - Equipment to be taken from the Division XII Task Force Box Card in the listed order

- Bloomingdale is the back-up tow vehicle for the trailer 630.894.8488

| Notify MABAS XII President (Chief Mike Kopp) | Pager: 630.244.9468 |
| --- | --- |

| Effective: | March 1, 2003 | Revised: | August 19, 2007 | Authority: | MABAS Division XII |
| --- | --- | --- | --- | --- | --- |