- Subject: 	Mass Decontamination:

- 1	Setting Up The Emergency Decon Corridor

- 1.1	Evacuate victims upwind, upgrade, and upstream of the hazard.

- 1.2	Isolate and establish control of the victims. Decon should be immediate.

- 1.3	Communicate with victims in order to prepare for decontamination. Segregate

- symptomatic victims from the asymptomatic victims and male from female. Take

- into consideration families, small children, the elderly, handicapped persons and

- uncooperative persons.

- 1.4	Position the first arriving engine with the pump panel facing a hydrant. Start to

- decontaminate as soon as possible. Speed is of the utmost importance if lives

- are at risk. Consider placing the deck gun or fog line in use before additional

- units arrive.

- 1.5	Have the second arriving engine position 12 to 16 feet apart from the first engine

- facing the opposite direction. The pump panels will be on the aspect and all

- windows are closed. The basic mass decon corridor is between the two engines.

- 1.6	The first engine will provide a water supply to the second. Fog nozzles are

- attached to the 2 ½” discharges opposite the pump panels on both engines.

- 1.7	Pump pressures are set at 30 psi and fog nozzles are adjusted on wide fog. The

- fog patterns should meet in the center.

- 1.8	Direct symptomatic victims to remove clothing down to their undergarments. If

- available provide clear plastic bags for clothing and valuables

- 1.9	Have exposed victims walk through the fog streams for a 1- to 3- minute wash.

- 1.10	Provide modesty wear for cover if available.

- 1.11	Direct victims to safe refuge or medical triage or treatment areas.

- 2	Mass Decon: Additional Tactical Considerations

- 2.1	As additional resources arrive, basic mass decon can be expanded adding

- another engine company. The additional corridor may separate male and female

- victims or symptomatic from contaminated victims. Further triage can remove

- victims who were in the area but are not contaminated. These individuals should

- be segregated and held for further evaluation.

- 2.2	Approximately 80 percent of the contamination is removed with clothing. If the

- contaminant is radiological or biological, the victim should be wet down before

- removing the clothes. Wet clothing prevents re-aerosolization as clothing is

- removed.

- 2.3	Soap and water is recommended for decontaminating warfare agents. In some

- cases, bleach and water will cause more harm to the victim. Mustard agents are

- an example; bleach and water causes further tissue damage. When capabilities

- and resources allow, employ soap and water as the best decontaminate of

- choice.

- 2.4	In an actual biological attack, response to the agent may come hours or days

- later. Most likely this response will occur remote from the point of contamination

- and will not result in a mass decon incident. However, there have been many

- occasions where a group believes they have been contaminated but show no

- signs of exposure. In this case, psychological decon may be performed. Victims

- who initially are not symptomatic or contaminated can be decontaminated at

- another location. Use of a fixed facility or the decon trailer offers privacy and

- peace of mind.

- 2.5	When possible, attempt to capture runoff. Realistically, however, when

- confronted with hundreds of patients, runoff is a secondary concern. However,

- uncontrolled runoff may result. Environmental officials and municipal authorities

- must be notified to assist with runoff control and proper disposal of contaminated

- materials.