- Section:	Material Identification And Information

Purpose:  	To obtain proper information on the hazards of the chemical(s) involved.

1	A minimum of three reference sources shall be used to research all chemical

information.

2	A minimum of two team members shall check all pertinent data in each reference source to eliminate errors.

3	Material Safety Data Sheets shall be checked against at least two different reference sources.

4	Computer information shall be checked against at least two different reference sources.

5	Information received from any chemical manufacturer, shipper, or user shall be checked against at least two different reference sources.

6	Identification of materials in mobile vehicles shall begin by identifying the container shapes and types.  Efforts should be made to observe the Department of Transportation placards displayed on the exterior of the vehicle.  Observation of the placards and the containers should be made with binoculars from as far a distance as possible.

7	If readily available, the shipping papers will provide the most accurate material

information.

8	When appropriate, notify CHEMTREC to obtain their assistance in identification and notification of the shipper, manufacturer, consignee, and carrier.

9	Identification of material at fixed sites should include the use of knowledgeable facility personnel.  Facility personnel should make available the Material Safety Data Sheets to the Science Information Officer if possible.  Personnel should be observant of the NFPA 704 Identification System to help identify possible hazards.

10	Appendix G is to be used.